ENTEBBE INTERNATIONAL AIRPORT

Airside Operations Manual

Appendix 1 of the Aerodrome Manual

UCAA/EIA/AM/APP/01

Revision 02			                                                                       28 April, 2025

### Foreword

The airside of an aerodrome is a unique working environment, which is governed by special rules designed to prevent accidents and minimize the risk of injury to persons and damage to property.

Accidents in general, do not just happen but are caused by an act of negligence or the omission of care of duty, by one party or the other, sometimes both.  In aviation, usually accidents are highly costly in terms of loss of life and damage to property. Entebbe International Airport (EIA) Management has therefore published this manual to guide all airport staff and users operating in the airside, to prevent accidents through a concerted effort of promoting safety.

These SOPs and work instructions have also been developed in accordance with Regulation 260 of the Civil Aviation (Aerodromes) Regulations 2022 (CAARs) and comprises of three parts: Apron Management, Apron Safety and Airside Vehicle Safety Operating Procedures (AVSOP).

All actors shall implement these procedures and work instructions to achieve the highest standards of safety, security, and quality of service in civil aviation.

Al Hajj Eng. Sooma Ayub

Director Airports and Aviation Security

### Approval Page

Prepared by:

Signature: ……………………………………………… 	Date 	…………………………………

General Manager – Entebbe International Airport

Reviewed by:

Signature: ……………………………………………… 	Date 	…………………………………

Director Airport and Aviation Security

Approved by:

Signature: ……………………………………………… 	Date 	…………………………………

Director Safety, Security and Economic Regulation

### Table of Contents

0.1.	Foreword	i

0.2.	Approval Page	ii

0.3.	Table of Contents	iii

0.4.	Amendments	vii

0.5.	Distribution list	viii

0.6.	Checklist of Effective Pages	ix

0.7.	Abbreviations:	x

0.8.	Definitions:	xiii

1.1.	Overview of Apron Management Functions	18

1.2.	Airside Administrative Structure:	19

1.2.1.	Names and Contacts of Persons in Charge of Airside Safety:	20

1.3.	Personnel Requirements for Airside Operations:	21

1.3.1.	Procedure for Continued Competency of Operations Department personnel:	21

1.3.2.	Entry Requirements for Airside Operations Section:	22

1.4.	Apron Management Procedure:	29

1.4.1.	Procedure for Shift Handover/Takeover:	29

1.5.	Procedure for Manpower Scale Down During Pandemic:	31

1.6.	Procedure For Coordination Between ATM and ACT:	32

1.7.	Procedure for Aircraft Stand Allocation:	33

1.7.1.	Apron Parking Reference Chart:	34

1.7.2.	Apron Stop Marks:	35

1.7.3.	Apron Stop Mark Legend:	35

1.8.	Procedure for Parking and Aircraft:	36

1.8.1.	Marshalling of incoming Aircraft:	36

1.8.2.	Marshalling of Outgoing Aircraft:	37

1.8.3.	EIA Aircraft Parking and Docking Chart:	38

1.9.	Procedure for Allocation of TPBBS:	39

1.10.	Procedure for Operating TPBBS:	40

a)	If found unclean, liaise with public utility to provide cleaning contractor to clean up the TPBB.	40

b)	Extend the TPBB for cleaning and retract thereafter to the parking position	40

1.11.	Procedure for VVIP Operations at EIA:	42

1.12.	Procedure for Code ‘F’ Aircrafts at EIA:	44

1.13.	Allowable Overload operations at EIA:	45

1.13.1.	The following are used to determine overload operations:	45

1.13.2.	Disallowable Overload Operations at EIA:	45

1.13.3.	Processing of Requests for Overload Aircraft Operations:	45

1.14.	Procedure for Control of Aircraft Engine Runup:	46

1.15.	Procedure for Aircraft Push Back at EIA:	47

1.16.	Procedure for Maintaining Aircraft Movement Records:	48

1.17.	Procedure for Facilitating Movement of Passengers, Cargo, and Other Stakeholders at the Airside:	49

1.17.1.	Leader Vehicle and Services:	50

1.18.	Course Curriculum for airside Safety Training:	51

2.1.	Basic Airside Safety Rules:	53

2.1.1.	Airside Driving permit:	53

2.1.2.	Speed Limit:	53

2.1.3.	Exemptions on Speed limits:	54

2.1.4.	Safety in the Vicinity of Aircraft:	54

2.1.5.	Protection from Jet Blast:	55

2.1.6.	Helicopter Movements and Danger Areas:	56

2.1.7.	Around Aircraft Points to Remember:	57

2.2.	Procedures for Apron Safety Management:	58

2.2.1.	Procedure for Airside PPE:	58

2.2.2.	Procedure for inspection of Movement Area:	59

2.2.3.	Procedure for inspection of Movement Area:	60

2.2.4.	Procedure for Reporting Runway Surface Condition:	60

2.3.	Procedure for Use of Stands 20-25 on Apron 1:	63

a)	Procedure for Coordinating Ramp Handling/Vehicle/Equipment movements To Aircraft Stand 20-25 on Apron One.	63

b)	Procedure For Coordinating Aircraft Parking on Stand 20-25:	64

i.	65

2.4.	Procedure For Use of Apron Five:	66

2.5.	Procedure For Removal of Contaminants in Movement Areas:	68

2.5.1.	Scrubbing And Cleaning of Movement Area	68

2.5.2.	Sweeping of the Movement Area	69

2.6.	FOD Management:	70

2.6. 1	Procedure in the Event of Spillages:	70

2.7.	Aircraft Fueling procedures:	72

2.8.	Procedure for Airside Accident/Incident Management:	80

2.8.1.	Procedure for investigation and Follow-up on Airside Incidents and Accidents:	81

2.8.2.	Procedure for Conducting Monthly Aircraft turn around Audit Surveys:	82

2.8.3.	Procedure For monitoring and Enforcement of Airside Safety Requirement	85

2.9.	Responsibilities of Vehicle/Equipment Operations at Airside:	89

2.9.1.	Employers Vehicle/Equipment Responsibilities	89

2.9.2.	Apron Management Vehicle/Equipment Responsibilities	89

2.9.3.	Vehicle/Equipment Operator	89

2.9.4.	Airside Pavement Markings	91

2.9.5.	Aerodrome Lighting System	92

2.9.6.	Pavement Signs	92

2.10.	Radio Procedures at the Airside:	94

3.2.1.	General R/T Instructions:	94

3.2.2.	Some Tips Before you go into the Maneuvering Area:	94

3.2.3.	Before Transmitting a Message:	94

3.2.4.	Microphone Procedures/Techniques:	95

3.2.5.	Permission to Enter Maneuvering Area:	95

3.2.6.	ICAO Phonetic Alphabet and The pronunciation of Numbers:	95

3.2.7.	Radio Call Up Procedures:	99

3.2.8.	Radio Test Procedures:	100

3.2.9.	Vehicle Identification/Radio Frequencies at EIA:	101

2.11.	Procedure for ADP and AVOP:	104

3.3.1.	Procedure For   ADP	104

3.3.2.	Procedure For AVOP	104

2.12.	Procedure for Enforcing Airside Vehicle/Equipement Operations:	106

2.13.	Roles and Responsibilities:	108

3.5. 1	Airport operations:	108

3.5. 2	Aerodrome Maintenance and Aerodrome Engineering, Planning and Development:	108

3.5. 3	Ground Handler Roles and Responsibilities:	108

3.5. 4	Airside Organizations Roles and Responsibilities:	109

3.5. 5	Air Traffic Control Roles and Responsibilities:	109

3.5. 6	Wildlife Management Roles and Responsibilities:	109

3.5. 7	The Manager SMS EIA or Designee shall:	109

2.14.	Preventive Measures:	111

3.6.1.	FOD Removal and maintenance:	111

3.6.2.	FOD Removal Equipment:	111

3.6.3.	FOD Walks:	111

3.6.4.	FOD incident reporting and Investigations:	111

3.6.5.	Training of Personnel:	112

3.6.6.	Procedure for FOD Management:	112

3.6.7	Procedure for Remote monitoring of Airside activity	114

Attachment 1: Nature of Airside Offence	116

Attachment 2: Foreign Object Debris (FOD) Prevention Program for EIA	119

Form 1: Staff Proficiency and Performance Monitoring Form (Aircraft Marshalling)	121

Form 2: Shift Handover/Takeover Form	124

Form 3: Apron Packing Reference Chart Form:	126

Form 4: Aircraft Parking/Stand Inspection Form	127

Form 5: TPBB Docking Record and inspection form	128

Form 6: Manoeuvring Area Inspection form	129

Form 7: Apron Inspection form	132

Form 8: Runway Surface Condition Report Form (GRF)	134

Form 9: Spillage Control Form	135

Form 10: Airside incident Report Form	136

Form 11: Preliminary Incident and Accident Investigation Report Form	137

Form 12: Airside Daily Operational Report Form	138

Form 13: monthly Aircraft Turn Around Form	139

Form 14: Apron Equipment Survey During Peak Traffic	143

Form 15: Airside Violation Form	145

Form 16: On Sport Equipment and Personnel Report Form	146

Form 17: Airside Driving permit Application Form	147

Form 18: ESSAT Checklist Form	149

Form 19: Dolly/Cart Audit Form	152

Form 20: FOD Form	153

Form 21: Quarterly FOD Walk Report Form	155

Form 22: Aircraft Fueling Safety Inspection Checklist	156

Form 23: Staff Deployment/Parking Plan Form	158

Form 24: Tank Farm Inspection Checklist	159

Form 25: Low Visibility Procedure Implementation Form	165

References:	168

### Amendments

| Revision No | Revision Date | Reason for Change | Amended Pages/Sections | Date Inserted | Inserted By |
| --- | --- | --- | --- | --- | --- |
| 00 | 19 Aug 2020 | Initial | All Pages |  | MO |
| 01 | 03 Feb 2023 | Manual Review | All Pages |  | MO |
| 02 | 28 April 2025 | Manual Review | All Pages |  | MO |

### Distribution list

| Copy No. | Issued to | Date | Sign |
| --- | --- | --- | --- |
| 001 | DAAS |  |  |
| 002 | GM-EIA |  |  |
| 003 | MO |  |  |
| 004 | MSMS |  |  |
| 005 | MAM |  |  |
| 006 | MQA |  |  |
| 007 | ASM |  |  |
| 008 | POO-AS |  |  |
| 009 | POO-AF |  |  |
| 010 | OC Tower |  |  |
| 011 | SOO-AS |  |  |
| 012 | SOO-AS/SCAD |  |  |
| 013 | Airport Medical Facility |  |  |
| 014 | RFFS |  |  |
| 015 | DANS |  |  |
| 016 | Apron Control Tower |  |  |
| 017 | Airside Operations Office |  |  |
| 018 | TOCC |  |  |

### Checklist of Effective Pages

| Page No | Revision Date | Version No | Page No | Revision Date | Version No | Page No | Revision Date | Version No |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ii | 28-Apr-25 | 2 | 17 | 28-Apr-25 | 2 | 37 | 28-Apr-25 | 2 |
| iii | 28-Apr-25 | 2 | 18 | 28-Apr-25 | 2 | 38 | 28-Apr-25 | 2 |
| iv | 28-Apr-25 | 2 | 19 | 28-Apr-25 | 2 | 39 | 28-Apr-25 | 2 |
| v | 28-Apr-25 | 2 | 20 | 28-Apr-25 | 2 | 40 | 28-Apr-25 | 2 |
| vi | 28-Apr-25 | 2 | 22 | 28-Apr-25 | 2 | 41 | 28-Apr-25 | 2 |
| 2 | 28-Apr-25 | 2 | 26 | 28-Apr-25 | 2 | 43 | 28-Apr-25 | 2 |
| 3 | 28-Apr-25 | 2 | 27 | 28-Apr-25 | 2 | 45 | 28-Apr-25 | 2 |
| 4 | 28-Apr-25 | 2 | 28 | 28-Apr-25 | 2 | 46 | 28-Apr-25 | 2 |
| 6 | 28-Apr-25 | 2 | 29 | 28-Apr-25 | 2 | 48 | 28-Apr-25 | 2 |
| 7 | 28-Apr-25 | 2 | 30 | 28-Apr-25 | 2 | 55 | 28-Apr-25 | 2 |
| 12 | 28-Apr-25 | 2 | 31 | 28-Apr-25 | 2 | 57 | 28-Apr-25 | 2 |
| 13 | 28-Apr-25 | 2 | 32 | 28-Apr-25 | 2 | 58 | 28-Apr-25 | 2 |
| 14 | 28-Apr-25 | 2 | 33 | 28-Apr-25 | 2 | 59 | 28-Apr-25 | 2 |
| 15 | 28-Apr-25 | 2 | 34 | 28-Apr-25 | 2 | 60 | 28-Apr-25 | 2 |
| 16 | 28-Apr-25 | 2 | 35 | 28-Apr-25 | 2 | 61 | 28-Apr-25 | 2 |
| 62 | 28-Apr-25 | 2 | 81 | 28-Apr-25 | 2 | 136 | 28-Apr-25 | 2 |
| 63 | 28-Apr-25 | 2 | 82 | 28-Apr-25 | 2 | 140 | 28-Apr-25 | 2 |
| 64 | 28-Apr-25 | 2 | 83 | 28-Apr-25 | 2 | 150 | 28-Apr-25 | 2 |
| 66 | 28-Apr-25 | 2 | 84 | 28-Apr-25 | 2 | 151 | 28-Apr-25 | 2 |
| 67 | 28-Apr-25 | 2 | 85 | 28-Apr-25 | 2 | 152 | 28-Apr-25 | 2 |
| 68 | 28-Apr-25 | 2 | 86 | 28-Apr-25 | 2 | 158 | 28-Apr-25 | 2 |
| 69 | 28-Apr-25 | 2 | 106 | 28-Apr-25 | 2 | 159 | 28-Apr-25 | 2 |
| 70 | 28-Apr-25 | 2 | 107 | 28-Apr-25 | 2 | 160 | 28-Apr-25 | 2 |
| 71 | 28-Apr-25 | 2 | 108 | 28-Apr-25 | 2 | 161 | 28-Apr-25 | 2 |
| 72 | 28-Apr-25 | 2 | 109 | 28-Apr-25 | 2 | 162 | 28-Apr-25 | 2 |
| 73 | 28-Apr-25 | 2 | 117 | 28-Apr-25 | 2 | 156 | 28-Apr-25 | 2 |
| 74 | 28-Apr-25 | 2 | 118 | 28-Apr-25 | 2 | 158 | 28-Apr-25 | 2 |
| 75 | 28-Apr-25 | 2 | 126 | 28-Apr-25 | 2 | 159 | 28-Apr-25 | 2 |
| 77 | 28-Apr-25 | 2 | 128 | 28-Apr-25 | 2 |  |  |  |
| 79 | 28-Apr-25 | 2 | 134 | 28-Apr-25 | 2 |  |  |  |

### Abbreviations:

| Abbreviation | Meaning |
| --- | --- |
| acft | Aircraft |
| AOO/AS | Assistant Airside Safety Officer |
| AC | Air Conditioner |
| ACT | Apron Control Tower |
| ACTC | Apron Control Tower Controller |
| AIMO | Aeronautical information Management Officer |
| AOA | Airport Operations Assistant |
| ARFFS | Aerodrome Rescue and Fire Fighting Services |
| ASC | Apron Safety Committee |
| ASM | Aviation Security Manager |
| ASP | Airport Security Permit/Pass |
| ATA | Actual Time of Arrival |
| ATM | Air Traffic Management |
| ATMO | Air Traffic Management Officer |
| AVS | Airside Vehicle Sticker |
| AVSEC | Aviation Security |
| AVSOP | Airside Vehicle Standard Operating Procedure |
| C/AVPOL | Commandant Aviation Police |
| DAAS | Director of Airports and Aviation Security |
| DAOO | Duty Airport Operations Officer |
| DAS | Dario Air Services |
| DATMO | Duty Air Traffic Management Officer |
| DFS | Daily Flight Schedule |
| DG | Director General |
| DM | Duty Manager |
| DO | Duty Officer |
| DS | Day Shift |
| EIA | Entebbe International Airport |
| EJAF | Entebbe Joint Aviation Facility |
| ESSAT | Equipment Serviceability Status Audit Team |
| ETA | Estimated Time of Arrival |
| ETD | Estimated Time of Departure |
| FOD | Foreign Object Damage/Deposit |
| GHA | Ground Handling Agent |
| GM | General Manager |
| GM-EIA | General Manager - Entebbe International Airport |
| ICAO | International Civil Aviation Organization |
| IS | In-Coming Shift |
| MAEPD | Manager Aerodrome Engineering, Planning& Development |
| MAM | Manager Aerodrome Maintenance |
| MAM-Log | Marshallers Aircraft Movement Logbook |
| MO | Manager Operations |
| MONUSCO | United Nations Mission for the Stabilization of Congo |
| NAS | Entebbe Handling Services |
| NOTAM | Notice To Airmen |
| NS | Night Shift – A, B, C or D |
| O/C ANS | Officer in Charge, Air Navigation Services |
| OS | Outgoing Shift |
| Pax | Passengers |
| PBB | Passenger Boarding Bridge. |
| POB POO/AS | People On Board Principal Operations Officer Airside Services |
| PTMO | Principal Transport and Mechanical Officer |
| R/T | Radio Telephony |
| RFFS | Rescue and Fire Fighting Service |
| SL | Shift Leader |
| SMS SOO/AS/OPS SOO/AS/ SCAD | Safety Management System Senior Operations Officer Airside Services/Operations Senior Operations Officer Airside Services/safety compliance, audit & documentation |
| STD | Scheduled Time of Departure |
| TPBB | Telescopic Passenger Boarding Bridge. |
| VIP | Very Important Persons |
| VVIP | Very, Very Important Persons |
| WHO | World Health Organization |
| WHMA | Wildlife Hazard Management Assistant |
| WHMO | Wildlife Hazard Management Officer |
| WHMS | Wildlife Hazard Management Section |

### Definitions:

| Word | Meaning |
| --- | --- |
| Accident: | An occurrence associated with the operation of an aircraft which, in the case of a manned aircraft, takes place between the time any person boards the aircraft with the intention of flight until such time as all such persons have disembarked, or in the case of an unmanned aircraft, takes place between the time the aircraft is ready to move with the purpose of flight until such time as it comes to rest at the end of the flight and the primary propulsion system is shut down, in which: - a person is fatally or seriously injured as a result of: - being in the aircraft, or - direct contact with any part of the aircraft, including parts which have become detached from the aircraft, or - direct exposure to jet blast, except when the injuries are from natural causes, self-inflicted or inflicted by other persons, or when the injuries are to stowaways hiding outside the areas normally available to the passengers and crew; or - the aircraft sustains damage or structural failure which: - adversely affects the structural strength, performance or flight characteristics of the aircraft, and - would normally require major repair or replacement of the affected component, except for engine failure or damage, when the damage is limited to a single engine, (including its cowlings or accessories), to propellers, wing tips, antennas, probes, vanes, tires, brakes, wheels, fairings, panels, landing gear doors, windscreens, the aircraft skin (such as small dents or puncture holes), or for minor damages to main rotor blades, tail rotor blades, landing gear, and those resulting from hail or bird strike (including holes in the radome); or - the aircraft is missing or is completely inaccessible. |
| Aerodrome | A defined area on land (including buildings, installations, and equipment) intended for use wholly or in part for the arrival, departure and surface movement of aircraft. |
| Aircraft | Any machine capable of deriving support in the atmosphere from the reaction of the air. |
| Aircraft Damage | Any adverse condition which affects the structural strength, performance or flight characteristics of the aircraft or causes delay in flight operation due to repairs. |
| Aircraft Stand | A designated area in an apron intended to be used for parking an aircraft |
| Airport | A defined area on land (including buildings, installations, and equipment) intended for use wholly or in part for the arrival, departure and surface movement of aircraft. |
| Aerodrome Traffic | All traffic in the maneuvering area of the airport and all the aircraft flying in the vicinity of the airport. |
| Airside | The movement area of an airport, adjacent terrain and buildings or portions thereof access to which is controlled. |
| Airside Driving Permit | The card evidencing the authorization issued by Manager Operations to the holder to drive vehicle/equipment in the airside of Entebbe International Airport in accordance with the category/color codes of vehicles/equipment specified therein |
| Apron (Ramp) | A defined area in an aerodrome, intended to accommodate aircraft for the purposes of loading or unloading passengers, mail or cargo, fueling, parking or maintenance. |
| Apron Traffic | All aircraft, vehicles, equipment, and pedestrians using the apron of the airport. |
| Blind Transmission | A transmission from one station to another, when a two-way communication cannot be established, and it is believed that the called station (party) can hear transmission but is unable to transmit. |
| Aircraft Parking Plan | White board written indicating expected flights of the day |
| Airport Users | Passengers, airlines crew, staff, contractors, and visitors |
| Airside Vehicles | Operations vehicles, usually equipped with R/T Communication |
| Configuration Chart | A chart of the apron, giving recommended dimensions of aircraft parking stands |
| Crosswalk (Zebra -crossing): | Any portion of a road, on an apron or any other area designated by a sign or surface marking as a pedestrian crossing. |
| Control Tower | A unit established to provide Air Traffic Control Service to airport traffic. |
| Designated Vehicle Corridor | A road delineated by surface marking on an apron. |
| Facility Damage | Any damage or adverse condition which prevents or limits the use of an aircraft handling facility or requires repairs. |
| Equipment | Any motor vehicle or mobile device, either self-propelled or towed or of a specialized nature, used for runway and airfield maintenance or in the maintenance, repair and servicing of aircraft including cargo and passenger handling equipment. |
| Equipment Damage: | Any damage or adverse condition, which limits or prevents the use of aircraft handling equipment. |
| Guide Vehicle | A vehicle with a rotating red light /Beacon, used to guide aircraft or other vehicles (e.g., Follow Me). |
| Hold Clear | Instructions to hold at least 45m (150’) from the edge of the runway while waiting for permission to cross or proceed onto a runway. |
| Incident | An occurrence other than an accident, associated with the operation or handling of an aircraft, which affects or could affect safety of operation. |
| Injury | Any condition, which requires medical assistance, including first aid. |
| Fatal Injury | Any injury, which results in death within 30 days of the incident/accident. |
| Light Signal from Airport Control Tower | A light used by Control Tower to control airport traffic when there is no radio communication. It is also referred to as Aldis Lamp. |
| Maneuvering Area | That part of the aerodrome intended to be used by aircraft for the taking off and landing, excluding apron. |
| Mayfly | Document indicating scheduled/non-scheduled flights to operate on a given day |
| Movement Area: | That part of the aerodrome to be used for the surface movement of aircraft and includes the maneuvering areas and aprons. |
| Municipal Skip | Large receptacle for collection of airport garbage pending final disposal by the Municipality |
| Notice of Infringement or Composition: | A report made on any violations of the traffic rules and the licensing regulations set forth herein by Manager Operations. |
| Off-block time | Time when chocks are removed from the wheels of an aircraft ready to move |
| On-block time | Time when chocks are inserted onto the wheels of a stationary aircraft. |
| Operational Stand: | An area on an airport apron designated for parking of aircraft for the purpose of loading and unloading passengers, cargo, and provision of ground services. |
| Perimeter Road: | That road within the airside that allows vehicles to move around the airport clear of the apron, taxiways, and runways (movement areas). |
| Property Damage: | Any damage or adverse condition, which limits or prevents the use of a structure or building, which requires repair. |
| Restricted Area: | An area of the airport designated by a sign or barrier as an area to which access by persons or a vehicle requires the production of a valid identification. |
| Runway Vacated: | Indicates a vehicle, at least 45m (150ft) to the side of the nearest edge of the runway in use, wherever practical. |
| Taxiway: | That part of an aerodrome used for maneuvering aircraft and airport equipment between the apron area and the runway. |
| Threshold: | The beginning of that portion of the runway usable for landing. |
| Traffic Signs: | These include all signals, warning signs, posts, direction posts, signs, lines, humps or other devices for guidance or direction of persons using the aerodrome. |
| Vehicle Corridor: | Parallel 0.25m, wide, solid white lines spaced 12.6m apart, on the ramp, immediately in front of the terminal building, intended to provide guidance to vehicles and equipment operators traversing from one side of the apron to another. |
| Vehicle Operator: | The person responsible for the operation and safety of vehicle and equipment usually referred to as driver. |
| Warning Devices: | A siren and flashing yellow, blue, or red light mounted mainly on the cab of the vehicle. |

Part 1: Apron Management

# Overview of Apron Management Functions

Apron Management Service is a unit in airport operations department mandated to regulate activities of various stakeholders at airside to ensure safety.

It contributes towards reductions of hazards at the apron and responsible of coordinating the interaction of the Movements of aircraft, vehicles, equipment, airport employees, and passengers, with different demands without compromising safety.

To achieve its mandate, apron management service is actively engagement in the following:

      - Aircraft Stand Allocation and Management to ensure sufficient clearance Between the Aircraft, Vehicles or Buildings.

      - Enforcement of Airside SOPs and Other Regulations.

      - Vehicle/Equipment Control and Inspection.

      - Accident/Incident Investigation.

      - Emergency Planning and Response.

      - Airside Works Control.

      - Aircraft Surface Guidance.

      - Provision of Standard Aircraft Marshalling Signals.

      - Operation of Passenger Boarding Bridges.

      - FOD Management.

      - Periodic Inspections and Audits of the Movement Area.

      - Apron Safety Risk Management.

      - Airside Safety Promotion done to improve the Safety Culture and coordination of various activities to ensure smooth and safe airside operations.

# Airside Administrative Structure:

# Names and Contacts of Persons in Charge of Airside Safety:

| Name | Title | Telephone |  | Email |
| --- | --- | --- | --- | --- |
|  |  | Office Hours | After Working Hours |  |
| Christopher Omondi | MO | +256 414 353017 /2230 | +256 758 483681 +256 772 483681 | comondi@caa.co.ug |
| George Etolu | POO/AS | +256 414 352293 | +256 771 806510 | getolu@caa.co.ug |
| Elly Otafiire | POO/AF |  | +256751766093 | eotafiire@caa.co.ug |
| Senior Airport Operations Officer/Airside Services | SOO/AS | +256 414 353201 +256 414 353028 | +256 700 419417 +256 772982170 | OpsAirside@caa.co.ug |
| Apron Management | AOO/AS | +256 414 353261 +256 414 353648 | +256 759 609 484 | OpsAirside@caa.co.ug |

# Personnel Requirements for Airside Operations:

Qualifications and Training Requirements: the MO is responsible for determining operations department staff qualifications, training and staffing requirements for purposes of adequate manning and satisfactory level of proficiency. Details entry and on job trainings requirements for all positions are outlined below.

Formal Training:  operations staff training shall cover both theoretical and practical on-the-job training. This has been designed to enable staff to acquire and maintain the skills required to perform their duties confidently and competent. Staff shall undertake recurrent training as per Training program and plan.

Training and Competency Assessment of new Staff:

On recruitment, a staff shall undertake the following trainings and competency assessments before deployment to work independently.

- Orientation and induction training:

This shall be carried out by a designated trainer who will submit a training report to POO and MO,

- Initial formal training:

The staff shall undergo an initial training in the following training areas; aircraft handling marshalling techniques, basic airport operations, aerobridge operations.

- On job training:

- After the above trainings, the staff shall undertake on job training for six months and shall have a monthly assessment in the areas of airside safety operations and procedures, aircraft marshalling and bridge operations.

- The staff shall be trained by a qualified and competent OJT Instructor in the specific area.

- After six Months of OJT, a Staff shall be deemed to have successfully completed OJT on attaining a pass mark of 70% in competency assessment in each area of airside safety operations, bridge operations and aircraft marshalling.

- The competency assessment check is performed using the staff proficiency checklist form

- On successful completion of OJT, a staff can be deployed to work independently.

- Training records and competency assessments shall be maintained at the office of head of section and reports sent to MO.

Performance Assessment:

The Purposes is of maintaining an initially acquired competency, assessment on every staff shall be conducted once a year. Individual records of the assessment shall be maintained at the Office of the head of section and reports sent to the MO. Impromptu proficiency checks may be conducted on any staff to always ensure satisfactory level of proficiency.

The Staff Proficiency Check Form is attached in the Appendixes.

# Procedure for Continued Competency of Operations Department personnel:

Operations personnel shall be trained on any new procedures, equipment, and systems prior to the discharge of their duties. Proficiency on aircraft marshalling techniques shall be checked every after six months using Form 1: UCAA/EIA/AM/APP1/02/F01. This is meant to ensure continued competence of the staff. The training may be conducted in any of the following forms:

- Local training.

- International training.

- Seminars/Fellowships and workshops.

- Refresher training.

- Factory training.

- Pre-implementation training and instruction.

- Mentoring.

## Entry Requirements for Airside Operations Section:

|  |  | Airside Operations |  |
| --- | --- | --- | --- |
| 1 | Principal Operations Officer/ Airside services | Entry Requirements | On Job Recommended Trainings Within Four Years |
|  |  | A master’s degree from a recognized university in relevant field. OR A bachelor’s degree from a recognized university in relevant disciplines with a postgraduate qualification in Ramp Operations and management and experience of not less than 10 years in Airside operations and safety management including 4 years at Senior Supervisory level. Sound knowledge of ICAO SARPS, Regulatory requirements. Sound knowledge of CAA regulations, procedures, and practices Proven leadership and people management skills Must have ability to work and coordinate with stakeholders. Must have a working knowledge of computer. Should be between 35 to 55 years of Age. Age Limit does not apply for internal adverts | - Ramp operations and management - Integrated safety management systems - Aeronautical studies - Advanced Aerodrome Design and Operations - Aircraft Accident and incident Investigation techniques and Management - Civil Aviation Management program. - Advanced airport operations and management - Root cause analysis - Disaster/Crisis/Emergency Planning & Business Continuity - Aerodrome safety internal auditor - Advanced aerodrome standards - Airport Safety during Aerodrome Works - Runway safety - Aviation human factor Training. - Terminal Operations and mgt - Redeclaration of runway distances - Aviation Leadership program. - Effective aerodrome risk Assessment and management - Safety Management Systems implementation. - Training developers’ course - International Trainer of trainers’ course (TOT) - Aerodrome Certification and compliance - Marshalling techniques - Radio telephony techniques - Airfield inspection techniques - Procedure dev’t & Implementation - Passenger boarding bridges operations - Ramp safety and Operations - ACI-ICAO Annex 14: Implementation - Apron control - Advanced ramp Safety Management Systems |
| 2 | Senior Operations Officer/ Operations/Safety and Compliance | Entry Requirements | On Job Recommended Trainings Within Four Years |
|  |  | A relevant bachelor’s degree from a recognized University and 8 Years work experience at a Supervisory level with relevant trainings in Ramp Operations and Management and Proven leadership skills. OR A Diploma with a minimum of 15 Years work Experience at airside Operations, 4 of which should have been at a Supervisory level. Must be proficient in Computer applications. Sound knowledge of ICAO SARPS and regulatory requirements. Proven leadership and people management skills Must have ability to work and coordinate with stakeholders. Must have a working knowledge of computers. Should be between 30 to 55 years of Age Age bracket does not apply for internal adverts. | - Ramp operations and management - Integrated safety management systems - Aeronautical studies - Aircraft Accident and incident Investigation techniques and Management - Advanced airport operations and management - Aerodrome Safeguarding and OLS - Emergency Planning & Business Continuity - Aerodrome safety inspector - Basic aerodrome standards - Airport Safety during Aerodrome Works - Runway safety - Aviation human factor Training. - Redeclaration of runway distances - Effective aerodrome risk Assessment and management - Safety Management Systems implementation. - Training developers’ course - International Trainer of trainers’ course (TOT) - Apron control - Aerodrome Certification and compliance - Marshalling techniques - Radio telephony techniques - Airfield inspection techniques - Procedure dev’t & Implementation - Passenger boarding bridges operations - Ramp safety and Operations - ACI-ICAO Annex 14: Implementation - Apron control - Supervisory management skills - Primary Air Traffic Control Aerodrome control - Basic Airport operations |
| 3 | Operations Officer/Airside services | Entry Requirements | On Job Recommended Trainings Within Three Years |
|  |  | An honor’s bachelor’s degree in a relevant field 5 years relevant work experience including at least 3 years in Airport operations Must have worked in a supervisory capacity for 2 years. Training in Basic Airport operations, Marsahing techniques, Aerobridge operations; Airfield inspection Airport Safety Customer Care Training in people skills and leadership Should be between 30 to 50 years of Age. Age bracket does not apply for internal adverts. | - Ramp operations and management - Integrated safety management systems - Aeronautical studies - Aircraft Accident and incident Investigation techniques and Management - Advanced airport operations and management - Aerodrome Safeguarding and OLS - Emergency Planning & Business Continuity - Aerodrome safety inspector - Basic aerodrome standards - Airport Safety during Aerodrome Works - Runway safety - Aviation human factor Training. - Redeclaration of runway distances - Effective aerodrome risk Assessment and management - Safety Management Systems implementation. - Training developers’ course - International Trainer of trainers’ course (TOT) - Apron control - Aerodrome Certification and compliance - Marshalling techniques - Radio telephony techniques - Airfield inspection techniques - Procedure dev’t & Implementation - Passenger boarding bridges operations - Ramp safety and Operations - ACI-ICAO Annex 14: Implementation - Apron control - Supervisory management skills - Primary Air Traffic Control Aerodrome control - Basic Airport operations |
|  | Assistant Operations Officer/Airside services | Entry Requirements | On Job Recommended Trainings Within Three Years |
|  |  | A Honors Diploma from a recognized institution from a recognized institution. OR A Level with Seven (07) year relevant experience in aircraft Marshalling and training in the following: marshalling technique, aerobridge operations and basic airport operations. A bachelor’s Degree is an added advantage. Must possess valid driving permit, computer skills and basic training in customer service and safety. Should be between 24 to 35 years of Age. Age bracket does not apply for internal adverts | Marshaling techniques Basic airport operations Aerobridge operations Airfield inspection |

# Apron Management Procedure:

Procedures and work instructions are written as a formal process of specifying the way of performing a given duty, the airside Standard Operating Procedures (SOPs) are tailor-made to guide, particularly Airport Operations staff deployed in this area, on who is to do what, when where and how a given task is to be carried out correctly.

## Procedure for Shift Handover/Takeover:

Purpose: to ensure smooth changeover.

Scope: from the time, outgoing shift hands over to the time incoming shift takes over.

Responsibility: POO

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| SL/OS (OO/AS) | 1 | Fill the shift handover form using Form 2: UCAA/EIA/AM/APP1/02/F02 |
|  | 2 | Hand over all Office equipment, tools or items and operational vehicles to the incoming SL. |
|  | 3 | Briefs the in-coming shift on events and current NOTAM that affect airside operations. |
|  | 4 | Signs in the Office logbook, indicating hand over done |
| SL/IS | 5 | Receive Shift handover form, records incoming personnel present, and countersigns to acknowledges that the hand over process was properly done. |
|  | 6 | Sign in the Office logbook, indicating taking over shift. |
|  | 7 | Briefs team on expected duty load and how to manage the parking plan. Deploys staff on the following tasks: PBBs operations, aircrafts Marshalling, movement area Inspection, enforcement, facilitation, FOD BOSS cleaning, coordination of apron cleaning with contractors and VVIP operation. (Apron 5, ACT and Apron 4 management) |
|  | 8 | SL incoming shift verifies and confirms serviceability status against the hand over by checking the vehicles, Office equipment, PPPs, and tools. |
|  | 9 | Consult GHA Duty Managers and review the DFS and the aircraft parking plan, as necessary. |
|  | 10 | Submits the hand over Form - CAA/DAAS/ OPS/ASOP/011 to SOO/AS-OPS. |
| SOO/AS-OPS | 11 | Compiles key issues from handover form and bring them to POOs attention. |

Generated Records:

- Hand-over record form -

- Daily Incidents Record…Incident Logbook/Incident Reports

Performance Measurements

- Number of flights safely handled.

Number of compliments/complaints

# Procedure for Manpower Scale Down During Pandemic:

Purpose: to protect staff from pandemic and avoid depletion of experienced and trained staff due to pandemics

| Action by | Step | Activity |
| --- | --- | --- |
| WHO and Ministry of health | 1 | Declares a public health emergency due to outbreak of pandemic |
| WHO and Ministry of health | 2 | Asks organizations to scale down manpower due to pandemic |
| DG | 3 | Asks directors and managers to scale down on manpower |
| DAAS, GM, MO | 4 | Direct heads of sections to cut down their respective manpower |
| POO, SOO/AS, OO/AS | 5 | Meet to determine manpower to be scaled down |
|  | 6 | Personnel deployed at apron five will be scaled down by 50% |
|  | 7 | Personnel deployed at apron four and two will be scaled down by 50% |
|  | 8 | Personnel deployed at apron tower will be scaled down by 50% or 100% if need be and the responsibility clearing engine start and pushback reverted to ATM |
|  | 9 | Personnel deployed at apron one shall be retained 100% |
|  | 10 | Personnel deployed and dedicated to office hour operations will be scaled down by 50% |
|  | 11 | NB scaled down shall be dictated by traffic and prevailing directives |

Generated Records:

- List of manpower scaled down

- List of staff retained

- Directives

- Pandemic declaration

Measurement Indicators:

- Number of retained staff

- Number of aircraft safely handled

- Number of staff scaled down

# Procedure For Coordination Between ATM and ACT:

Purpose: ensure coordination and enforcement of Apron Safety.

Scope: Progressive round the clock.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| Shift Leader | 1 | Having received Estimated Time of Arrival (ETA) of each arriving aircraft giving detailed operational information, shall advise the ATM of the aircraft daily stand allocated and any other safety issues as early as possible |
|  | 2 | Keep a listening watch on frequencies: 121.9 for ground operations and 118.1MHz to monitor communication between Tower and Aircraft to keep informed of progress of air traffic and in case of or during emergency. |
|  | 3 | The means of communication between ATC and ACT are: VHF 121.9. Intercom ACT ext. 3028, Tower 3250; |

Generated Records:

- Parking plan

- MAM-Log and AODB

Measurement Indicators:

- Number of flights recorded in each period

- Number of aircrafts parked in each period

# Procedure for Aircraft Stand Allocation:

Purpose: provide aircraft safety clearance and enhance efficient ground handling operations at the apron.

Scope: progressive round the clock for each in-bound flight until it has parked.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| Shift Leader (OO-AS/AOO) | 1 | - Using the flight schedule, ensure daily allocation of aircraft stands. Form 3: UCAA/EIA/AM/APP1/02/F03. - Coordinate with GHAs on any changes or ad hoc flight - Draws up the daily aircraft parking plan and briefs the AOO for implementation |
|  | 2 | - Inspects the whole apron to establish aircraft operational stands available using Form 4: UCAA/EIA/AM/APP1/02/F04. - Write out an aircraft parking plan. - Allocate expected aircraft according to the daily plan based on the Daily Flight Schedule, Apron Maintenance Schedule, Airport Development schedule, and available Stand dimensions |
| AOO/AS | 3 | - Receive and writes details of the in-bound flight from AIMO/ATMO in the MAM-Log and/or input in Airport Operations Database (AODB) - Informs DATMO the allocated stand for in-bound flights, on tel. Ext. 3250 or R/T 121.9 MHz/ tetra radio. |
| SOO/OO/AS/SL | 4 | - Reviews aircraft stands allocated, if acceptable, gives AOO/AS a go ahead to implement the plan. - If not, an alternative arrangement is discussed, desirably with GHA or any agent concerned. |

Generated Records:

- Record of the aircraft parking plan

- MAM-Log/AODB

Performance Measurement

- Number of aircraft Movement

- Complete AODB Records

## Apron Parking Reference Chart:

To be inserted

## Apron Stop Marks:

Aircraft Safety Line

B777, B787, A330, A340, A350, B747-800

C (Code E)

B (Code D)

B767, B757, A310

A (Code C)

B737, E170, E190, A319, A320, A321, C130, B727

## Apron Stop Mark Legend:

- Stop Marking (A) Code C = A 319; A 321; A 320; B 737-301-901; E 170, E190.

- Stop Marking (B) Code D= A 330-201; Dc 10–30; B 767–201; B 757:201-301, A 301 – 601.

- Stop Marking (C) Code E= B747:201-801; B767–301; A330-301; A340:501–601, B777:201-301.

# Procedure for Parking and Aircraft:

## Marshalling of incoming Aircraft:

Purpose: to park aircraft safely by using correct signals and safety clearances.

Scope: from the time, ATC hands over traffic to apron control until departure.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
|  | 1 | Ensures GHAs position equipment to the allocated stand at most 20 minutes prior to the scheduled aircraft arrival |
| 3 AOO/AS OO/AS SOO/AS | 2 | Inspects the allocated stand for the arriving aircraft 10 minutes early, for commercial flights and 15 minutes for VIP flights, to ensure that the stand is clear of any FOD/Obstacle; |
|  | 3 | Waits on the stand for the aircraft to taxi into the apron, guided by the pavement markings provided; |
|  | 4 | Marshal/lead in incoming Aircraft using correct/standard signals |
|  | 5 | Remains on the stand to observe the initial safe positioning of the ground handling equipment until the aircraft doors have been opened; |
|  | 6 | Caution! at any stage of aircraft marshalling, if the aircraft Movement is at risk, the marshaller on duty shall signal to the aircraft to stop and therefore allow for a safe alternative to be exercised; |
|  | 7 | Record chocks-on/on-block time |
|  | PAKING AIRCRAFT OUTSIDE STAND LEAD IN LANE |  |
|  | 1 | Inspects the stand to determine adequacy of the available space to accommodate a given aircraft using Form 4: UCAA/EIA/AM/APP1/02/F04. |
|  | 2 | Determines proximity to the fuel hydrants |
|  | 3 | Deploys wing walkers with appropriate training on both sides of the aircraft to ensure safety |
|  | 4 | Uses nearby stop marks and hydrant pits to determine where to stop the aircraft depending in the aircraft code |
|  | 5 | Ensures appropriate corning is done for aircraft safety and compliance to the safety separation distances |
|  | PARKING AIRCRAFT NOSE IN ON THE WESTERN PART OF APRON FOUR |  |
|  | 1 | Sends a request to down grade RWY 12/30 to only taxiway to DSSER for approval if the aircraft to be parked on apron 4 west code is C to E |
|  | 2 | Determines aircraft to be parked nose out depending on available space and wingspan |
|  | 3 | When the request is approved, engages wing walkers to assist in parking aircraft nose out |

Generated Records:

- Filled MAM-Log/AODB

- Incident reports.

Measurement Indicator

- Number of aircraft Movements.

- Number of reported incident/accidents.

## Marshalling of Outgoing Aircraft:

In case of marshalling aircraft out of their parking stand/position, the following procedures are used:

| Action By | Step | Activity |
| --- | --- | --- |
| AOO/AS, OO/AS, SOO/AS | 1 | Look out for the aircraft anti- collision lights start flashing |
|  | 2 | Monitor the safe withdrawal of equipment, GHA personnel and Airline operations and maintenance technician, noting areas likely to be affected by the blast or prop-wash of the departing flight and create a safe route to access (taxi-lane) the taxiway; effect clearance of these areas |
|  | 3 | Look out for the pilot’s signal from the cockpit, ‘ready to start engine number 1, 2, 3, 4 |
|  | 4 | Marshal out aircraft using correct/standard signals |
|  | 5 | Record chocks-off/off-block time in the MAM-Log and AODB |

## EIA Aircraft Parking and Docking Chart:

To be inserted

# Procedure for Allocation of TPBBS:

Purpose: to ensure effective TPBB usage.

Scope: progressive round the clock

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| AOO/AS, OO/SL | 1 | Inspects the TPBBs using Form 5: UCAA/EIA/AM/APP1/02/F05. |
|  | 2 | Determines appropriate stand to allocate each expected aircraft for the usage of the TPBB in accordance with size of the aircraft; grandfather policy and first come first serve where applicable. |
|  | 3 | Writes out aircraft parking plan for the day, indicating stand allocation for all aircraft expected to utilize the TPBBs; review when necessary |
| AOO/AS | 4 | Writes details of the in-bound flight in the MAM-Log and AODB |
| SOO/AS, OO/AS | 5 | Reviews stand allocated for TPBB usage: if acceptable, AOO/AS goes ahead to implement; if not, an alternative arrangement is made. |
| AOO/AS, OO/AS | 6 | Implements agreed plan and informs DATMO the allocated stand for the in-bound flight on tel. Ext. 3250 or R/T 121.9 MHz |

Generated Records:

- parking Plan Record/Stand Inspection Record

- MAM-Log/AODB

Measurement Indicators:

- Number of docking hours recorded in each period

# Procedure for Operating TPBBS:

Purpose: Safe and effective TPBB usage.

Scope: progressive round the clock

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| Pre-Arrival TPBB Testing |  |  |
| Operator | 1 | Check surrounding area under the telescopic passenger boarding bridge (TPBB) to ensure that nothing is left within the TPBB safety zones (TSZ) 20 minutes before ETA |
|  | 2 | Check the cleanliness of both the interior and exterior of the TPBB every after the departure of a serviced aircraft |
|  |  | If found unclean, liaise with public utility to provide cleaning contractor to clean up the TPBB. Extend the TPBB for cleaning and retract thereafter to the parking position |
|  |  | - Check the air-conditioning system for serviceability. - If found to be unserviceable report the fault to technical personnel to rectify the fault; |
|  | 5 | - After the checks, switch on TPBB A/C 20 minutes prior to arrival. - If at night, switch on external flood lights after docking the PBB on the aircraft fuselage. The TPBB interior lights will automatically switch on after docking |
|  | 6 | Ensure that the safety strap behind the roll-door is hooked on the safety switch. |
|  | 7 | Turn the key switch from off to on position at the operating post or desk (control console); |
|  | 8 | Choose aircraft type and height to pre-position the TPBB (R1 and R5 switches) |
|  | 9 | Adjust swing cabin towards the arriving aircraft stand |
|  | 10 | Test the cabin roll-door up and down |
|  | 11 | Test-drive the TPBB forward and reverse into asl position, using the control command or joystick in on position of ‘‘Point & Go’’ (R9 Switch); |
| Docking TPBB on Arriving Aircraft |  |  |
|  | 14 | When the aircraft enters the stand, open cabin roll-door (by pushing it up) and immediately take position at the cabin operating post |
|  | 15 | Dock the TPBB after receiving clearance from the Maintenance engineer/Flight coordinator. |
|  | 16 | Remove the safety belt and wait for the GHA/Airline official to open the door and insert the safety shoe below the A/C door |
| TPBB Back-Off Procedure |  |  |
|  | 17 | Take position in the cabin of the TPBB and waits for the aircraft door to be closed. This is done when after the re-fueling process has been completed |
|  | 18 | Retract the canopy from the fuselage of the aircraft, initially 3 meters for the back-off process to start. This after closure of the aircraft door. |
|  | 19 | Retract and park the TPBB in its parking position/slot. |
|  | 20 | Switch off Appliances (AC, etc.) and lock service doors. |

Note:

- Exercise caution not to over steer the TPBB wheels beyond 90 degrees limit, either way.

- When the limit switch has registered a warning, e.g., ‘‘near right switch’’, that flashes as ‘‘tunnel a’’ slides into the cabin. Turn the joystick (point & go) to your right, lest the tunnel hits the air conditioning unit.

# Procedure for VVIP Operations at EIA:

Purpose: to ensure safe handling of VVIPs

Scope: at all times of VVIP arrival and departure.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| VVIP Pre-Arrival Arrangements |  |  |
| GM-EIA | 1 | Organizes inter-agency preparatory meeting. |
| MO/SOO/AS-OPS | 2 | Draws operational plan for VVIP handling |
| VVIP Arrival Procedures |  |  |
| OO/AS, SOO/AS | 3 | - Ensures that Follow Me is clean, secure, and kept sterile once screened all the time. - On receiving ETA from ATM/SFC, the marshaller does the following: - Organizes transport for VVIPs handling staff: Immigration; Customs; Joint Security Personnel; VIP Operations Staff; Carpet Layers to VVIP terminal. - A pre-arranged staging area for Follow Me is organized in conjunction with ATM to wait for the backtracking VVIP aircraft. - Follow Me leads the VVIP aircraft giving space of about 50 meters. - At the carpet area, the AOO/AS takes over from the Follow Me and parks the aircraft using correct/standard marshalling signals with assistance of the signaler at the red carpet who aligns, and signals stop when the aircraft door aligns with the carpet by trajectory viewing. - Relocate the aircraft for servicing or secured parking after disembarkation of the principal and offloading of luggage either by use of a push back5 minute - procedure or on her own power. C) VVIP using Commercial Flights: In case, a VVIP using a commercial flight that terminates at Apron I, the following is done: - Principal disembarks first before other passengers. - Lead the VVIP pre-arranged motorcade to VVIP terminal via runway 12/30. |
|  | VVIP Departure Procedures: |  |
|  | 4 | - Monitor the arrival of the Principal and Position at Parking Stand. - Monitor the pilot’s request to remove chocks and oblige - Marshal out the aircraft using correct/standard signals and hand over the standby Follow Me - Follow Me request ATC lead the taxiing aircraft to the RWY in use from VVIP terminal to RWY 17/35 and vacate RWY via A3. |

Generated Records:

              - MAM-Log/AODB

Measurement Indicator:

            - Number of facilitated escorts in a month during VVIP operations and AODB Records.

# Procedure for Code ‘F’ Aircrafts at EIA:

Purpose: to ensure safe handling of Code F aircraft.

Scope: from the time of arrival until the departure time

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
|  |  | Arrival Procedure |
| AOO/AS, OO/AS, SOO/AS | 1 | Receives ETA from AIMO/ATMO and passes on the designated parking stand to the ATMO. Stages follow me at TWY A1, A3, A4 waiting for the arriving aircraft and thereafter leads her to the designated parking stand |
|  | 2 | Inspect a designated area to provide safety separations; using Form 4: UCAA/EIA/AM/APP1/02/F04. |
|  | 3 | The Marshaller parks the A/C, with the help of wing walkers so that the tail fin is within the safety line. |
|  | 4 | Departure Procedure: The aircraft is either pushed or towed to taxi lane for onward Movement. |

Generated Records:

- Air Transport Clearance Forms

- MAM-Log/AODB

Measurement Indicator:

            - Number of facilitated parking in cases of diversion routings at a given time.

# Allowable Overload operations at EIA:

Pavement can sustain a definable load for an expected number of repetitions during its design life. As a result, occasional minor overloads are acceptable. Overload operations are allowed at EIA under the following circumstance:

- Where magnitude and Frequency don’t require detailed analysis.

- Where there is limited loss in pavement life expectancy.

- Where the pavement rehabilitation programme is soon to take place

## The following are used to determine overload operations:

- For flexible pavements, occasional movements by aircraft with ACN not exceeding 10 per cent above the reported PCR should not adversely affect the pavement.

- for rigid or composite pavements, in which a rigid pavement layer provides a primary element of the structure, occasional movements by aircraft with ACN not exceeding 5 per cent above the reported PCR should not adversely affect the pavement.

- The annual number of overload movements is limited to below 5 per cent of the total annual aircraft movements.

## Disallowable Overload Operations at EIA:

Overload operation shall not be permitted under the following circumstances:

- Pavements exhibiting signs of distress or failure.

- When the pavement rehabilitation programme is not in the near future.

## Processing of Requests for Overload Aircraft Operations:

Upon receipt of request for Aircraft operations with the ACN above the declared PCN at Entebbe International Airport,

    - MO, shall forward a request to MAM for evaluation of Overload Operations.

    - Using feedback from MAM, MO shall communicate allowing or rejecting the Overload Operation to the aircraft operator.

# Procedure for Control of Aircraft Engine Runup:

Purpose: to control noise level that affect human health because of engine run-up.

Scope: from start to end of aircraft engine run-up.

Responsibility:  POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
|  | 1 | Designated location for engine run-up is the turn pad on RWY 30 |
| AOO/AS, OO/AS, SOO/AS | 2 | Requests to use turn pad on RWY 30 for engine run-up. When permission is granted BY ATC |
|  | 3 | Inspect the turn pad on RWY 30 using Form 6: UCAA/EIA/AM/APP1/02/F06. |
|  | 4 | Inspect and ensure that the area around the aircraft engines is free of obstacles which may be affected by the engine air intake and the blast; |
|  | 5 | Call ARFFS on R/T 121.9 MHz or Tel. Ext. 3328/3322 and asks the personnel to provide standby fire cover throughout the engine run-up; |
|  | 6 | Ensures regulated use of perimeter road behind RWY 30 when the engine run up is taking place to avoid jet blast |
|  | 7 | Monitors the duration given for engine run-up (maximum 10 minutes) and liaises with GHA: - If the engine run-up is affecting other operations, stops it. - If the period elapses, discontinues the exercise; - If maximum/full engine run is required, requests to use RWY 17/35 from ATC |
|  | 8 | Writes brief notes of the engine run-up in the daily incident logbook. |

Generated Records:

Incident Logbook.

Measurement Indicator:

- Rate of incidents pertaining to engine run-up noise per 1010 aircraft Movements

# Procedure for Aircraft Push Back at EIA:

Purpose: to ensure safety of the aircraft during push back procedure

Scope: from the time of request to safe departure of parked A/C

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| AOO/AS, OO/AS, SOO/AS | 1 | Upon receipt of aircraft push back notification by ATC or on seeing anti- collision lights prior to push back, AOO/AS, OO/AS And SOO/AS oversees pushback activities by GHA and records the off-chocks time and any safety violations |
|  | 2 | Conducts post push back stand inspection to ensure; FOB bins are emptied and FOD removed, equipment are withdrawn from the stand, any oil spills are recorded and cleanup is done. Records the findings |
| ACT | 3 | Upon receipt of request for a push back clearance by the Capt. in command, the apron controller checks the activities around the aircraft then clears the push back process to commence. |

Generated Records:

- Aircraft Movement schedules

- MAM-Log/AODB

Measurement Indicator:

            - Number of facilitated push backs per day.

# Procedure for Maintaining Aircraft Movement Records:

Purpose: to keep accurate records of aircraft Movements.

Scope: every time all times there is aircraft Movements at the aerodrome.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| AOO/AS | 1 | Write the aircraft parking plan according to the Seasonal Flight Schedule; |
|  | 2 | Receive ETA from AIMO/DATMO and records details in the MAM-Log/AODB |
|  | 3 | Passes the allocated stand number to DATMO on R/T 121.9 MHz or Tel. Ext. 3250/2250. |
|  | 4 | Parks incoming aircraft according to the Aircraft Parking Plan. |
|  | 5 | Up-dates the MAM-Log and the AODB accordingly for both incoming and outgoing aircraft |
| OO/AS | 6 | Checks both the MAM-Log and AODB to ensure that the records are maintained up to date daily; |
| SOO/AS/OP | 7 | Uses the MAM-Log and AODB records to compile the weekly report to POO/AS and MO for further management |
| MO/POO/AS | 8 | Uses the report to compile DAAS management and EXCO brief |

Generated Records:

- MAM-Log/AODB

- Airside weekly report

- DAAS Management and EXCO briefs

Measurement

            - The number of aircraft Movements for a given period.

# Procedure for Facilitating Movement of Passengers, Cargo, and Other Stakeholders at the Airside:

Purpose: to ensure safe expeditious facilitation of airport users.

Scope: at all times of Airside Movement.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| AOO/AS | 1 | Park passenger aircraft closer to the Passenger Terminal Arrival Hall and boarding gates to facilitate quick passenger access; |
|  | 2 | Park cargo aircraft closer to the Bonds to facilitate convenient and quick conveyance of cargo. |
|  | 3 | Park VIP aircraft closer to the VIP Lounge Access gate and for VVIP aircraft, park on the functional stand of apron 2 in coordination with SFC. |
|  | 4 | Provide airport users with airside vehicles to access remote aircraft stands, other airside sites and in special cases outside the airside, when operationally required. |
| OO/AS/SL | 5 | Review aircraft parking plan and always stand allocation to ensure smooth traffic flow. |
|  | 6 | Approves requests to use airside vehicles to access Movement Area by airport users. |
|  | 7 | Approves requests to use airside vehicles to perform duties that promote airside safety and security; |

Generated Records:

- Marshallers occurrence logbook

- Vehicle Movement log sheets

Measurement Indicator:

- Rate of fuel consumption by airside vehicles.

## Leader Vehicle and Services:

EIA has leader van and services deployed at the airside to carry out the following functions:

        - Follow-Me

- Carry out apron inspection.

- Guide aircraft in Low Visibility on the apron and along taxiways.

- Guide aircraft arriving for the first time at EIA to the designated aircraft stand

- Guide VVIP aircraft to/from the Movement Area to/ from the apron parking stand

- Guide aircraft relocating to/from one apron/ aircraft stand to/from a remote apron/aircraft stand

- Lead aircraft towing activities on the Movement Area.

- Escort vehicles on Movement Area during construction works/work in progress and maintenance sights sites.

- Escort emergency vehicles to the site in case of actual emergency.

- Act as a command centre/post during emergencies.

- Provide transport to and from the Movement Area.

- Acts as a lead vehicle for other vehicles / equipment entering Movement Area without VHF radios

        - Follow Me 3, Follow Me 4

In addition to the follow-me vehicle, another vehicle called Alpha Charlie Mobile (Airport Operations) is used to carry out duties of follow-me. It also carries the following activities:

- Act as a collection centre for the collected /gathered foreign object debris that may be swept/ picked from the Movement Area in cases of emergencies for the quick recovery of airport operations e.g., Dry grass that may be blown by the engine blast.

- Escorting of vehicles on Movement and safety areas during construction works/work in progress and maintenance sites.

- Used for monitoring all Airside activities.

- Carrying out aircraft stand pre-use inspection.

# Course Curriculum for airside Safety Training:

Purpose: improve overall airside safety performance and operational efficiency

Course Content: The course is divided into two levels.

Level 1: Basic Airside Safety

This level is offered to new staff and everyone who works at Airside for orientation and covers the following:

- Airside safety regulations

- Development of safety culture

- ICAO phraseology

- Identification of airside hazards

- Bird/wildlife hazards.

- Fire hazards

- Airside accident/incident reporting

- Human factors in airside operations

- Operation of vehicles/equipment in the Movement Area

- Airside markings and signage

- Personal protection and occupation

- Emergency situations

- Foreign object debris (FOD) prevention programme

- Apron layout

Level 2:         ADP Training

This level is offered to equipment/vehicle operators, and it covers the following:

- Introduction to airside safety, overview and the role of ICAO safety culture

- Operation of vehicles in Movement Area

- Duties and responsibilities of vehicle operators

- Employer and employee responsibilities

- Apron safety procedures

- Moving around aircraft

- Emergency procedures during aircraft fuelling,

- Airside accident/incident reporting

- Radio procedures and ICAO phonetics

- Runway incursions

- Safety rules around the PBB

- Pavement markings and signage

- Removal of a disabled aircraft

- Fire prevention and the handling of extinguishers

- EIA Layout

- Foreign object debris (FOD) prevention programme

Part 2: Apron Safety Management

# Basic Airside Safety Rules:

## Airside Driving permit:

- Every driver operating within the airside, must hold a Valid Airside Driving Permit (ADP) authorizing them to drive a vehicle/equipment of that description; and must always carry it when operating a vehicle on the airside.

- All airside personnel whose duty requires them to drive/operate vehicle/equipment may apply for the issue of an airside driving permit with the following other requirements: valid airport pass and state driving license and must have attended the ADP course and scored the pass mark or higher.

- The classification of the equipment/vehicle or ADP colour code is as follows:

      - Red Colour = for all Apron Equipment.

      - Green Colour = Fuel Dispensers, Pushback, High loaders, Low Loader, Ambulift, Pallet Loader, Mobile Pax steps, Catering Truck, Lorry, Conveyor Truck, Fire Trucks.

      - Blue = Ramp Bus, Coaster.

      - Brown Colour = for Cars, Vans and Pick-Ups, Forklifts, Towing Tractors, Electric Tugs, Station Wagons.

- The airside driving permit is issued by MO based on applicant’s knowledge of both the Uganda Road Traffic Rules and the EIA Airside Vehicle Standard Operating Procedures after sitting for a written test.

- Application for the     ADP must be made to MO in writing with the following details:

  - Official address of the applicant.

  - Reasons for the application.

  - One copy of the applicant’s valid state driving license

- On expiry of an     ADP, the permit holder shall forthwith return the permit to MO who will withdraw or revoke any issued if the above regulations are not complied with. The MO is the process owner and will therefore have the authority to suspend, withdraw or cancel the permit at any time.

## Speed Limit:

Within the airside every driver shall obey all regulatory signs and unless otherwise indicated by the sign, adhere to the following speed limits:

- 5km per hour within 15 metres of a parked aircraft.

- 25km per hour on the vehicle corridor in the vicinity of an aircraft operational stand – apron speed limit.

- 40km per hour on the perimeter roads away from the aircraft operational stand

## Exemptions on Speed limits:

- The following vehicles/equipment are exempted from the above-mentioned speed limits:

    - All vehicles responding to aerodrome emergency with a beacon.

    - All lead vehicles.

- These may leave the established vehicle corridor, if necessary, when responding to an emergency.  Nevertheless, these vehicles described above, must yield the right of way to taxiing aircraft and aircraft under tow.

## Safety in the Vicinity of Aircraft:

- Vehicle

- Vehicles/equipment must give way to always moving aircraft, even when they are under- tow.

- Vehicle/equipment operators must be aware of safety distances when driving in the vicinity of parked aircraft.

    - 5m clear from the wing tip of parked aircraft.

    - 15m clear of a re-fuelling aircraft.

- No vehicle shall drive over any hose or bonding cable laid on the ground by fuelling agents during aircraft re-fuelling.

- Vehicles and persons must remain:

    - 5m radius clear from the aircraft tank vents.

    - 7.5m clear in front of an operating aircraft engine.

    - 76m clear behind an aircraft with engine idling.

- Vehicles must not be driven or parked under aircraft fuselage or aircraft wings.

- Vehicles that need to be at the turf (grass) area besides the taxiway shall maintain a minimum of 45 metres from the edge of the taxiway.

- Personnel

          - Do not approach an aircraft, which has its engines running. Aircraft with engines running will display a flashing red beacon (anti-collision light);

          - Operator shall not start their vehicles within 15m of an aircraft, which is being refuelled.

          - Use appropriate hand signals to guide the vehicles.

          - Operators of vehicles shall alert the ground engineer should it be necessary to approach the aircraft for whatever reason when it is ready for push back or has engine run.

          - No person or vehicle shall cross a lane if an aircraft is taxiing within 200m.

          - A minimum safety distance of at least 250m must be maintained behind an aircraft taxiing under its own power.

          - Be aware of wet surfaces. Reduce speed and check your brakes.

          - Always set the hand brake on parking your vehicle.

          - Secure all objects during strong wind or storm.

          - Report all fuel, oil and other chemical spillages to airside operations immediately to apron management office.

          - Know your vehicle’s dimensions.

          - Always switch off the engine of your vehicle or equipment when it is not in use

          - Pick any FOD sighted on the apron and dispose it off in the provided trash/FOD bins.

## Protection from Jet Blast:

The figure below shows the safe distances from jet blasts released because of aircraft Movements on the apron and Movement Area.

Figure 1: Minimum Safe Distances of Taxiing Aircraft

Figure 2:  Safety Distances from Aircraft

## Helicopter Movements and Danger Areas:

Anybody involved in the servicing operations who approaches a helicopter must have undergone training specific to helicopter operations and must observe the following:

- Never drive inside of/ or below the rotor disc whether the rotor is turning or not. Stay clear of the tail rotor, too.

- Never approach a helicopter until it has stopped, the wheels chocked, anti-collision lights switched off, engine shut down and the rotor stopped.

- You must always get the pilot’s permission to approach the helicopter.

- Be aware of the rotor disc. In certain weather conditions you can lose sight of the rotor disc edge or rotor tips.

- Know your vehicle height and dimensions.

- Always check your brakes before approaching a helicopter.

- Check the ground surface condition and slope and make allowances for stopping.

## Around Aircraft Points to Remember:

- Do not drive behind an aircraft with the engines running.

- Do not walk or drive in front of a running engine, allow at least 7.5m of running engine to avoid suction by engines and harm by propellers

- Do not approach an aircraft until it has stopped moving the wheels have been chocked and the engines shut down.

- Do not drive or park under aircraft wings.

- Do not park outside designated places.

- Do not obstruct the pushback area.

- Put all FOD in bins provided.

- Pour all waste oil in FOD bins provided.

- Report all fuel, oil and other chemical spillage.

- Keep clear of leading and trailing edges of the Aircraft wing.

- Use a guide man/Marshaller when reversing Equipment/Vehicle.

- Do not leave a vehicle unattended when engines are running;

- Must maintain vehicle corridor except when servicing aircraft.

- Do not traverse aircraft stands.

# Procedures for Apron Safety Management:

Procedure for Airside PPE:

Purpose: to ensure staff are protected against injury and noise while at the airside.

Scope: always the personnel are deployed at airside.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| Employers | 1 | Ensure that all personnel working in airside must be provided with safety wear/gear: high visibility/reflective jackets/vest/tabard, band, overcoat, helmet and/or socks; safety boots/shoes; ear defenders/muffs; etc. All personnel handling hazardous materials must be provided with PPE e.g., Helmet for re-fueling and construction site. |
| All Airside Staff | 2 | Staff working at the airside shall at all times put on PPE while carrying out their duties. |
| AOO/AS, OO/AS | 3 | Check to ensure that staff operating at the airside put on safety precaution/protection wear/aid. |
|  | 4 | Any Violator will be issued a violation form, penalty and his/her ASDP/ASP be withdrawn. |
| SOO/AS/SCAD | 5 | Gathers facts and punches ADP/ASP for each violation made. For third violators, punches ADP/ASP and hands over to MO for further action |
| GM-EIA | 6 | Exemptions: the following categories of people are exempted from putting on HVVs: - Security/Protocol personnel escorting VVIPs |

Note: in your own interest, be seen!

Generated Records:

- Occurrence logbook

- Incident occurrence report

- Violation forms

- Penalty receipt book

Performance Measurement/Indicators:

- Rate of violation

Procedure for inspection of Movement Area:

Purpose: To continuously monitor the conditions of Movement Area and aerodrome facilities.

Scope: Movement Area and associated facilities.

Responsibility: POO/AS

| Action by | Step | Activity |
| --- | --- | --- |
|  |  | Inspection of Aprons |
| AOO/AS, OO/AS | 1 | Inspect the Aprons at the start of shift using a checklist (Apron Form 1) |
|  | 2 | Fills the Apron Inspection Form |
|  | 3 | If apron is contaminated: - SOO/AS/SC writes a report to concerned officer to mitigate unsafe conditions - Call utility supervisor on tel. +256 704 217 320, Ext. 3392/3143 who contact the cleaning contractor to deploy their staff to clean the areas affected expeditiously. - Monitor activities/actions undertaken to restore safety |
| OO/AS | 4 | Ensures that a special inspection is carried out immediately after any accident/incident/works or any other conditions that may affect aviation safety – Part V: 60. |
|  |  | Inspection of Manoeuvring Areas |
| OO/AS, AOO/AS | 5 | - Verifies serviceability of the vehicle and communication equipment. - Request for permission to access manoeuvring areas from ATMO on R/T 121.9 MHz and only proceed after receiving permission ATMO - Inspect the manoeuvring Area using a checklist; Form 6: UCAA/EIA/AM/APP1/02/F06. - Record status in the Manoeuvring Area Inspection Logbook. |
| SOO/AS/SC | 6 | Write a report to concerned officer to mitigate unsafe conditions |

Generated Records:

- Daily Apron Inspection Form

- Manoeuvring area inspection form

- MALB – Manoeuvring Area Logbook

- Accident/incident report

Measurement Indicators:

- Number of field Manoeuvring area forms

- Number of accident/incident reports in each period.

Procedure for inspection of Movement Area:

Special manoeuvring area inspection will be done under the following conditions:

- After an aircraft incident/accident on the runway

- After construction works

- After any other time where there are conditions at the aerodrome that may affect aviation safety

Procedure for Reporting Runway Surface Condition:

Purpose: Ensure runway surface condition is reported to pilots whenever it rains.

Scope: from the time it rains to the time runway surface is dry for all the three thirds of the Runway.

Responsibility: Shift leader Airside operations/OO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| AOO/OO/AS | 1 | Receives and records runway surface condition assessment from WHM section as per the GRF at 30-minute intervals starting from the onset of rain until all the three thirds of the Runway are found dry again |
| OO/ AOO/AS | 2 | quality assures the Runway condition code and immediately (not more than 5 minutes) shares the runway surface condition assessment report on e-mail to AIS Uganda copying ATM, NOTAM office, briefing office, OpsAirside@caa.co.ug and GRF@caa.co.ug |
| OO/AS | 3 | Signs and Files a copy of the RCR sent out |
| OO/ AOO/AS | 4 | Receives feedback AIREPs from ATMO and Uses the feedback to request a review and update of the runway surface condition report from WHM and forwards to Manager aerodrome maintenance (MAM) conditions that may require rectification copying GRF@caa.co.ug |
| MAM | 5 | On receiving a feedback report from AOO/OO/AS, MAM takes action to address the anomaly identified; |

Generated Records:

- Runway surface condition report recordings

- Runway surface condition report

- Runway surface condition feedback

- Action taken on the feedback

Measurement Indicators:

- Number of runway surface condition report recordings

- Number of runway surface condition report

- Number of Runway surface condition feedback

- Number of Anomalies identified, and action taken

Forms:

- Runway surface condition report form

Rain

Are all the three thirds dry

Stop

Send report to airside operations and ATM on 121.9

Is report correct

Re-asses Runway and review condition Report

WHM carries out runway inspection asses the condition and make a report

Airside send Report to ATM by 121.9 and email to AIS Uganda copy to UCAA briefing, GRF group NOTAM office, briefing office, OpsAirside@caa.co.ug.

From feedback, is Runway dry?

Stop

No

No

No

Yes

Yes

Yes

Flow Chart

# Procedure for Use of Stands 20-25 on Apron 1:

- Procedure for Coordinating Ramp Handling/Vehicle/Equipment movements To Aircraft Stand 20-25 on Apron One.

Purpose: Ensure coordination and enforcement of Apron Safety.

Scope: progressive round the clock.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| AOO-AS/, OO-AS | 1 | Having received Estimated Time of Arrival (ETA) of an arriving aircraft, shall enter Estimated Time of Arrival (ETA) in Airport Operations Data Base system and aircraft movement logbook with expected parking plan. |
|  | 2 | Advises Ground Handling Agents (GHAs) of the aircraft parking stand on specified apron with any other safety issues as early as possible. |
|  | 3 | Ensures Vehicles and Ground handling equipment access stand 20-25 through the designated service road near the airport rescue and firefighting services (ARFFS) |
| ATMO | 4 | Ensure Crossing of taxiway C2 is controlled on VHF frequency 121.9 via communications |
| AOO-AS/, OO-AS | 5 | Ensure Vehicle/equipment movements around stand 20-25 follow the approved routes |
| ATMO | 6 | Ensure Movement of vehicle/equipment on the back of aircraft stand 20 – 25 access road is controlled on VHF frequency 121.9 |

Procedure For Coordinating Aircraft Parking on Stand 20-25:

Purpose: Ensure coordination and enforcement of apron Safety.

Scope: progressive round the clock.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| OO/AS | 1 | Reviews daily schedule on takeover |
|  | 2 | Coordinates with GHA’s on the intension of all special /ad-hoc flights |
|  | 3 | Prepares aircraft parking plan for the day |
| AOO/AS, OO/AS | 4 | Having received Estimated Time of Arrival (ETA), shall enter information on AODB and aircraft logbook. |
|  | 5 | Passes parking stand and or apron to ATM for every arrival |
|  | 6 | Ensures GHAs position equipment at most 20 minutes prior to the scheduled aircraft arrival |
|  | 7 | Inspects the aircraft stand and or apron 10 minutes before aircraft arrival and advises ATM on any safety concern as early as possible. |
|  | 8 | Parks her/aircraft using standard ICAO signals |
| ATMO | 9 | Ensure all aircraft access stand 20-25 through only TWY ‘C2’, ‘C3’, then Apron 1 Taxi Lane’’ but not through TWY ‘B’ except during emergencies and prior coordination with airside operations through ATM has been made |
|  | 10 | Ensure all aircrafts exit stand 20-25 ONLY through TWY ‘B’ except during emergencies and prior coordination with airside operations through ATM has been made |
|  | 11 | Ensure all Aircraft enter and exit apron one ONLY through TWY ‘D’ from TWYs ‘C2’, ‘C3’, AND not through stand 20-25 except during emergencies and prior coordination with airside operations through ATM has been made |
| AOO/AS, OO/AS | 12 | Ensure all aircraft park on stand 20-25 “Nose out” facing West of airfield except during emergencies and prior coordination with airside operations has been made |
| All operators | 13 | Ensure aircraft parked on stand 20-25 are secured by the operators/owners against strong wind and probable jet blast effect. |
| Ground Handlers/Head set Operators | 14 | Ensure all aircraft push backs are aligned on apron taxi-lane before engine power-up starts |

Generated Records:

- Parking plan

- MAM-Log

- AODB

Measurement Indicators:

- Number of flights recorded in each period

- Number of aircrafts parked on stand 20-25 in each period

# Procedure For Use of Apron Five:

Purpose: Ensure coordination and enforcement of apron Safety.

Scope: progressive round the clock.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| AOO/AS | 1 | Reviews daily schedule on takeover |
|  | 2 | Coordinates with GHA’s on the intension of cargo flights and others to be parked on apron 5 |
|  | 3 | Prepares aircraft parking plan for the day |
|  | 4 | Allocates the parking stands to aircraft on a first come first serve basis depending on the type of aircraft and the size of the stand |
| OO/AS | 5 | Reviews and approves parking plan and ensures that when a multi aircraft stand 51 is allocated to aircraft, no other aircraft is allocated on stand 51A and 51B. |
|  | 6 | Ensure that only code A-C are allocated on subsidiary stand 51A and 51B |
| AOO/AS, OO/AS/SL | 7 | Having received Estimated Time of Arrival (ETA), shall enter information on AODB and aircraft logbook. |
|  | 8 | Passes parking stand and or apron to ATM for every arrival |
|  | 9 | Ensures GHAs position equipment at most 20 minutes prior to the scheduled aircraft arrival |
|  | 10 | Inspects the aircraft stand 10 minutes prior to aircraft arrival and Advises ATM on any safety concern as early as possible. |
|  | 11 | Parks her/aircraft using standard ICAO signals |
|  | 12 | When parking code F aircraft on stand 51 ensure that no other aircraft shall be parked on stand 51A and 51B while stand 51 is in use, wing walkers will be used to ensure safety. |
|  | 13 | Ensure subsidiary stand 51A and 51B are used only when 51 is not in use. |
|  | 14 | Monitor aircraft push backs to ensure safety. |
|  | 15 | Ensure that equipment and delivery vehicles accessing apron 5 is only through the designated service road except when there is an emergency or when there is prior approval by ATC. |
|  | 16 | Approves idle power engine run-ups on the apron of up to 5 minutes. Full engine runs shall be done only in an approved location (Runway threshold). |
|  | 17 | Ensure that vehicles/equipment maintain the vehicle corridor except while servicing the aircraft |

Generated Records:

- Parking plan

- MAM-Log

Measurement Indicators:

- Number of flights recorded in each period

- Number of aircrafts parked on apron 5 in each period

# Procedure For Removal of Contaminants in Movement Areas:

Scrubbing And Cleaning of Movement Area

Purpose: to always ensure a clean and safe Movement Area.

Scope: from identification of contaminants to their safe handling and clearance.

Responsibility: Principal Operations Officer.

| Action by | Step | Activity |
| --- | --- | --- |
| AOO/AS, OO/AS | 1 | In case of mud, contact the cleaning contractor to scrub; and then RFFS to wash with water |
|  | 2 | In case of spillage, - Informs polluter to immediately cleaning, if not done, Calls public utility to ask the cleaning contractor to undertake immediate use of absorbents to contain and/or clean or scrub the contaminated area with a detergent and a charge will be imposed on the airline/GHA/Fuelling company. - The used absorbent must be collected properly from the paved surfaces and disposed it off safely. Then ask RFFS to undertake the flushing of the fuel residues spilled immediately. - Inspects the spot to determine quality of cleaning/ scrubbing done: - If the work is complete and thorough, declares the aircraft stand (spot) useable/ serviceable. - If not, asks public utility to inform cleaning contractor to undertake further cleaning/ scrubbing using absorbent, or other form of detergent; |
|  | 3 | Investigates the equipment responsible for the spillage to determine the cause: - If the defect is corrected/rectified, allows the equipment to continue being operated on the apron. - If the defect is likely to persist, orders the equipment to be withdrawn from operations. |

Sweeping of the Movement Area

| Action by | Step | Activity |
| --- | --- | --- |
| AOO/AS | 4 | - Make routine inspection of Movement Area and reports the presence of FOD, and areas having wear and tear to OO/AS, using Movement Area Inspection Logbook and checklist. - Checks aircraft stand prior to the arrival of the flight - Maintaining good housekeeping practices identifying areas contaminated by FOD and reports to OO/AS; |
|  | 5 | Inspect each stand after a flight has departed and before another one parks, hand-picks any deposit of trash, debris, etc. And reports to SOO/AS/SC areas infested with FOD; |
|  | 6 | Contacts Public Utility Supervisor to deploy cleaning contractors immediately to the affected spot Facilitate cleaning contractor to access the Movement Area where cleaning is required and supervises the work to ensure it is done well and expeditiously. |
|  | 7 | Monitor cleaning contractors to ensure they adhere to acceptable safety requirements; |
|  | 8 | The Frequency of Sweeping the Movement Area is: - Daily. - Monthly – the FOD BOSS. - There is also Ad hoc sweeping done as and when need arise. |

Generated Records:

- Manoeuvring Area inspection Form

- Occurrence Logbook

- Inspection Reports.

- Incident/Accident Reports.

- Quarterly Reports

Measurement

- Rate of incidents involving apron spillage per 1010 aircraft Movements.

- Rate of aircraft damage caused by FOD per 1,010 aircraft Movements.

# FOD Management:

EIA has an FOD Prevention Program aimed at establishing policy, responsibilities, and measures to prevent damage to aircraft operations. Refer to attachment 2 for details.

- Procedure in the Event of Spillages:

General: this procedure applies to all spills including Aviation Fuel, Hydraulic Oil, Chemical and Sewage.

Procedure for Fuel Spill:

- Inform Apron Management on +256 312 353201/61 or VHF 121.9 or ARFFS on +256 312353322/3328). Do not drive through the spilt liquid.

- Inform ARFFS for fuel spillages covering an area of at least 4m² since it constitutes a hazard for immediate cleanup. And it will attract a fee.

- In case of Hydrant spillage on Apron 1, Inform EJAF if spillage is from underground pit on Tel. +256 752 793 063, Total Duty Line at +256 752 793042, or +256 752 793 099, as well as Apron Management and ARFFS.

- In case of hydrant spillage on Apron 01 and 05, inform Tristar on +256 788 141 035

- Activate the fuel cut off switch at the affected section installed at strategic places at the Apron, if your conversant with the system operation.

- EJAF activate emergency fuel supply procedure by cutting off the flow to the hydrant system.

- Do not start/switch on engines of motorized equipment within spill areas before the spill is cleaned up. However, if the engine is already running and the hazard to personnel is diminished gradually, the vehicle may be driven out of the area.

- Remove Ground Equipment/Vehicle from within 15m of the spillage area.

- Cordon off the spillage area

- Divert traffic

- The cleaning contractor conducts immediate clean-up using an absorbent or any other appropriate detergent as a charge is levied on Airline/GHA/EJAF/Tristar

Hydraulic Oil Spillage:

- Mop or apply absorbent material except sawdust. This is done by the equipment or vehicle. The absorbent material must then be collected properly and disposed of safely.

- Inform Apron Management in case of heavy oil spillage requiring scrubbing the affected area giving the following Details:

    - Name and Tel contact.

    - Location and extent of the spill.

    - Effects of spill to aircraft and persons.

Dangerous Substances:

- In case of breakage or mishandling, spillage of dangerous good occur, follow the Emergency procedures for Dangerous goods (4.15.5 – Actions for Dangerous Goods Emergency Response) of the Aerodrome Manual.

- Report all substances spillage to ARFFS Tel. +256 312353322 and tell them:

    - Your name and location.

    - The name of the substance if you know it.

    - The quantity spilled.

    - Keep other people away from the area until help comes.

## Aircraft Fueling procedures:

Purpose:

- To provide guidance to minimize hazards arising out of fuelling operations.

- To a certain extent applies to maintenance of aircraft fuel system

Scope: at all times of handling and dispensing fuel and maintenance of the system.

Responsibility: POO/AF

Definition: for the purpose of this procedure the following definitions shall apply:

- fuelling: shall mean dispensation of fuels and power boost fluids into an aircraft for the generation of engines power and augmentation of thrust and shall include both fuelling and defueling.

- fuelling zone or precautionary area is regarded as the area extending 6 metres radially from the aircraft fuelling point, venting point, and fuelling equipment.

General

- No persons shall fill or replenish the fuel tanks of an aircraft from vehicles or vessels containing petroleum in bulk or from fuel hydrant installations except from vehicles or installations of a type approved by approved oil marketing company at EIA.

- The fuel and other products delivered by the oil companies shall be of a quality already agreed to between the purchaser and the supplier but shall conform to approved specifications.

- All the statutory rules regarding transportation, storage and handling of aviation fuels and similar products and for illumination of the fuelling zone as agreed by civil aviation authority shall be followed by the concerned personnel.

- No leaking or defective fuelling equipment shall be used.

Supervision of fuelling:

- The fuelling of an aircraft shall be done under the supervision of a person nominated by operator or as specified in the quality control manual and shall be carried out by authorized person from the oil company.

- The operator's representative shall ensure that the fuelling of an aircraft is carried out in accordance with the prescribed manner, following the conditions laid down by the aircraft manufacturer.

- The approved personnel shall not Move from the place of fuelling leaving the aircraft and fuelling vehicles unattended.

Fuelling Conditions

- The aircraft operator shall furnish written instructions to the fuelling company regarding the correct procedure of fuelling and precautions to be taken for types of aircraft.

- These instructions shall include the fuelling pressure, rate of delivery, etc. Special precautions, such as in the case of switch re-fuelling shall also be communicated to the oil company in advance.

- All special conditions and requirements of aircraft manufacturers during their servicing and fuelling shall be followed by all concerned.

fuelling Place

- The re-fuelling of an aircraft shall be done in an open place so designated. As a general guide, the Fuelling places should be at least 15 metres away from the nearest building except those parts of building, which are constructed for the purpose of direct loading/unloading of aircraft.

- The aircraft shall not be fuelled within 30 metres of radar equipment under test or in use in aircraft or ground installations.

- The re-fuelling shall be carried out on a level surface as far as possible.

- The nominated person shall ensure that there is adequate restraint of the aircraft by checking that the brakes are applied, and the wheel chocks are placed properly before the fuelling operations are started.

Approach to an Aircraft

- While placing the re-fuelling equipment oblique approach to the aircraft shall be followed.

- The equipment driven shall be marshalled in position. Similarly, withdrawal of re-fuelling equipment shall be done under supervision.

- The re-fuelling equipment shall never be reversed towards the aircraft.

Positioning of Fuelling Equipment

- The re-fuelling equipment shall be placed in such a way that a clear exit path is maintained all round the equipment and from the aircraft to allow its quick removal in case of need.

- In such cases when the fuelling vehicle is parked under the aircraft, it shall be ensured that any accidental spillage from the aircraft air vent does not fall on the Fuelling equipment.

- The vehicles and equipment shall not be placed where they may cause obstruction to the evacuation of persons from occupied portions of the aircraft in case of an emergency.

Manning of Fuelling Vehicles

- Every re-fuelling vehicle shall be manned by at least one competent person.

- The operation of the vehicle shall be done only by the competent person(s).

- Adequate manpower shall be available to the competent person(s) to shut off the flow of fuel and or manoeuvre/retreat the equipment in case of an emergency.

Fuel Quantity Delivery

- The exact quantity of fuel in terms of volume along with its breakup distribution in various tanks if desired shall be detailed to the oil company representative.

- The oil company representative shall issue a delivery receipt indicating the quantity of fuel refuelled in the aircraft.

- The density of fuel supplied shall be declared on this receipt.

Approval of Fuel Samples and Starting of Fuelling

- Fuelling of an aircraft by the oil company shall be started only after the fuel samples have been approved by the aircraft operator and a clearance has been given to the oil company to start the fuelling operations.

- It shall be ensured that the fuel samples are free of water and other contaminants, for instance by use of approved water detecting device.

Special Precautions To Be Taken In The Fuelling Zone:

- Within Fuelling zone, the use of naked lights or operation of switches on lighting systems of other than approved pattern shall be forbidden.

- Smoking is not allowed at any part of airside at EIA.

- Within Fuelling zone, smoking, the use of naked lights or operation of switches on lighting systems of other than approved pattern shall be forbidden.

- Unless Fuelling takes place in a designated `No Smoking Area', `No Smoking` signs shall be predominantly displayed not More than 15 metres away from the Fuelling equipment and aircraft tank vents.

- Persons engaged in Fuelling operations shall not carry matches or other means of ignition. They shall also not use footwear with exposed iron or steel studs, nails, or lips.

- Equipment with all metal wheels or rod capable of producing sparks shall not be Moved in the Fuelling zone while Fuelling is in progress.

- Aircraft borne auxiliary power units (APUs) which have an exhaust influx discharging into the zone shall be started before filler caps are removed or Fuelling connections made.

- No auxiliary power unit in the Fuelling zone shall be started in the Fuelling zone when Fuelling operation has been commenced. If an APU has stopped for any reason, it shall not be started when the Fuelling is continuing and there is a risk of fuel vapour ignition.

- Ground power units (GPUs) whenever used shall be positioned not less than 6 metres from the aircraft filling and venting points and the Fuelling equipment.

- Only approved vehicles and equipment shall be run in the Fuelling zone. These vehicles and equipment shall be subjected to regular inspection and maintenance to preserve their safety characteristics

- Exhaust of the re-fuelling vehicle shall not pass over the Fuelling hose intake hose.

Safety Precautions Against Static Electricity Discharge Bonding and Earthing

- During Fuelling, the prevention of fire risk due to static electricity discharge is dependent upon efficient bonding between the aircraft and the fuel supply source. Ideally, earthing should be through apron earthing points designed for this purpose. Drag chains and conductive tyres normally accepted for earthing purposes are seldom effective under all climatic conditions.

- The aircraft, fueller, hose nozzle, filters, funnels, or any other appliance through which fuel passes shall be electrically bonded throughout the Fuelling operation.

- Connection shall be made to designated points on clean and un-painted metal surfaces of the aircraft and the fueller.

- Before transfer of fuel commences, following procedures shall be carried out in respect of bonding the fueller and the aircraft.

- Whenever apron earthing points exist the aircraft and Fuelling equipment shall be earthed to them.

    - The aircraft shall be effectively bonded to the Fuelling equipment.

    - Whenever wing Fuelling is employed, the nozzle of the hose shall be bonded to the aircraft structure before filler cap is removed.

    - In case of pressure Fuelling, metal to metal contact between the aircraft filling and the Fuelling hose coupling shall be ensured. In addition, a bonding pin or clip from the Fuelling hose coupling to the aircraft structure near the fuel receptacle may be used to provide additional safeguard.

    - When re-fuelling from drums, barrels or similar equipment is carried out, similar precautions shall be taken to bond the pumping equipment, hose nozzle and fuel container.

    - Whenever funnels are used, they shall be bonded both to the nozzle of the hose or can and to the aircraft. If a chamois leather is used, the metal ring around the leather shall be bonded to the funnel.

    - The fuel valve shall not be opened before bonding connections are made. Similarly, at the end of the Fuelling operations, first the fuel valve shall be closed, fuelling coupling disconnected or the Fuelling nozzle removed and then only shall the bonding connections be broken.

    - All cables, clips and plugs used for bonding or earthing shall be maintained in proper condition and regularly tested for electrical continuity and a record thereof maintained.

Safety Precautions Against Fire Hazard During Re-fuelling:

- When any part of undercarriage assembly has become abnormally heated during landing or taxying, the aircraft shall not be fuelled until heat has dissipated. Fuelling equipment shall not be positioned until the operator's representative has satisfied himself that there is no risk from heated components.

- In checking for high temperatures, care shall be taken in approaching the wheels. The wheels shall be approached from the fore or aft and never from the sides.

- Fire extinguishers of adequate capacity and approved type shall be readily available before the transfer of fuel begins. The straps securing them to the re-fuelling equipment shall be unfastened before the Fuelling operations are started. Adequate number of trained persons shall be readily available to operate these units.

- The fire extinguishers shall be maintained to the standards recommended by their manufacturers.

- Fuelling crew shall be instructed in the procedure for summoning the aerodrome firefighting and rescue services.

- Fuelling operation shall cease when a turbo prop jet aircraft manoeuvres to bring the rear jet outlets within 43 metres of the Fuelling equipment or the aircraft Fuelling point and vent system.

Safety from Electrical/Storms/Heavy Rain

- Extreme care shall be exercised while fuelling during electrical storms.

- Over wing Fuelling shall not be carried out during electrical storms.

- Fuelling shall be suspended during severe lightning disturbances, heavy storm/rain on/in the vicinity of the aerodrome.

Hazards from adjacent Aircraft Operation

- Before and during Fuelling, it shall be ensured that no hazard arises to the personnel or equipment from the efflux from other aircraft or apus. If the hazardous conditions exist, fuelling operation shall immediately be suspended until conditions permit resumption of fuelling operations.

- Note: it may be noted that the engine efflux of modern jet aircraft when taxiing could have speeds up to 65 knots and a temperature above 50 degrees Celsius even at 30 metres from the exhaust point. This temperature may not be dangerous from the fire point of view, but the personnel and the equipment could be affected.

Maintenance of Re-fuelling Vehicle and Equipment used

- All vehicles, their engines and equipment shall be subject to regular inspection and maintenance to preserve their safety characteristics.

- The engine, the electrical and exhaust systems of such vehicles shall not make any sparks or flames. Vehicles moving in the danger zone shall be fitted with spark arrestors and flame traps.

Use of Photographic Flash Equipment

No photographic flash bulbs and electronic flash equipment shall be permitted to be used within 6 metres filling or venting points of an aircraft.

Operation of Radar

Aircraft shall not be fuelled within 30 metres of radar equipment under test or in use in aircraft or ground installations.

Specific Precautions in Case of Fuel Mixtures

- Mixture of wide cut and kerosene turbine fuels could result in the air fuel mixture in the tank being in the combustible range at common ambient temperatures during Fuelling. The risk of this type of sparking would be minimum if antistatic additive is present in fuel.

- In cases of fuel mixtures, a reduced rate of Fuelling shall be adopted. The amount of reduction in flow rate needed is dependent upon the Fuelling equipment in use and the type of filtration employed on the aircraft Fuelling distribution system.

Servicing and Maintenance of Aircraft during Fuelling

- Aircraft or other maintenance work which may create a source of ignition shall not be carried out while Fuelling is in progress.

- All ground equipment, such as platform steps shall be clear of the aircraft. Because after Fuelling, the aircraft tends to settle down due to increase in weight and this changed attitude of the aircraft may thus cause damage to the aircraft/such equipment.

- The main engines of the aircraft shall not be operated. Operation of APUs & GPUs shall be strictly in accordance with applicable sops.

- Only those switches which are essential in carrying out the Fuelling or servicing operation shall be operated when fuelling in progress.

- Strobe lighting which is likely to give spark shall not be operated.

- All connections between the ground equipment and the aircraft shall be made before the filler caps are removed and shall not be broken until the Fuelling ceases.

- In case of the use of battery trolleys, the electrical circuit shall remain unbroken till the Fuelling operation is completed.

- Vehicles operating in the Fuelling zone shall not pass under or park near the aircraft unless specifically required to do so for maintenance or Fuelling purposes.

- Aircraft combustion heaters shall not be used.

- Only checking and limited maintenance work such as the exchange of units shall be carried out on radio, radar, and electrical equipment. Testing of such equipment shall be deferred until the Fuelling is completed.

- Radio equipment of approved types installed on Fuelling and servicing equipment may be operated.

- All hand torches and inspection lamps and their cable connections used in the Fuelling zone shall be of explosion proof/intrinsically safe type.

- Only authorized persons and vehicles shall be permitted within the Fuelling zone, and their number shall be kept to the minimum.

Fuelling with Passengers Aboard, During Embarkation and Disembarkation:

To reduce transit time and for security reasons, sometimes airlines allow passengers to embark and disembark or remain on board during Fuelling operations. The decision to allow passengers to embark, disembark or remain on board during Fuelling is the responsibility of the airline and following precautions shall be exercised by the airline concerned:

- Fixed wing aircraft with a seating capacity of less than 20 shall not be permitted to be refuelled with passengers on board.

- Passengers are warned that Fuelling will take place and that they shall not smoke, operate switches, or otherwise produce sources of ignition.

- The `No Smoking' and `exit' signs are illuminated.

- Provision is made for safe evacuation of passengers via at least two of the main passenger embarkation and disembarkation doors in the event of the emergency. A responsible person should be positioned at each door to supervise evacuation of passengers, if needed.

- If during Fuelling, the presence of fuel vapour is detected in the aircraft interior, or any other hazard arises, Fuelling should be stopped.

- Ground servicing activities and work within the aircraft should be conducted in such a manner that they do not create a hazard or obstruct exits.

- Access to and egress from the areas where aircraft escape chutes may be deployed should be kept clean.

- When passengers are embarking or disembarking during Fuelling, their route should avoid areas where fuel vapours are likely to be present and be under the supervision of an Airline Official or Representative. No Smoking' should be enforced strictly during such passenger Movements.

- Monitoring to ensure that safety precautionary measures are in place before, during and after re-fuelling is done by the Marshaller having received information about aircraft re-fuelling with passengers on board.

Fuel Spillage

- In the case of a spillage covering an area greater than 5 square metres, fuelling operations shall stop. All persons shall be evacuated from the affected area to a place at least 15 metres from the spillage. Movement of persons and vehicles in the affected area shall be avoided. It shall be ensured that all activities are restricted to reduce the risk of ignition.

- Engines of vehicles within 6 metres of a spillage shall not be started until the area is declared safe.

- Prior to recommencing Fuelling, action must be taken to clean the spilled fuel. Fuel must not be washed into sewers or drains.

Re-fuelling of Helicopters:

- Because of design features of helicopters namely proximity of fuel intake and tanks to the passenger compartments, it is recommended that passengers should not be allowed to remain in the aircraft and be away from the Fuelling zone when Fuelling is in progress.

- Engines should not be operated when Fuelling is in progress.

Training of Fuelling Operations Personnel:

- It is essential that all personnel connected with the Fuelling operations are adequately trained by their employers and are supplied with appropriate instructions and guidance on safe operating procedures.

All such personnel shall be fully conversant with the operation of firefighting equipment

Tristar Fuel farm and hydrant Operations.

UCAA shall provide an oversight on Tristar fuel farm and hydrant operations. A detailed report based on UCAA/EIA/AM/APP1/02/F23 shall be obtained from Tristar Energy Ltd every month. This shall be done to ensure that Joint Inspection Group (JIG) Standards and Aviation Fuel Quality Requirements for Joint Operations (AFQRJOS) are followed.

Inspection of Refuelling at EIA

The UCAA Aviation Fuel unit shall witness at least 1 aircraft refuelling from beginning to end to check for compliance with the standards and procedures every 15 days. The checklist used will be UCAA/EIA/AM/APP1/02/F24.

Defueling at EIA.

General defueling:

- Defueling shall take place into a mobile fueller or static tank.

- Defueling directly into a hydrant system shall not be permitted.

- Defueling at EIA shall be done in accordance with the individual fuel company’s defueling procedure and based on the reason for defueling.

# Procedure for Airside Accident/Incident Management:

Purpose: to track and take corrective action for a safe airside

Scope: throughout 24hrs operations

Activity

- The operator/driver/party involved immediately reports any incident/accident to Apron Management Office on Tel. +256 759 609 484, +256 312353261 SOO/AS/SC on Tel. +256 312353201, VHF 121.9 and POO/AS on Tel. +256 3123532293. In case of accident involving fire, fuel, and casualties, also contact ARFFS Tel. +256 414 320879; +256 312353322/3328.

- Provide transport for the Emergency Medical Responders.

- Take photographic and other forms of evidence before removal of the vehicle/equipment involved in accident/incident to ensure safety in the movement area.

- Fill and submit a report of the accident/incident to Apron Management not more than 4 hours from the time of occurrence. This is done by the Equipment/Vehicle operator/airside staff at the time of incident/accident occurrence.

- OO/AS/SL goes to the scene for fact finding and originates a final report (not more than 8 hours from time of occurrence) to be submitted to the SOO/AS/SC, who will verify the report and submit to POO/AS and MO, copying Manager SMS, GM-EIA, and DAAS, within the 12 hours of occurrence.

- OO/AS sends a summary of facts of the occurrence and submits to SMS office by email for further management.

    - Procedure for investigation and Follow-up on Airside Incidents and Accidents:

Purpose: ensure that all airside occurrences are investigated, and recommendations reviewed and approved for implemented

Scope: from start of the investigation to evaluation of implemented recommendation.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| SOO/AS/SC | 2.2.10.1 | Conduct preliminary investigation of all airsides major occurrences, through interviews, review of incident reports, scene analysis, CCTV review, documentation and process review, data and root cause analysis. Form 11: UCAA/EIA/AM/APP1/02/F11. |
|  | 2.2.10.2 | Forwards a report to POO for review |
| POO | 2.2.10.3 | Reviews the report and forwards to MO |
| MO | 2.2.10.4 | Verifies the report and shares with GM-EIA and SMS office |
| MSMS/SAG | 2.2.10.5 | Review the report and if appropriate recommends implementation of the recommendations. If not conducts separate investigation or recommends GM to appoint a final investigation team |
| COMMITTEE | 2.2.10.6 | Conducts investigations and forwards a report to GM copying DAAS, MSMS and MO |
| GM | 2.2.10.7 | Sanction implementation of the recommendation |
| MO | 2.2.10.8 | Instructs SOO/AS/SCAD to follow up Implementation of all recommendations and files evidence on implementation |
| SAG | 2.2.10.9 | Reviews the effectiveness of implemented recommendations. If not forwards to SRB |
| SRB | 2.2.10.10 | Recommends and implements more safety actions to reduce the risk |

Generated records:

Investigation report. Reviewed report, authorization to implement the recommendation

Measurement indicator:

- Evaluation reports

- Implementation reports

# Procedure for Conducting Monthly Aircraft turn around Audit Surveys:

| Action by | Step | Activity |
| --- | --- | --- |
| Pre-flight |  |  |
| SOO/AS/SCAD | 1 | Ensures: - Pre-flight briefing held. - Ramp crew ready prior to aircraft Arrival. - Arrival FOD walk complete. - TPBB in home position. - GSE staged behind equipment clearance/storage lines. - Marshaller in position for aircraft Arrival with suitable bats/wands. - Stop line for aircraft type identified. |
| Arrival |  |  |
| SOO/AS/SCAD | 2 | Checks for: - Conformity of the marshaller’s signals to ICAO standards. - Staff waited until aircraft was safe to approach (engine shut down, anti-collision beacons switched off, and thumbs up). - Time the chocks were inserted. - Cargo doors/passenger doors, sills/locks are checked prior to GSE approaching aircraft. - The aircraft doors are opened. - Arrival walk around the aircraft is performed by the engineer. - The time the hi-loader docks on the fuselage and the approach procedures. - The time the conveyer truck is connected to the door. - The time in-flight trucks dock on the fuselage and check the procedures to on-dock and off-dock. - Fuel dispenser/bowser positioned properly in accordance with standard operating procedures for re-fueling processes; Fueling clearance zone requirement followed. - Wing tip / engine markers were they properly placed? |
| Airside Personnel |  |  |
| SOO/AS/SCAD | 3 | Ensures: - PPE worn. - Safety culture being practiced. - Personnel walking where not authorized (e.g., Under fuselage, stepping over towbar, or between carts). - Personnel use correct manual handling techniques. |
|  | 4 | Check for Servicing: - Waste servicing - The time of engagement and the approaching procedures followed. - Emptying procedures followed. - Water servicing vehicle operating. - Water servicing procedures followed. - Portable water units kept away from lavatory units or other sources of contamination. Tug operation: - Apron speed limits followed. - Correct approach directions used for offload. - Tug serviceability status. - Stand by ARRFS truck: - ARRFS truck positioned in home position. - Personnel should be on board the truck all the time while at the apron. |
| SOO/AS/SCAD | 5 | TPBB: - Pre-use check completed. - TPBB positioning procedures followed. - Inspection of the door and surrounding area completed before opening. - Passenger door opened and secured properly. Passenger Steps: - Pre-use check completed. - Mobile passenger steps positioning procedures followed. - Inspection of the door and surrounding area completed before opening. - Passenger door opened and secured properly. |
| Departure |  |  |
| SOO/AS/SCAD | 6 | Ensures: - Ramp free of any items that could interfere with the safe operation of GSE or cause FOD. - Pre-departure walk around the aircraft performed. - Correct size/type of tractor and towbar used for pushback. - Ground power removal procedures followed. - Pre-pushback procedures followed. TPBB: - Receive confirmation from crew prior to closing door. - Passenger door closed and secured properly. - Off-dock retraction procedures followed. Passenger Steps: - Receive confirmation from crew prior to closing door - Passenger door closed and secured properly. - Mobile passenger steps retraction procedures followed. Departure: - Agent remains at a safe position on the aerobridge until the ACFT clears the gate. - Correct marshalling signals / communication used during dispatch. - Wing walking procedure followed. - Pushback procedure followed. - Steering by-pass pin and towbar disconnect procedure followed. - Final dispatch / release procedures followed. |
| SOO/AS/SCAD | 8 | Fills the Survey Form; Form 14: UCAA/EIA/AM/APP1/02/F14 |

Generated Records:

- Monthly equipment peak hour audit report

Measurement (Performance) Indicator:

- Number of equipment peak hour safety audits made.

- Number of compliments and complaints

- Rate of occurrence of incidents/accidents on airside.

# Procedure For monitoring and Enforcement of Airside Safety Requirement

Purpose: to ensure adherence to all airside requirements

Scope: always when at airside.

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| AOO/AS | 1 | a) makes continuous checks on staff adherence to airside safety requirements. In case of any violations. - Penalizable cases- Issue a violation form/penalty and hands them over to OO/AS - Non-penalizable cases- - Withdraws ADP and/or ASP - Makes offender fill airside safety violation form - Hands over offender’s ASP/ADP and filled airside safety violations form to OO/AS/SL. - OO/AS reviews the case and forwards it SOO/AS/SCAD |
| SOO/AS/SCAD | 3 | Gathers the facts of the violation and takes a course of action Punches the ADP and/or ASP each time there is a violation (once for the first violation, twice for the second violation and third for any other consequent violations) Forwards ADP and/or ASP with three punches to MO for further action |
| MO | 4 | Receives the ADP with three punches and suspends it. Forwards the ASP with three punches for further management |
| ASM | 5 | Cancels ASP authorizing offender access to airside and advises the offenders employer to redeploy the culprit to Landside. |

Generated Records:

- Occurrence logbook record

- Statements from offender

- Airside violations forms

- Penalty Violation Receipts and forms

Measurement Indicators:

        - Reduced number of incidents

Part 3: Airside Vehicle Control

  - Responsibilities of Vehicle/Equipment Operations at Airside:

## Employers Vehicle/Equipment Responsibilities

Each employer must ensure that:

- Their employees have qualifications to operate vehicles and equipment.

- Vehicle/equipment should have company colour and logo

- Employees have acceptable colour perception, reasonable eye-sight range, good hearing ability and sound mental faculties.

- Every vehicle operating at airside should have individual Airside Vehicle Permit (AVP). This AVP must be clearly always displayed on the vehicle that vehicle is operating airside for easy identification.

## Apron Management Vehicle/Equipment Responsibilities

- EIA has established requirements for vehicles/equipment inspection and certification to ensure that all vehicles operating at the airside are in good sound condition to enhance safety.

- That all such vehicles as above are inspected by CAA Head Mechanical and SOO/AS/SC airport operations before initial issue of the permit. Thereafter periodical checks must be done by the Equipment Serviceability Status Team (ESSAT) to ensure that they are safe and fit for the intended use and that their condition is such that the vehicle /equipment will not endanger the operator, other vehicle/Equipment, Pedestrians, Aircraft or Property

## Vehicle/Equipment Operator

- Before operating a Motor vehicle on the airside of the airport, the vehicle operator must become familiar with the regulations and procedures in this manual and obtain authorization from manager operations.

- The vehicle operator must check to see if the vehicle is operating satisfactorily and has the required safety equipment (Fire Extinguishers, Markings, or Indicators). All operators shall notify their immediate supervisor of any equipment malfunction.

- If you encounter any obstruction or potentially hazardous condition on the aircraft Movement surface, report its nature to the DATMO/SOO/AS so that corrective action may be taken.

- All personnel with EIA restricted area access permits and shall wear them on the outer clothing, ensuring that they are always visible when in the restricted area

- A person who is not in possession of a valid access permit shall not enter or remain in any area of the airport that is designated by sign or barrier as a restricted area unless identified and authorized to do so by MO.

- Persons not displaying the access permits shall be considered unauthorized and should be reported immediately to the DO/AVSEC. All designated gates are kept closed or locked to prevent unauthorized personnel or vehicles access to the airside.

- Headlights must be turned on whenever a vehicle is operating in the manoeuvring area and at night.

- Every operator of a vehicle involved in an accident in the airside area shall report the accident forthwith to their supervisor, the Safety Officer/SOO/AS/SC.

- Smoking is not permitted on the apron areas or other prohibited areas. This prohibition applies to persons both inside and outside vehicles and equipment.

- No person shall park a vehicle in any area of the airport designated by sign as loading area

- Every operator of a vehicle on the airside area, other than an emergency vehicle with the warning devices operating, shall yield right–of–way to the above-mentioned vehicles.

- Any vehicle towing baggage carts shall do so not exceeding three carts.

- No person shall:

    - Throw, deposit or knowingly leave on a road, apron, or manoeuvring area at the airport any glass, nails, trash, scraps or metal, chemical substance or other material that may damage any aircraft, vehicle or injure persons.

    - Throw, deposit or knowingly leave any form of trash or garbage at airport except in containers provided for that purpose.

    - Vehicles operating on the airport manoeuvring area shall have a functioning two-way radio operated by a competent person or be escorted by a vehicle so equipped and manned. Each operator shall ensure that the two-way radio communication is working before the vehicle enters the airport manoeuvring area.

    - Vehicles can seriously interfere with electronic equipment. No vehicle should proceed closer than 150m (501ft) from an ILS transmitter building except with the permission of control tower as far as possible vehicle operators shall use service/perimeter roads to reach their destination.

    - Operators and vehicles will remain clear of the scene of accident unless authorized by MO.

    - Operators and vehicles will remain clear of aircraft carrying distinguished visitors unless authorized by MO.

    - No vehicle shall drive over any red carpet laid for VVIP ceremonial Function.

## Airside Pavement Markings

General

- Both vehicles and aircraft Movement on the ground are guided by the pavement markings, lights, and signs, on the airside, which are different from those used on roads and highways.

- The following are markings, lights, and signs used in Entebbe international airport and each vehicle or equipment operator on the airside is required to know.

Aircraft Movement Guidelines

A single yellow line extending from the runway centre line along a taxiway to, and in Most cases, the apron. The nose wheel of the aircraft is centred on this line to ensure that the main wheels (undercarriage) are on the pavement and that the wings will not contact known obstructions (building), flood lights etc. On the aprons. Taxiway edges and aircraft stands are also marked yellow.

Hold-Lines:

Two solid and two broken lines across the width of a taxiway with the broken lines closest to the runway or another active taxiway. Vehicles and aircraft must stop behind the solid lines and not proceed until they are permitted to do so by the air traffic controller.

Runway Heading (Designator) Markings:

- Each end of the runway is numbered in tens of the degrees corresponding to the direction of the runway in relation to the magnetic compass.

- The compass of the aircraft will read 1701 when approaching the beginning of the runway marked 17; and 3501 when approaching the other beginning of the runway marked 35.

- Similarly, the compass of an aircraft will read 1201 when the aircraft is approaching the beginning of the runway marked 12 and it will read 3001 if the aircraft is approaching the other end, from the opposite direction, marked 30.

- Note: the numbers are painted white and readable from the runway approach.

- Runway Centre Line:

The centre of the runway is marked with broken white line runway edges are marked with solid white lines.

- Threshold Markings:

The beginning of the usable part of runway for aircraft landing is marked with a series of solid white lines parallel to the length of the runway. The lines in groups and numbers of groups vary according to the width of the runway.

- Displaced Threshold Markings:

If for any reason the threshold is be displaced from the end of the runway, white lines are painted close together to form arrows, pointing to a bar across the runway, to indicate the beginning of the usable part of the runway for aircraft.

- Helicopter Area:

The paved surface designated for arrival/departure of the helicopter is a large white “h” within a white Square or Circle

## Aerodrome Lighting System

There are different types of lights used at EIA, which include the following:

Aerodrome Beacon:

The aerodrome beacon is a large rotating white/green light Mounted on top of control tower. It is provided for visual identification of the airport by the aircraft, but it is also a good Reference Point for Vehicles on The Airfield.

Edge Lights for Aircraft Surface Movement:

- Blue lights are used along the edge of the aprons and taxiways.

- White lights are used along the edges of the runways.

- Two sided lights, half red and half green, are used at the end of runways and the green half pointing towards the approach to the runway. These lights are called threshold lights.

Pavement Lights

- Every vehicle operator must know the meaning of these lights to avoid entering areas where they are not permitted to be as a guide to the vehicle Movement when within the manoeuvring areas (runways and taxiways) of the airport.

- Note: At times runway lights are used to control vehicles and staff working close to or in the runway. These lights are controlled by control tower. When flashed on and off they give warning to vehicles and pedestrians to clear the runway. When they are steadily kept on, give warning for all to clear the runway.

## Pavement Signs

Manoeuvring Area Signs:

- Signs used on the manoeuvring area (runways and taxiways) are designated and intended for use and guidance of aircraft. They are also of value to vehicle operators to identify areas they should not enter or as guides to vehicle operator while in the manoeuvring area.

- These signs are normally Mounted on the left, right or both sides of the runway or taxiway according to the requirements and are located 15-20m (50ft to 65ft) from the edge of the manoeuvring surface.

- Mandatory Instruction Signs with white letters/numbers on a red background includes:

    - “Hold” Sign: which may be used on conjunction with “hold” lines (pavement markings) on a taxiway

    - Runway Designator Signs: when used alone (not in conjunction with “hold” signs). When these are red, they carry a combined message to “hold clear” and that the runway heading is as indicated on the signs.

Directional Sign:

Normally has an arrow indicating the direction of travel to exits, aprons, terminal buildings, or other facilities.

Information Signs:

Provide information of interest primarily to aircraft but which may also be useful to vehicle operators as a reference point. It has black letters/numbers on a yellow background.

Designator Signs:

Identifies the names of runways by numbers and taxiways by letters. Taxiways are referred to by using phonetic alphabet so that taxiway “A” is spoken as “taxiway Alpha” taxiway “B” is “taxiway Bravo”, etc. And that a vehicle may not enter a taxiway without prior approval of control tower.

  - Radio Procedures at the Airside:

General R/T Instructions:

- ATM is responsible for the control of all vehicles Movements in the manoeuvring area. To maintain such control, vehicles operating on the manoeuvring area shall be fitted with two-way R/T on the appropriate channel or must be escorted with R/T equipped vehicle.

- Correct R/T operating techniques (in this manual) must be observed by all the users. It is important that all vehicles in the manoeuvring area maintain a continuous listening watch.

- If the radio equipment in the vehicle fails, the vehicle operator must vacate the runway/taxiway immediately and inform control tower by the Most expeditious means - through another vehicle or by the nearest telephone line.

- English language is to be used in all radiotelephony communication.

- The drivers shall use local time in all their communication; and when using UTC, they shall suffix it with (z) Zulu. The day begins at 0101 hours and ends at 2401 hours.

Some Tips Before you go into the Maneuvering Area:

- Know the layout of the manoeuvring area where you are to operate.

- Follow the procedures of operating in the manoeuvring area.

- Be precise in your communication and patient in listening.

- Comply with instructions.

- Be always observant; in other words, keep your eyes open and stay alert.

- Never go beyond hearing range of your radio. Avoid blind spots as much as possible

- Never leave any object (equipment or tools) on the Movement Area.

Before Transmitting a Message:

- Listen out on the frequency to be used to ensure that you will not interfere with any transmission from another station.

- Do not interrupt if you hear another station making transmission that obviously requires a reply from the addressee.

- Only a station having a distress or urgency message to transmit is entitled to interrupt any transmission of lower priority.

- Determine what you wish to say before you transmit it and avoid the inadvertent use of hesitation sounds like “er.” “Eh.” Or “um”. Etc.

Microphone Procedures/Techniques:

- Do not turn your head away from the microphone while talking or vary the distance between your Mouth and the microphone.

- Hold the microphone as close to the lips as possible without touching the lips, (not More than three inches or 7.5 cm away) in front of the Mouth.

- Press the “press-to-talk” (PTT) switch before you begin to speak and keep it depressed for the entire transmission. Avoid clicking on and off.

- Speak plainly and distinctly to prevent running consecutive words together.

- Do not whisper. Do not shout or speak rapidly. Talk slower if any detail or the message must be written down by the recipient.

- Use standard procedure words; phraseology and terminology listed in this manual.

- After transmission, release the PPT immediately to be able to receive feedback/response.

Permission to Enter Maneuvering Area:

- Under no circumstances shall a vehicle operator enter manoeuvring area without the approval of ATM.

- Additional permission will be required to enter runways.

- Under no circumstances shall a vehicle operator enter/cross a runway without the approval of ATM.

- Obtain permission before entering within 75m from the centre line (the line of holding positions) of a runway.

- Always monitor the radio when in the manoeuvring area.

- No vehicle radio operator in the manoeuvring area shall leave the radio unattended.

- Inform ATM when your vehicle has vacated the manoeuvring area and not while you are in the process of leaving.

- Use the correct call sign for the vehicle you are operating in every radio transmission.

- When crossing the runway with other vehicle in tow, permission may be sought once. Station must specify the number of those in tow and shall be responsible for their movement.

ICAO Phonetic Alphabet and The pronunciation of Numbers:

Always use the ICAO phonetic alphabet when phonetics is required for clarity in radiotelephony communications. The following words are used when transmitting individual letters e.g., Call signs.

Transmission of Letters

The words in the table below shall be used when individual letters are required to be transmitted. The syllables to be emphasised are in bold.  letters and pronunciation

Table 1: ICAO Letters& Pronunciation

| Letter | Word | Appropriate pronunciation |
| --- | --- | --- |
| A | Alpha | Al fah |
| B | Bravo | Brah voh |
| C | Charlie | Char lee or shar lee |
| D | Delta | Dell tah |
| E | Echo | Eck oh |
| F | Foxtrot | Foks trot |
| G | Golf | Golf |
| H | Hotel | Ho tell |
| I | India | In dee ah |
| J | Juliett | Jew lee ett |
| K | Kilo | Key loh |
| L | Lima | Lee mah |
| M | Mike | Mike |
| N | November | No vem ber |
| O | Oscar | Oss cah |
| P | Papa | Pah Pah |
| Q | Quebec | Keh beck |
| R | Romeo | Row me oh |
| S | Sierra | See air rah |
| T | Tango | Tang go |
| U | Uniform | You nee form or oo nee form |
| V | Victor | Vik tah |
| W | Whiskey | Wiss key |
| X | X-ray | Ecks ray |
| Y | Yankee | Yang kee |
| Z | Zulu | Zoo loo |

Transmission of Numbers

The syllables to be emphasised are in bold.

Table 2: Transmission of Numbers

| Numeral Or Numeral Element | Latin Alphabet Representation |
| --- | --- |
| 0 | Zero |
| 1 | Wun |
| 2 | Too |
| 3 | Tree |
| 4 | Fower |
| 5 | Fife |
| 6 | Six |
| 7 | Seven |
| 8 | Ait |
| 9 | Niner |
| Decimal | Dayseemal |
| Hundred | Hun dred |
| Thousand | Tousand |

Examples of Radio Transmission Of Numbers:

Numbers 		Spoken As

10	:	Wun Zero

75	:	Seven Fife

101	:	Wun Hun dred

583	:	Fife Ait Tree

12010	:	Wun Too Tousand

38143	:	Tree Ait Wun Fower Tree

118.1	:	Wun Wun Ait Day- See-Mal Wun

121.9	:	Wun Too Wun Day-See-Mal Niner

Standard Words And Phrases:

The following words and phrases shall be used in radiotelephony communications as appropriate and shall have the meaning given below:

Table: Showing Phrases and Meaning

| Word/Phrase | Meaning |
| --- | --- |
| Acknowledge | Let me know that you have received and understood this message. |
| Affirm | Yes. |
| Approved** | Permission for proposed action granted. |
| Break | Indicates the separation between messages. |
| Break Break | Indicates the separation between messages transmitted to different aircraft in a busy environment. |
| Note: the phraseology “break break” may be confused with an instruction to an aircraft formation and should be used with caution. |  |
| Cancel | Annul the previously transmitted clearance. |
| Check | Examine a system or procedure. (Not to be used in any other context. No answer is normally expected.) |
| Confirm | I request verification of: (instruction, action, information). |
| Contact | Establish communications with... (your details have been passed). |
| Correct | True or accurate. |
| Correction | An error has been made in this transmission (or message indicated). The correct version is... |
| Disregard | Ignore. |
| Hold short** | Stop before reaching the specified location. Note: only used in limited circumstances where no defined point exists (e.g., Where there is not suitably located holding point), or to reinforce an approval limit. |
| How do you read | What is the readability of my transmission? |
| I say again | I repeat for clarity or emphasis. |
| Maintain ‡ | Continue in accordance with the condition(s) specified or in its literal sense, e.g., “maintain a listening watch”. |
| Monitor | Listen out on (frequency). |
| Negative | No; or permission not granted; or that is not correct; or not capable. |
| Negative I say again | May be used if repeated incorrect read backs are given by the pilot and additional emphasis is required. |
| Read back | Repeat all, or the specified part, of this message back to me exactly as received. |
| Report** | Pass requested information. |
| Request | I should like to know... Or I wish to obtain... |
| Roger | I have received all your last transmission. Note: under no circumstances to be used in reply to a question requiring a direct answer in the affirmative (affirm) or negative (negative). |
| Say again | Repeat all, or the following part of your last transmission. |
| Speak slower | Reduce your rate of speech. |
| Standby | Wait and I will call you. Note: no onward approval to be assumed. The caller would normally re-establish contact if the delay is lengthy. Standby is not an approval or denial. |
| Unable | I cannot comply with your request, instruction, or approval. Unable is normally followed by a reason. |
| Vacate | Leave the current location |
| Wilco | I understand your message and will comply with it (Abbreviation for will comply). |

Note: the phrase go ahead has been deleted, in its place the use of the calling call sign followed by the answering call sign shall be considered the invitation to continue with transmission by the caller.

Radio Call Up Procedures:

A call up procedure is a set of method used to establish a two-way communication between an airport vehicle (station) and ATM... Before making a “call up”, make sure no one else is communicating. A call up consists of two elements:

- Call sign of the station called, and

- Identification of the station from which the call is made.

Examples: 	“tower” 	station called.

“Follow-me” 	station calling.

Or

“Fire two” station called

“Marine south” station calling (boat launching station).

Authorisation Request and Response: Standard Phraseology:

Examples:

- Vehicle operator: “tower, follow-me on apron, request permission along TWY a to Kigungu tunnel.

- Control tower response: “follow-me, proceed.

- NB: if the request for permission to proceed is denied, the response from tower will be: “negative, follow-me, hold position, oncoming traffic TWY a.

Authorisation Request When Accompanying A Non-Radio Equipped Vehicle:

- Use the term “plus one, two or three…” to indicate to the controller the number of vehicles in the group, being closely escorted or led.

- Example: (Vehicle Operator): tower, follow-me plus one, request to cross runway 12/30 to satellite Fire Station”.

Control Instructions:

- Example: (Control Tower): “wildlife…, tower, vacate runway 35 via access road and report when off the runway.”

Radio Test Procedures:

- On-the-air radio test, when necessary, should be short, to avoid interfering with normal communication.

- The readability of the signal may be reported in plain language, but most often it is reported according to the following scale:

- Unreadable.

- Readable now and then.

- Readable but with difficulty.

- Readable.

- Perfectly readable.

Example:

| (ground): (Marine north (station): | “Marine, tower. Commence test count “Test count: one, two, three, four five, over” |
| --- | --- |
| (ground): | “Read you five.” |

Vehicle Identification/Radio Frequencies at EIA:

For radio telephony communication, airport vehicles are given the following identifications:

Table 3: Vehicle Identification/ RT Frequency at EIA

| Call Sign | Service Centre | Radio Frequency |
| --- | --- | --- |
| Ground | Ground Frequency | 121.9 MHz |
| Tower | ATM Tower | 118.1 MHz |
| Fire | Fire Officer to Pilot | 121.6 MHz |
| Fire 1 | Fire Rescue (Commander) | 118.1/121.9 MHz |
| Fire 2 | Fire Rescue Scania 3010l | 121.9 MHz |
| Fire 3 | Fire Rescue Mercedes 4010l | 121.9 MHz |
| Fire 4 | Fire Rescue Scania 12010l 6seats | 121.9 MHz |
| Fire 5 | Fire Rescue Scania 12010l 4seats | 121.9 MHz |
| Fire 6 | Emergency One 12010l | 121.9 MHz |
| Fire 7 | Training 12010l | 121.9 MHz |
| Fire 8 | Fire Rescue Mercedes 5010l | 121.9 MHz |
| Fire 9 | Fire Rescue Mercedes 5010l | 121.9 MHz |
| Fire 10 | Water Tender Mercedes 10101l | 121.9 MHz |
| Airport Rescue | Marine Officer to Pilot | 121.6/121.9 MHz |
| Rescue 1 | Water Rescue RIB South | 121.9 MHz |
| Rescue 2 | Water Rescue RIB North | 121.9 MHz |
| Rescue 3 | Water Rescue Catamaran | 121.9 MHz |
| Rescue 4 | Water Rescue | 121.9 MHz |
| Rescue 5 | Pickup South | 121.9 MHz |
| Rescue 6 | Pickup North | 121.9 MHz |
| Rescue 7 | Minibus | 121.9 MHz |
| EOC | Emergency Operations Centre | 121.9 MHz |
| Ambulance 1 | Fire Rescue | 121.9 MHz |
| Ambulance 2 | Airside Operations | 121.9 MHz |
| Watch Room | Fire Control | 121.9 MHz |
| Satellite | Satellite Station | 121.9 MHz |
| Marine North | Rescue North Control | 121.9 MHz |
| Marine South | Rescue South Control | 121.9 MHz |
| Rendezvous | Rendezvous Point (Landside) | 121.9 MHz |
| Rendezvous 2 | Rendezvous Point (Airside) | 121.9 MHz |
| Command Post | Mobile Command Post | 121.9 MHz |
| Follow me 1 | Follow Me Service | 121.9 MHz |
| Follow me 2 | Follow Me Service | 121.9 MHz |
| Follow me 3 | Airside Pick up | 121.9 MHz |
| Operations | Airside Operations Office | 121.9 MHz |
| Apron | Apron Control Tower | 121.9 MHz |
| Wildlife | Wildlife Operations Office | 121.9 MHz |
| Wildlife 1 | Wildlife Operations | 121.9 MHz |
| Wildlife 2 | Wildlife Operations | 121.9 MHz |
| Wildlife 3 | Wildlife Operations | 121.9 MHz |
| Electrik | Electrical Maintenance office | 121.9 MHz |
| Electrik 1 | Electrical Maintenance mobile vehicle | 121.9 MHz |
| Electrik 2 | Electrical Maintenance van | 121.9 MHz |
| Friction | Any Friction Testing Vehicle | 121.9 MHz |
| Grass 1 | Grass Cut Runway 12/30 | 121.9 MHz |
| Grass 2 | Grass Cutting Runway 17/35 | 121.9 MHz |
| Sweeper 1 | Pavement Sweeper | 121.9 MHz |
| Pavements 1 | Pavement Cutter | 121.9 MHz |
| Painting 1 | Pavement Painting | 121.9 MHz |
| Maintenance | Maintenance Engineering Pickups | 121.9 MHz |
| See-Ness 1 | CNS Maintenance | 121.9 MHz |
| Aytem 1 | ATM Inspection | 121.9 MHz |
| Dysser 1 | Aerodrome Regulator | 121.9 MHz |
| Avsek 1 | AVSEC Service Deployment& Inspection | 121.9 MHz |
| AVPOL 1 | Security Patrol | 121.9 MHz |
| Air Force 1 | Air Force Operations | 121.9 MHz |
| Air cargo | Uganda Air Cargo Operations |  |
| UN 1 | UN Operations | 121.9 MHz |
| UN 02 | United Nations Operations | 131.450 VHF 7576 HF |
| Das | Das Handling Limited office | 123.4 VHF |
| NAS Radio | NAS office | 131.7 VHF |
| Das Handling 1 | Das Operations | 121.9 MHz |
| Menzies Handling 1 | NAS Operations | 121.9 MHz |
| Jet 1 | EJAF | 121.9 MHz |
| Total 1 | Total Energy | 121.9 MHz |
| Vivo 1 | Vivo Energy | 121.9 MHz |
| Avgas 1 | Fueling Operations | 121.9 MHz |
| Eagle 1 | Eagle Air Operations | 121.9 MHz |
| Air Serv 1 | Air Serv Operations | 121.9 MHz |
| KEA 1 | Kampala Executive Aviation | 121.9 MHz |
| GA 1 | General Aviation Operations | 121.9 MHz |
| Newrest 1 | Inflight Services | 121.9 MHz |
| Contractors 1 | Project Management, Implementation& Supervision | MHz |

  - Procedure for ADP and AVOP:

Procedure For   ADP

Purpose: to control unauthorized vehicle/equipment access and operation at the airside.

Scope: from the time of submission to issuance of new ADPs.

Responsibility: MO

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| SOO/AS | 1 | - Carries out annual ADP training. - Receives special ADP requests from stakeholders. - Upon receiving approved ad hoc ADP training from MO, SOO/AS/SC draws out a training program. - Candidates including those taught by GHAs are examined, the required pass mark is 70% - Compile list of successful candidates for issuances of Permits. - Issues the ADP to employer after verifying with the payment receipt from CAA revenue Office. - Validity: once issued, the ADP shall be valid for a period of 2 years. h) ADP Refresher trainings will be conducted once every after two years |

Procedure For AVOP

| Action by | Step | Activity |
| --- | --- | --- |
| MO | 2 | Receives request for vehicle intending to operate on the airside; |
|  | 3 | Send request to SOO/AS/SC for vehicle safety inspection; |
| SOO/AS/SC | 4 | Carries safety inspection to ensure compliance with airside safety requirements including appropriate beacon colour and lighting intensity and reverts to ASM. |
|  | 5 | Beacon colour and lighting intensity requirement. - Beacon colour for Vehicle/equipment associated with emergency or security shall be flashing blue and others shall be flashing yellow. - Low-intensity obstacle lights. Type D shall be displayed on follow-me vehicles with Peak intensity (cd) at given background luminance shall be (60-90 fpm) and at night below 50 cd/m2 its 40. - Low-intensity obstacle lights. Type C shall be displayed on other airside vehicles/equipment with Peak intensity (cd) at given background luminance shall be (60-90 fpm) and at night below 50 cd/m2 its 200. |

Generated Records:

- List of drivers to be issued ADP

- Application for ADP (Form 17)

- Payment vouchers for ADPs

Measurement Indicator:

- Number of ADP issued in each period.

  - Procedure for Enforcing Airside Vehicle/Equipement Operations:

Purpose: to implement safety requirements.

Scope: from the time of initial request to deploy the vehicle/equipment at the airside to their withdrawal.

Responsibility: POO-AS

Text

| Action by | Step | Activity |
| --- | --- | --- |
| Operator | 1 | Writes a formal request to introduce vehicle/equipment to airside to GM-EIA |
| GM | 2 | Approves and forwards the request to MO who also forwards to SOO/AS/SCAD to initiate inspections. |
| SOO/AS/SCAD Head Mech | 3 | - Makes initial checks on Motorised vehicles/equipment to be introduced at airside. - Ascertain their mechanical condition. - Ascertain the availability of safety requirements. - Writes inspection report and forwards for AVP processing. |
| SOO/AS/SC |  | - Make ad hoc inspection |
| SOO/AS/SC | 4 | Check motorised vehicles/equipment in airside for the following: - Valid AVP. - Fire extinguisher. - Company logo and the airside vehicle safety compliance sticker. - ASC acceptable airside colour. - Rotating beacon (amber/yellow) Mounted on strategic position of the vehicle depending on use. - ESSAT apron-worth sticker. - 2-way radio communication r/t 121.9 MHz - Parking in designated parking areas. - Unattended to Motorized vehicle/equipment with running engine. - Monitor serviceability of vehicle/equipment and ensure that the unserviceable/defective are withdrawn from the apron. - Conduct ad-hock inspections on airside vehicles/equipment - Writes brief notes in the occurrence logbook. - Informs POO/AS of the defect observed or detected of any airside vehicle/equipment |
| ESSAT | 5 | ESSAT Inspection Motorised vehicle/equipment to ascertain both their mechanical condition and availability of serviceable and valid airside vehicle safety requirement. |
| SOO/AS | 6 | Makes ESSAT inspection report for POO, copy MO and GM-EIA |

Generated Records:

- Incident/Accident Report.

- Inspection Reports

Measurement Indicators:

- Rate of incidents related to the presence of un-licensed vehicles/equipment on airside.

  - Roles and Responsibilities:

Airport operations:

- Inspect all runways, taxiways, aircraft stands and cargo areas 2 times daily, for the presence of FOD in line with established procedure for inspection of movement areas.

- Identify hazard locations and fills the hazard identification form and notify Manager Operations/SMS.

- Pick up FOD on all movement areas that is Runways, Taxiways and Aprons, daily.

- Receive and respond to FOD calls from Aerodrome Control Tower and aircraft operators.

- Inspect and as needed ensure that the Airside trash cans are emptied daily.

- Inspect aircraft stands before each flight.

- Carry out special inspections of runways, taxiways, and aprons.

  - at cessation of works on the movement areas,

  - when a runway not previously inspected is brought in use,

  - following an aircraft accident or incident,

  - following an abandoned take-off by turbine engine aircraft due to engine failure or by an aircraft due to tyre burst or bird strike,

  - following a heavy storm.

- Ensure apron sweeping, including the areas between Vehicle Service Roads and terminal building.

- Sweep Runways and Taxiways as and when needed.

- Ensure that the contractor cleans the apron daily.

- Ensure that the FOD bins deployed at airside are regularly emptied.

- Conducts FOD awareness to EIA airside organisations, contractors, and employees on EIA’s FOD policy.

- Organize and coordinates quarterly FOD walks and other activities in support of the FOD program.

Aerodrome Maintenance and Aerodrome Engineering, Planning and Development:

- Ensure work sites are clean at the end of each day.

- Ensure work site dumpster(s)/trash bins are covered at the end of each day.

- Ensure trash is removed from work sites when dumpster/trash bin is full and/or as needed.

Ground Handler Roles and Responsibilities:

- Collect and dump all FOD in their areas of operation daily.

- Inspect aircraft stands before and after each flight.

- Inspect tow carts to confirm that trash is being placed in the appropriate tow carts.

- Inspect aircraft stand areas and confirm a trash bin is at every aircraft stand.

- Inspect tow carts to ensure they are not leaking or have damaged wheels, have an operable safety chain and closable hook, and two lids.

- Notify Airport Operations and/or appropriate airport representative of any safety concern arising from FOD.

- Participate in quarterly FOD walks.

Develop and implement FOD prevention programs/plans that support this program.

Airside Organizations Roles and Responsibilities:

- Collect and dump all FOD in their areas of operation daily.

- Notify Airport Operations and/or appropriate airport representative of any safety concern arising from FOD.

- Participate in quarterly FOD walks.

- Develop, implement, and maintain FOD prevention programs/plans including FOD awareness training that support this program.

Air Traffic Control Roles and Responsibilities:

- Inspect all runways and taxiways for the presence of FOD in line with established procedure in the Manual of Air Navigation Services Operations Procedures Part I.

- Notify Airport Operations and/or appropriate airport representative of any safety concern arising from FOD.

Wildlife Management Roles and Responsibilities:

While carrying out bird scare runs on the runways and taxiways, pick up any FOD and notify Airport Operations and/or appropriate airport representative of the FOD.

The Manager SMS EIA or Designee shall:

- Use available information to aid in identifying specific areas of concern and to determine corrective action for program improvement.

- Designate selected areas to be spot-checked on a quarterly basis, such as turn-up areas, workspaces, hangar bays, and storage areas.

- Conduct pre-scheduled but unannounced audits of work areas to assess effectiveness of the FOD prevention and tool control programs.

- Ensure implementation of corrective actions for FOD prevention throughout the airport.

- Review the FOD Prevention Program annually to ensure that it is adequate and current.

  - Preventive Measures:

FOD Preventive and control measures are implemented, and reports generated will be used to review this program.

EIA will post FOD awareness signage at different location on the airside.

Airside organisations and Contractors will implement a FOD Prevention Program within their area of activity and responsibility. These FOD prevention Programs will be in line with this Program.

FOD Removal and maintenance:

FOD bins/containers will be provided in such a location that any debris or litter will be not transit past that point onto a parking area, taxiway, or runway. The containers will have a secure cover to prevent wind effects.

The philosophy at EIA is simple: If you see it, pick it up and place it in appropriate FOD container! FOD containers will be placed to attract attention and personnel participation in the program.

FOD Removal Equipment:

EIA maintains an apron sweeping equipment used to sweep the apron. Currently EIA maintains a FOD BOSS that is used to automatically pick up any debris on the surface it is driven over. This activity is done monthly and whenever necessary.

FOD Walks:

“FOD Walks” as groups or crews is one of the pro-active strategies put in place to implement this program. All airside organisations, contractors and employees are encouraged to participate in these activities quarterly.

FOD incident reporting and Investigations:

FOD reporting is not only encouraged but essential. If any aircraft, vehicle, or Person on foot finds a situation with FOD consequences, whether potential or existing, this must be reported to Air Traffic Control Tower on Radio, Manager Operations EIA, or his/her representative in a timely manner.

To measure program effectiveness, each FOD incident shall be reported to Air Traffic Control Tower on Radio, Manager Operations EIA, or his/her representative.

Each reported occurrence of FOD resulting in damage to aircraft, engines, tyres, or components will be investigated as soon as possible to determine the cause of damage, source of the FOD and ways to prevent recurrence.

Training of Personnel:

Training is the major component in any FOD prevention program. As a minimum, EIA will conduct FOD awareness and training at all induction courses and repeated at times of recurrent training.

All Personnel on the airside will be trained to recognise and be responsible for the elimination of conditions that could result in FOD. Personnel are encouraged to report to their manager/supervisor of any condition that may result in FOD.

Additionally, training will include a review of FOD incidents and likely incident scenarios. Management will ensure that all appropriate airside staff receive this training.

Procedure for FOD Management:

Purpose: to detect, reduce, and eliminate foreign object debris at airside.

out Scope: always when at airside.

Responsibility:  POO

Text:

| Actor | Step | Activity |
| --- | --- | --- |
| OO/AS, AOO/AS | 1 | - Carries out FOD awareness programs, campaigns, and publicity to remind stakeholder of the dangers of FOD at airside. - This is incorporated in airside safety awareness program and FOD walk |
| OO/AS, AOO/AS, ATM | 2 | - Carries on both scheduled, ad hoc, and special inspection on the Movement Area to detect FOD and other anomalies. - When FOD is detected, removes, or arranges for removal immediately - Informs ATM of FOD removed from maneuvering areas and may be taken to tower on ATM’s request. - Ensures that daily, scheduled, and special cleaning/sweeping of the aprons and maneuvering areas is done to eliminate FOD. |
| OO/AS | 3 | - Organizes/coordinates joint airport-stakeholders FOD walks to check for FOD and to identify the sources and encourages everyone to remove FOD on seeing. - Ensure that the FOD picked during FOD walk is sorted and disposed of or kept in case of aircrafts parts and other items of interest. |
| GHA, AOO/AS | 4 | Inspect aircraft stand before flight arrival and after departure to ensure all FOD is removed. |
| SOO/SCAD, OO/AOO/AS | 5 | - Ensures contractors involved in projects are aware of a need to keep all their materials on-site and not allow spillage to aircraft areas - Ensure FOD inspection is done before the area which was under construction is restored to normal use. |
| OO/AS, All Stakeholders Detecting & Removing FOD | 6 | Fill in the FOD form |
| OO/AS, AOO/AS | 7 | Ensure anyone found dumping or causing dumping of FOD at airside fills airside violations form. |
| SOO/SCAD | 8 | - Investigates FOD to identify likely contributor and writes a report to POO copying MO and MSMS - Presents analyzed FOD reports at airside safety groups meetings. |

Generated Records:

- FOD Forms

- FOD Reports

Measurement Indicators:

        - Reduction FOD and FOD incidents

# Procedure for Remote monitoring of Airside activity

Purpose: Surveillance of airside activity to ensure proactive identification, reporting and mitigation of the hazards.

Scope: From the time a shift reports on duty to the time of handover

Responsibility: POO/AS

Text:

| Action by | Step | Activity |
| --- | --- | --- |
| AOO/AS DOO DM | 1 | - Remotely monitor airside activity - Identify the hazards and other safety concerns report to SOO/SCAD and POO - Record observations/hazards in the airside remote monitoring logbook |
| SOO/SCAD | 2 | - Ensures that the hazards reported are mitigated proactively and reported to POO and SMS office. - Compile monthly reports to POO and MO |
| POO | 3 | Escalates unresolved issues to MO and DAAS management |

Generated Records:

- Airside remote monitoring reports

- Unresolved issues escalated to MO and DAAS management.

Performance Measurement

- Number of airside remote monitoring reports

- Number of hazards identified and resolved.

Attachments:

- Attachment 1: Nature of Airside Offence

- Using a vehicle without a valid airside vehicle permit in the Movement Area.

- Failing to comply with conditions of AVP.

- Driving a vehicle in the Movement Area without a valid   ADP.

- Exceeding speed limit of 5kph/25kph/40kph where applicable.

- Failing to give way to an aircraft in the Movement Area.

- Leaving broken down vehicle unattended in the Movement Area.

- Leaving a vehicle with running engine unattended in the movement area.

- Failing to obtain prior clearance from DTMO to proceed to any part of the Movement Area other than the apron.

- Failing to stop at a point of entry to the manoeuvring area/failing to ascertain that there is no aircraft Movement before proceeding into the manoeuvring area.

- Overtaking and passing other vehicles proceeding in the same direction in the Movement Area.

- Parking in or near the Movement/vehicle traffic route other than approved.

- Failing to slow down the vehicle when it is approaching an aircraft.

- Failing to give way to aircraft passenger.

- Failing to approach at an angle at the right side of an aircraft.

- Drawing up or stopping the vehicle under the wing, tail, or the fuselage of an aircraft.

- Starting of vehicle when it is within 15 metres of the re-fuelling aircraft.

- Backing vehicle to an aircraft without a marshaller.

- Approaching an aircraft which has its engines running.

- Failing to keep vehicle at least 5 metres from the wing tip of a stationary aircraft/failing to keep vehicle at least 45 metres from the edge of the taxiway when traveling parallel to taxiing aircraft.

- Falling to keep vehicle to the right side of the runway or taxiway.

- Driving the vehicle into the runway or taxiway when it is not equipped with a two-way radio communication.

- Crossing Movement Area in front of the path of taxiing aircraft or aircraft on tow.

- Passing aircraft within 8 metres from air intake or within 80 metres from the rear of a running jet engine, or less than 201m in front or 250m behind a taxing aircraft.

- Failing to obtain prior clearance from DATMO when proceeding onto the apron when your regular duties prohibit.

- Failing to remove servicing vehicle/equipment from aircraft parking area after aircraft has taxied away.

- Depositing of foreign matter or things on the surface of the Movement Area.

- Smoking whilst in any part of the Movement Area or hangar.

- Passengers failing to be seated in the passenger seat provided when traveling in a vehicle.

- Tow a vehicle without a suitable tow bar.

- Failing to maintain a vehicle in a good mechanical condition (owner).

- Failing to conform to traffic sign in the aerodrome.

- Parking vehicle in a manner causing obstruction, danger or undue inconvenience to other users in the aerodrome

- Parking vehicle in a manner causing obstruction to water hydrants and fuel emergency stops.

- Parking within vehicle corridor and unbroken white line/unbroken double white lines.

- Parking on the side of an unbroken yellow line of the Movement Area.

- Parking or stopping a vehicle on the cross walks of any boarding gate.

- Using a vehicle which is not licensed under road traffic rules.

- Failing to obey traffic direction given by authorized Officers engaged in regulation of traffic within the aerodrome.

- Failing to be in possession of airport pass whist in the prohibited or restricted area of an aerodrome.

- Failing to report an accident to the duty apron controller within 24 hours of the occurrence.

- Driving a vehicle without due care of reasonable consideration.

- Any violation of traffic rules and licensing regulations set forth herein.

- Operating un-identified vehicle/equipment in Movement Area of an airport.

- Operating an identified/unidentified vehicle/equipment without airside safety requirements

- Traversing the aircraft parking stands

- Operating a vehicle, you are not qualified to drive in Movement Area.

- Failing to wear protective clothing in Movement Area.

Penalties for Airside rules and penalties:

| No | Contravention | Penalty |
| --- | --- | --- |
| 01 | Over speeding shall attract penalty of; | 100,000/= |
| 02 | Spills per every square meter shall attract penalty of; | 500,000/= |
| 03 | Failure to put on reflective vest shall attract penalty of; | 100,000/= |
| 04 | Driving without valid national driving permit shall attract penalty of; | 100,000/= |
| 05 | Driving without valid ADP shall attract penalty of; | 100,000 |
| 06 | Driving equipment/vehicles without airside vehicle permit shall attract penalty of | 200,000/= |
| 07 | Driving equipment/vehicles without airside safety requirement shall attract penalty of | 100,000/= per requirement |
| 08 | Driving equipment/vehicles without ESSAT compliance sticker shall attract penalty of | 100,000/= |
| 09 | Driving equipment/vehicles in dangerous mechanical condition at airside shall attract penalty of | 300,000/= |
| 10 | Driving a grounded equipment without verification inspection by ESSAT shall attract penalty of | 300,000/= |
| 11 | Traversing aircraft stands shall attract penalty of | 300,000/= |
| 12 | Causing runway incursion shall attract penalty of | 500,000/= |
| 13 | Entering/crossing any a taxiway or taxing lane areas without ATC permission shall attract penalty of | 300,000/= |
| 14 | Parking outside designated parking areas shall attract penalty of | 100,000/= |
| 15 | Obstruction of fuel emergency stops and water hydrants shall attract penalty of | 100,000/= |
| 16 | Failure to give way to aircraft shall attract penalty of | 300,000/= |
| 17 | Leaving equipment and FOD bins on aircraft stand after servicing aircraft shall attract penalty of | 100,000/= per equipment |
| 18 | Use of dollies and baggage carts without breaks shall attract penalty of; | 300,000/= per equipment |
| 19 | Leaving unit load devises unsecured | 300,000/= per equipment |
| 20 | Smoking at airside | $1000 |
| 21 | Special Fire Cover to Aircraft on the Apron | $1000 |

- Attachment 2: Foreign Object Debris (FOD) Prevention Program for EIA

Purpose of FOD Prevention Program

EIA airside organizations, contractors and employees generate FOD on a daily basis during routine airport operations, maintenance, waste handling, disposal activities, and construction operations. For this reason, EIA has developed this Foreign Object Debris (FOD) Prevention Program to manage and minimize the impact of FOD on the Aircraft Operations.

The purpose of this Foreign Object Debris (FOD) Prevention Program is to establish Policy, Responsibilities and Measures to prevent damage to aircraft and provides for a uniform FOD prevention procedure.

FOD Policy

EIA Top Management is committed to implementing a comprehensive FOD Program which Identifies, corrects, and eliminates causal factors to achieve the ultimate goal of having the most sustainable FOD free airport. FOD prevention is everyone's responsibility and implemented in coordination with all stakeholders.

This is achieved through the following:

- By increasing awareness of the Operational staff on causes and effects of FOD

- Aerodrome Operating Staff participation

- Carrying out regular inspections

- Routine Cleaning

- Provision of Disposal facilities

- Carry out FOD trends Analysis

Prepared By:   _____________________ 			Date: _______________

Manager Operations

Reviewed By:   _____________________ 			Date: _______________

General Manager Entebbe International Airport

Airside Forms

### Form 1: Staff Proficiency and Performance Monitoring Form (Aircraft Marshalling)

| ENTEBBE INTERNATIONAL AIRPORT |  |  | FORM NO. |  |  |  |  |  | UCAA/EIA/AM/APP1/02/F01 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TITLE |  |  |  |  |  | STAFF PROFICIENCY/PERFORMANCE MONITORING |  |  |  |  |  |  |  |  |
|  |  |  | ISSUE DATE |  |  |  |  |  | 30 December, 2022 |  |  |  |  |  |  |  |  |
|  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | REVISION NO. |  |  |  |  |  | 001 |  |  |  |  |  |  |  |  |
| AIRCRAFT MARSHILLING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DATE: | TIME: |  |  |  |  |  | STAND NO. |  |  |  |  |  |  | FLIGHT NO. |  |  |  |
| Scale: |  | Poor |  | 1 | Average |  |  |  |  | 3 | Very Good |  |  |  | 5 |  |  |
|  |  | Fair |  | 2 | Good |  |  |  |  | 4 |  |  |  |  |  |  |  |
| PART ONE: AIRCRAFT ARRIVAL PREPARATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Prior To Arrival Of Aircraft |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCE SCALE |  |  |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 | Sub Score: |  |  |  |  |
| Timely Arrival on Stand |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stand Inspected Prior |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOD If Any Removed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Obstructions Removed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PPE Worn? Hi-Vis, Feet, Ears? |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stand Size Limitation Check |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Parking Of Adjacent Aircraft |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aerobridge Checks |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Parking Planning |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| On Time Notification of Stand To GHA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PART TWO: AIRCRAFT PARKING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Marshalling of Aircraft |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCE SCALE |  |  |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 | Sub Score: |  |  |  |  |
| Aircraft Recognition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Manoeuvre Planned |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Awareness of Blast/Downwash |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Adequate Wingtip Clearance |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Marshaller Visible to Pilot At All Times |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PART THREE: MARSHALLING REQUIREMENTS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rules and Regulations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCE SCALE |  |  |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 | Sub Score: |  |  |  |  |
| Local Instructions Understood |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Correct and Clean Equipment Used |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ICAO Signals Used - Not Too Stylised |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Correct Speed, Clarity of Intension |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PART FOUR: AEROBRIDGE OPERATION PROCEDURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Rules and Regulations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCE SCALE |  |  |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 | Sub Score: |  |  |  |  |
| Pre-operations Inspections |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pre-docking tests |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Identify Emergency stop errors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Resolving an alarm |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Proper alignment of the canopy to the aircraft |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Docking Time (2 minutes) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PART FIVE: FOLLOW ME PROCEDURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Follow Me Procedures for aircrafts/Vehicles |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCE SCALE |  |  |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 | Sub Score: |  |  |  |  |
| Correct Leader Vehicle Speed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Correct "Follow Me" Message |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Appropriate RTF Used |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Appropriate Driving Style |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PART SIX: OTHER TASKS ASSIGNED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PERFORMANCE SCALE |  |  |  | 1 |  | 2 |  | 3 |  | 4 |  | 5 | Sub Score: |  |  |  |  |
|  |  |  |  |  |  |  |  | TOTAL |  |  |  |  |  |  |  |  |  |
| General Comment by Shift Leader/SOO/AS/OPS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  |  |  |  | Sign: |  |  |  |  |  |  | Date: |  |
| Comment by POO/AS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  |  |  |  | Sign: |  |  |  |  |  |  | Date: |  |

### Form 2: Shift Handover/Takeover Form

| ENTEBBE INTERNAIONAL AIRPORT |  |  | FORM NO. |  |  | UCAA/EIA/AM/APP1/02/F02 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TITLE |  |  | SHIFT HANDOVER/TAKEOVER REPORT FORM |  |  |  |  |  |  |  |
|  |  |  | ISSUE DATE |  |  | 30 December, 2022 |  |  |  |  |  |  |  |
|  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |
|  |  |  | REVISION NO. |  |  | 02 |  |  |  |  |  |  |  |
| Personnel: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a) Out-Going Shift: |  | b) in-coming Shift: |  |  |  |  |  | c) Date: |  |  |  |  | d) Time in: |
| D/S…....... | N/S: ....... | D/S…....... |  |  | N/S…....... |  |  |  |  |  |  |  |  |
| Name: |  | Name: |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  | Name: |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  | Name: |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  | Name: |  |  |  |  |  |  |  |  |  |  |  |
| Handover Report: Apron Equipment/Vehicles |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Subject matter To Be Handed Over/Deployed |  |  |  |  |  |  | Prevailing Condition |  |  |  |  |  |  |
| 1. Follow Me 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i) R/T 121.9 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ii) Vehicle Installed Tetra |  |  |  |  |  |  |  |  |  |  |  |  |  |
| iii) Spare Wheel |  |  |  |  |  |  |  |  |  |  |  |  |  |
| iv) Wheel Spanner |  |  |  |  |  |  |  |  |  |  |  |  |  |
| v) Jerk |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vi) Beacon |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vii) First Aid Box |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Follow Me 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i) R/T 121.9 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ii) Vehicle Installed Tetra |  |  |  |  |  |  |  |  |  |  |  |  |  |
| iii) Spare Wheel |  |  |  |  |  |  |  |  |  |  |  |  |  |
| iv) Wheel Spanner |  |  |  |  |  |  |  |  |  |  |  |  |  |
| v) Jerk |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vi) Beacon |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vii) First Aid Box |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. Alpha Charlie Mobile |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i) R/T 121.9 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ii) Vehicle Installed Tetra |  |  |  |  |  |  |  |  |  |  |  |  |  |
| iii) Spare Wheel |  |  |  |  |  |  |  |  |  |  |  |  |  |
| iv) Wheel Spanner |  |  |  |  |  |  |  |  |  |  |  |  |  |
| v) Jerk |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vi) Base Radio |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. Passenger Boarding Bridge (PBB) 1 - IS/LS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i) PBB Cleanliness |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ii) Terminal Access Lockable Doors |  |  |  |  |  |  |  |  |  |  |  |  |  |
| iii) Serviceability Status |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. Passenger Boarding Bridge (PBB) 2 - IS/LS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i) PBB Cleanliness |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ii) Terminal Access Lockable Doors |  |  |  |  |  |  |  |  |  |  |  |  |  |
| iii) Serviceability Status |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Remarks: (IS): ____________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________Sign: ________________________________________ |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Handover report: Office Equipment/Tools |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Subject Matter to Be Handed Over/Deployed |  |  |  | Prevailing Condition |  |  |  |  |  |  |  | Number |  |
| 1. Marshalling Bat |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i) Clean/Neat |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ii) Worn Out |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. Marshalling Wands (Torches) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i) Bright |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ii) Dim |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. Tetra Phone |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number: ……………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number: ……………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number: ……………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number: ……………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number: ……………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Number: ……………………. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. Base Radio 121.9 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5. Office Telephone |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6. Desk Computer Accessories |  |  |  |  |  |  |  |  |  |  |  |  |  |
| i) Monitor |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ii) UPS |  |  |  |  |  |  |  |  |  |  |  |  |  |
| iii) Mouse |  |  |  |  |  |  |  |  |  |  |  |  |  |
| iv) Keyboard |  |  |  |  |  |  |  |  |  |  |  |  |  |
| v) Camera |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vi) Memory Cards |  |  |  |  |  |  |  |  |  |  |  |  |  |
| vii) AODB |  |  |  |  |  |  |  |  |  |  |  |  |  |
| viii) Airside Manual |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Major Events of the Day/Night: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| General Remarks by Out-Going Shift: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  |  |  |  | Sign: |  |  |  |  |
| General Remarks by In-Coming Shift: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  |  |  |  |  | Sign: |  |  |  |
| Shift Leader Out-Going |  |  |  |  |  |  |  |  |  | Shift Leader In-Coming |  |  |  |
| Remarks by SOO/AS: |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  |  |  |  |  |  | Sign: |  |  |

### Form 3: Apron Packing Reference Chart Form:

To be inserted

### Form 4: Aircraft Parking/Stand Inspection Form

Monday

| ENTEBBE INTERNATTINAL AIRPORT |  |  |  |  | FORM NO. |  |  |  |  |  | UCAA/EIA/AM/APP1/02/F04 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TITLE |  |  |  |  |  | PARKING/STAND INSPECTION FORM |  |  |  |  |  |  |
|  |  |  |  |  | ISSUE DATE |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | REVISION DATE |  |  |  |  |  | 30-Dec-22 |  |  |  |  |  |  |
|  |  |  |  |  | REVISION NO. |  |  |  |  |  | 3 |  |  |  |  |  |  |
| Pre-Arrival Stand Inspection |  |  |  |  |  |  |  |  | Post Departure stand inspection |  |  |  |  |  |  |  |  |
| Flight number | A/C Type | ATA | Stand | Inspection (10/15 min Before) |  |  |  | Reported by | Flight number | ATD |  | Stand | Inspection (10/15 min Before) |  |  |  | Reported by |
|  |  |  |  | FOD | FOD Bins Emptied | Spillage | Obstacles | Name |  |  |  |  | FOD | FOD Bins Emptied | Spillages | Obstacles |  |
| KQ 414 | B190 | 935 | A1S04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Remark by AOO/AS: …………………………………………………………………………………………………………………………………………………………………………………………………

…………………………………………………………………………………………………………………………………………………………………………………………………

AOO/AS: Name:    ____________________________	Sign:    _________________________ Date:    _______________________      __

Comments: …………………………………………………………………………………………………………………………………………………………………………

OO/AS: Name:    ____________________________	Sign:    _________________________ Date:    _________________________

### Form 5: TPBB Docking Record and inspection form

| ENTEBBE INTERNATIONAL AIRPORT |  |  |  |  | FORM NO. |  |  |  | UCAA/EIA/AM/APP1/02/F05 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TITLE |  |  |  | TPBB DOCKING RECORD AND INSPECTION FORM |  |  |  |  |  |  |  |
|  |  |  |  |  | ISSUE DATE |  |  |  | 30 August, 2022 |  |  |  |  |  |  |  |
|  |  |  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | REVISION NO. |  |  |  | 01 |  |  |  |  |  |  |  |
| DATE: |  |  |  | TIME: |  |  |  |  |  |  |  | BRIDGE NO.: |  |  |  |  |
| OPERATOR | FLIGHT NO. |  | A/C REG |  |  | A/C TYPE | ORIGIN |  |  | DEST | DOCK ON |  |  | DOCK OFF |  |  |
| INSPECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scale |  | Satisfactory |  | Not Satisfactory |  |  |  | Scale |  |  |  |  | Satisfactory |  | Not Satisfactory |  |
| Bridge Safety Zone Clearance |  |  |  |  |  |  |  | Up/Down Mov't |  |  |  |  |  |  |  |  |
| interior Cleanliness |  |  |  |  |  |  |  | Canopy Forward |  |  |  |  |  |  |  |  |
| Exterior Cleanliness |  |  |  |  |  |  |  | Forward Extension |  |  |  |  |  |  |  |  |
| Front AC |  |  |  |  |  |  |  | Retraction |  |  |  |  |  |  |  |  |
| Middle AC |  |  |  |  |  |  |  | Lockable Doors |  |  |  |  |  |  |  |  |
| Back AC |  |  |  |  |  |  |  | Technical Step |  |  |  |  |  |  |  |  |
| Interior lighting |  |  |  |  |  |  |  | Cabin Roller Door |  |  |  |  |  |  |  |  |
| Exterior Lighting |  |  |  |  |  |  |  | Terminal Joint Door |  |  |  |  |  |  |  |  |
| Sideways Mov't |  |  |  |  |  |  |  | Interlock Brakes on Wheels |  |  |  |  |  |  |  |  |
| OPERATOR: |  |  |  | REMARKS: |  |  |  |  |  |  |  |  |  |  |  |  |
| SIGN: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Form 6: Manoeuvring Area Inspection form

| ENTEBBE INTERNATINAL AIRPORT |  |  |  |  |  |  |  |  |  |  | FORM NO. |  |  |  |  |  |  |  |  | UCAA/EIA/AM/APP102/F06 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  | TITLE |  |  |  |  |  |  |  |  | MANEUVERING AREA INSPECTION REPORT FORM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | ISSUE DATE |  |  |  |  |  |  |  |  | 30 August, 2022 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | REVISION NO. |  |  |  |  |  |  |  |  | 01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MANEUVERING AREA (CONDITION) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Runway 17/35 |  |  | FOD seen |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge Lights |  |  | Signage |  | Water Presence |  | standing water | water | water |
| Runway 12/30 |  |  | FOD seen |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge Lights |  |  | Signage |  | Water Presence |  | standing water | water | water |
| Alpha A1 |  |  | Needs Sweeping |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |  |
| Bravo: B |  |  | Needs Sweeping |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |  |
| Charlie C1 |  |  | Needs Sweeping |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |  |
| Charlie C2 |  |  | Needs Sweeping |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |  |
| Charlie C3 |  |  | Needs Sweeping |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |  |
| Delta D |  |  | Needs Sweeping |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |  |
| Alpha A2 |  |  | Needs Sweeping |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |  |
| Alpha A3 |  |  | Needs Sweeping |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |  |
| Alpha A4 |  |  | Needs Sweeping |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |  |
| Hotel H1 |  |  | Needs Sweeping |  |  |  |  |  | Uneven Surface |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |  |
| Juliet J1 |  |  | Needs Sweeping |  |  |  |  |  |  | Uneven Surface |  |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |
| Juliet J2 |  |  | Needs Sweeping |  |  |  |  |  |  | Uneven Surface |  |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |
| Juliet J3 |  |  | Needs Sweeping |  |  |  |  |  |  | Uneven Surface |  |  |  |  |  | Potholes |  |  |  |  | Cracks |  |  | Obstructions |  | Edge lights |  |  | Signage |  |  |  |  |  |
| SURFACE MARKINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Runway 17/35 |  |  |  |  | Needs Painting |  |  |  |  |  |  | No Painting Needed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Centreline |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stripes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Threshold Marking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Runway 12/30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Centreline |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stripes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Threshold Marking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Taxiways |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Centrelines |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Side Stripes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Obstruction Markings |  |  |  |  | Intact |  |  |  |  | Non-Existent |  |  |  |  |  |  |  |  |  |  |  | Comment: |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Needs Painting |  |  |  |  | No Painting Needed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Apron 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Apron 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Apron 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Apron 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| VEGETATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Acceptable |  |  |  | Tall Grass |  |  |  |  |  | Shrubs |  |  | Trees |  |  |  |  | Other: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PAPI’s |  | Serviceable |  |  |  |  |  |  |  | Unserviceable |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WIND SOCK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Runway 17/35 |  | Serviceable |  |  |  |  |  |  |  | Damaged |  |  |  |  |  |  | Needs Replacement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Runway 12/30 |  | Serviceable |  |  |  |  |  |  |  | Damaged |  |  |  |  |  |  | Needs Replacement |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Discussed with MAM/MO |  |  |  |  |  |  |  | Yes |  | No |  |  |  |  | No |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reported by AOO.AS |  |  |  |  |  |  | Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sign |  |  |  |  |  |  |  |
| Approved by OO.AS |  |  |  |  |  |  | Name |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Sign |  |  |  |  |  |  |  |
| SOO.AS.OP Remark SIGN |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Form 7: Apron Inspection form

| ENTEBBE INTERNATIONAL AIRPORT |  |  |  |  | FORM NO: |  |  |  |  |  |  |  |  | UCAA/EIA/AM/APP1/F07 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TITLE |  |  |  |  |  |  |  |  | APRON INSPECTION FORM |  |  |  |  |  |
|  |  |  |  |  | ISSUE DATE |  |  |  |  |  |  |  |  | 31-May-19 |  |  |  |  |  |
|  |  |  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | REVISION |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| PERSONNEL: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| INSPECTION CONDUCTED BY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | DATE: |  |  | START TIME: |
| (I) | Name: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (ii) | Name: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (iii) | Name: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STANDS CONTAINING FOD (TICK) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| APRON 1 |  | STANDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ACTION REQUIRED |  |
|  |  | 1 | 2 | 3 | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  | 10 | 11 | 20 | 21 |  | 22 |  | 23 |  | 24 |  | 25 |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
| APRON 2 |  | 1 | 2 | 3 | 4 |  | 5 |  | 6 |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
| APRON 4 |  | 1 | 2 | 3 | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
| APRON 5 |  | 50 | 51 | 51A |  | 51B |  | 52 |  | 53 |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
| STAND SITUATION (TICK) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| STANDS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | REMARKS |  |
| OCCUPIED STANDS |  | 1 | 2 | 3 | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  | 10 | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
| AVAILABLE STANDS |  | 1 | 2 | 3 | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  | 10 | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ____________________________________________________ |  |
| VEHICLE CORRIDOR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ORDERLY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CROWDED: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DISORDERLY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FUEL HYDRANTS/PITS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SERVICEABLE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BLOCKED: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DAMAGED: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIGNAGE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CLEAR |  |  |  | NOT CLEAR |  |  |  |  | OBSTRUCTION |  |  |  |  |  |  |  |  | DAMAGED |  |
| SERVICEABLE |  |  |  | FITTINGS DAMAGED |  |  |  |  |  |  |  |  |  |  |  |  |  | LAMPS BLOWN OFF |  |
| PAVEMENT MARKINGS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CLEAR/CLEAN |  |  |  | BLOTTED |  |  |  |  |  |  |  |  |  |  |  |  |  | DAMAGED |  |
| PAVEMENT CONDITIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| POTHOLES |  |  |  | CRACKS |  |  |  |  |  |  |  |  |  |  |  |  |  | DEPRESSIONS |  |
| EQUIPMENT PARKING YARD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ORDERLY |  |  |  | CROWDED |  |  |  |  |  |  |  |  |  |  |  |  |  | DISORDERLY |  |
| FOD BINS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ADEQUATELY DEPLOED |  |  |  | INADEQUATELY |  |  |  |  |  |  |  |  |  |  |  |  |  | FLOODED |  |
| FORMER PARADE GROUND |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PAVEMENT CLEAN |  |  | DIRTY |  |  |  |  |  | SHRUBS NEAT |  |  |  |  |  |  |  |  | OVERGROWN |  |
| REPORTABLE INCIDENT ON APRON(S) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| APRON 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| APRON 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| APRON 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| APTON 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIGN: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SIGN: |  |  |
| AOO/AS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SOO/AS/SC: |  |  |

### Form 8: Runway Surface Condition Report Form (GRF)

| ENTEBBE INTERNATIONAL AIRPORT |  |  | FORM NO. |  |  |  | UCAA/EIA/AM/APP1/02/F08 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | TITLE |  |  |  | RUNWAY SURFACE CONDITION REPORT FORM (GRF) |  |  |  |  |  |  |  |  |  |  |
|  |  |  | ISSUE DATE |  |  |  | 30 August, 2022 |  |  |  |  |  |  |  |  |  |  |
|  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | REVISION NO. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RUNWAY CONDITION REPORT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AIRPORT | DATE/TIME (DDMMHHmm) | RUNWAY |  | RWY CONDN’T FOR @ ⅓ |  |  |  | % COVERAGE FOR @ ⅓ |  |  | DEPTH OF LOOSE CONT’T FOR @ ⅓ |  |  | COND’T DESCRIPTION @ ⅓ |  |  | SITUATION AWARENES e.g Description of slippery wet |
| HUEN | 02011357 | 17 |  | 5 | 5 | 2 |  | 25 | 50 | 100 | 02 | 01 | 12 | Wet | Wet | Standing water | Rubber/water/water/water |
| % OF Coverage for Contaminants |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 - 25 |  | 25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 - 50 |  | 50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 51 - 75 |  | 75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 76 - 100 |  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### Form 9: Spillage Control Form

| ENTEBBE INTERNATIONAL AIRPORT | FORM NO |  | UCAA/EIA/AM/APP1/02/F09 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TITLE |  | SPILLAGE CONTROL FORM |  |  |  |  |
|  | ISSUE DATE |  | 30 December,22 |  |  |  |  |
|  | REVISION DATE |  |  |  |  |  |  |
|  | REVIVION NO |  | 01 |  |  |  |  |
| DATE: | TIME: |  | FLIGHT NO.: | LOCATION: |  |  |  |
| EQUIPMENT DEPLOYED/LEAKING | ORGANIZATION |  | EQIOPMENT NO. |  | OIL/FUEL/HYDRAULIC SPILLAGE |  | ACTION TAKEN |
| Push Back |  |  |  |  |  |  |  |
| GPU |  |  |  |  |  |  |  |
| Catering Truck |  |  |  |  |  |  |  |
| Low loader |  |  |  |  |  |  |  |
| High loader |  |  |  |  |  |  |  |
| Front Steps |  |  |  |  |  |  |  |
| Behind Steps |  |  |  |  |  |  |  |
| Bus/Van/Puck-up (Tick) |  |  |  |  |  |  |  |
| Conveyor |  |  |  |  |  |  |  |
| Fuelling Truck |  |  |  |  |  |  |  |
| Others: |  |  |  |  |  |  |  |
| INSPECTORS NAME: |  |  |  |  |  | SIGN: |  |
| AOO/AS SIGN: |  | REMARKS: |  |  |  |  |  |

### Form 10: Airside incident Report Form

| ENTEBBE INTERNATIONAL AIRPORT |  | FORM NO. |  |  |  | UCAA/EIA/AM/APP1/02/F10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TITLE |  |  |  | AIRSIDE INCIDENT REPORT FORM |  |  |  |  |  |
|  |  | ISSUE DATE |  |  |  | 30, August, 2022 |  |  |  |  |  |
|  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |
|  |  | REVISION NO. |  |  |  | 02 |  |  |  |  |  |
| TITLE OF THE REPORT: |  |  |  |  |  |  |  |  |  |  |  |
| GENERAL INFORMATION |  |  |  |  |  |  |  |  |  |  |  |
| Company/Airline: |  | Operator: |  |  |  |  | Flight No.: |  | Aircraft/ Equip’t Reg No. |  |  |
| Aircraft/ Equip’t type |  | Origin: |  |  |  |  | Destination |  | POB: |  |  |
| Date: |  | Time: |  |  |  |  | Weather: |  | Phase of operation: |  |  |
| ACCIDENT AND INCIDENT FACTS/SEQUENCE OF EVENTS: |  |  |  |  |  |  |  |  |  |  |  |
| Actions taken: |  |  |  |  |  |  |  |  |  |  |  |
| Observations: |  |  |  |  |  |  |  |  |  |  |  |
| Proposed corrective action to prevent future occurrences: |  |  |  |  |  |  |  |  |  |  |  |
| Recommended Corrective Action: |  |  |  |  |  |  |  |  |  |  |  |
| Reported by Name: |  |  |  | Sign: |  |  |  |  |  | Date: |  |
| Received by: |  |  |  | Sign: |  |  |  |  |  | Date: |  |
| Verified by POO: |  |  |  |  |  |  |  |  |  |  |  |
| Forwarded by MO: |  |  |  |  |  |  |  |  |  |  |  |
| GM-EIA |  |  |  |  |  |  |  |  |  |  |  |
| Type of legend |  |  |  |  |  |  |  |  |  |  |  |
| CAUSE/TO | EQUIPMENT |  | AIRCRAFT |  | PERSONNEL |  |  | PROPERTY |  |  | PASSENGER |
| EQUIPMENT | A |  | B |  | C |  |  | D |  |  | E |
| AIRCRAFT | F |  | G |  | H |  |  | I |  |  | J |
| PERSONNEL | K |  | L |  | M |  |  | N |  |  | O |
| PROPERTY | P |  | Q |  | R |  |  | S |  |  | T |
| PASSENGER | X |  | X |  | X |  |  | X |  |  | X |
| Cc: DAAS, GM-EIA, MSMS, POO |  |  |  |  |  |  |  |  |  |  |  |

### Form 11: Preliminary Incident and Accident Investigation Report Form

| ENTEBBE INTERNATIONAL AIRPORT |  | FORM NO. | UCAA/EIA/AM/APP1/02/F11 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TITLE | PRELIMINARY INCIDENT AND ACCIDENT INVESTIGATION FORM |  |  |  |
|  |  | ISSUE DATE | 30 August, 2022 |  |  |  |
|  |  | REVISION DATE |  |  |  |  |
|  |  | REVISION NO. | 02 |  |  |  |
| TITLE OF THE REPORT: |  |  |  |  |  |  |
| GENERAL INFORMATION |  |  |  |  |  |  |
| Company/Airline: | Operator: |  | Flight No.: |  | Aircraft Reg No. |  |
| Aircraft type: | Origin: |  | Destination: |  | POB: |  |
| Date: | Time: |  | Weather: |  | Phase of operation: |  |
| ACCIDENT AND INCIDENT DATA AS REPORTED/FACTS |  |  |  |  |  |  |
| PRELIMINARY INVESTIGATION |  |  |  |  |  |  |
| Objective of investigation: |  |  |  |  |  |  |
| Personnel Interviewed and responses: |  |  |  |  |  |  |
| Data Analysis/ Findings: |  |  |  |  |  |  |
| Probable cause: |  |  |  |  |  |  |
| Contributing Factors: |  |  |  |  |  |  |
| Limitation of the investigation: |  |  |  |  |  |  |
| Conclusions: |  |  |  |  |  |  |
| Recommended Corrective Action: |  |  |  |  |  |  |
| Investigator’s Name: |  |  |  | Sign: |  | Date: |
| Cc: DAAS, GM-EIA, MSMS, POO |  |  |  |  |  |  |

### Form 12: Airside Daily Operational Report Form

| ENTEBBE INTERNATIONAL AIRPORT |  | FORM NO. |  |  | UCAA/EIA/AM/APP1/02/F12 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TITLE |  |  | AIRSIDE OPERATIONAL DAILY REPORT FORM |  |
|  |  | ISSUE DATE |  |  | 30 August, 2022 |  |
|  |  | REVISION DATE |  |  |  |  |
|  |  | REVISION NO. |  |  | 01 |  |
| AIRSIDE OPERATIONAL DAILY REPORT |  |  |  |  |  |  |
| Shift on duty: |  |  |  | Date: |  |  |
| SHIFT MEMBERS PRESENT |  |  |  | SHIFT MEMBERS ABSENT |  |  |
| 1 |  |  |  | 1 |  |  |
| 2 |  |  |  | 2 |  |  |
| 3 |  |  |  | 3 |  |  |
| 4 |  |  |  | 4 |  |  |
| 5 |  |  |  | 5 |  |  |
| 6 |  |  |  | 6 |  |  |
| Issues at Briefing: |  |  |  |  |  |  |
| Operational Status: |  |  |  |  |  |  |
| Incidents/Accidents: |  |  |  |  |  |  |
| Shift Leader's remark: |  |  |  |  |  |  |
| Name: |  |  | Sign: |  |  |  |
| SOO/AS/OPS COMMENTS: |  |  |  |  |  |  |
| NAME: |  |  | SIGN: |  |  |  |
| CC: MO, POO/AS |  |  |  |  |  |  |

### Form 13: monthly Aircraft Turn Around Form

| ENTEBBE INTERNATIONAL AIRPORT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | FORM NO. | UCAA/EIA/AM/APP1/02/F13 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TITLE | MONTHLY AIRCRAFT TURN AROUND FORM |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ISSUE DATE | 30 Aug-2022 |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | REVISION DATE |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | REVISION NO. | 01 |  |  |  |  |
| PRE-ARRIVAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  | Pre-flight briefing held |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  | Arrival FOD walk complete |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  | Gate area cleared (clean and orderly) |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  | Passenger boarding bridge in position |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  | Marshaller in position for a/c arrival with suitable wands (where required) |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  | GSE staged behind aircraft safety lines |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  | Stop line for aircraft (a/c) type identified |  |  |  |  |  |  |  |
| ARRIVAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  | Approved hand signals used |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  | Staff waited until a/c was safe to approach (engines shut down, beacon switched off, thumbs up, etc.). |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  | A/c properly chocked |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |  |  |  |  |  |  | PBB progresses after thumbs up signal from the ground engineer. |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  |  | PBB Moves with no person in the cabin except the operator. |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  |  |  |  |  |  |  |  | The airline Official knocks on the door to alert the crew to open. |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  |  |  |  |  |  |  |  | GPU connected to aircraft properly |  |  |  |  |  |  |  |
| 15 |  |  |  |  |  |  |  |  |  |  |  |  |  | Correct headset or signals used during aircraft receipt process |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  | Tail stand fitted to aircraft |  |  |  |  |  |  |  |
| PBB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 17 |  |  |  |  |  |  |  |  |  |  |  |  | Pre-use check |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  |  |  |  |  |  |  |  | PBB positioning procedures followed |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  |  |  |  |  |  |  |  | Inspection of the door and surrounding area completed before opening. |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |  | Passenger door opened and secured properly |  |  |  |  |  |  |  |  |
| PASSENGER STEPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 21 |  |  |  |  |  |  |  |  |  |  |  | Pre-use check |  |  |  |  |  |  |  |  |  |
| 22 |  |  |  |  |  |  |  |  |  |  |  | Mobile steps positioning procedures followed |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  |  |  |  |  |  |  |  | Inspection of the door and surrounding area completed before opening. |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  |  |  | Passenger door opened and secured properly |  |  |  |  |  |  |  |  |  |
| PERSONNEL SERVICING PERSONNEL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 25 |  |  |  |  |  |  |  |  |  |  | PPE personnel |  |  |  |  |  |  |  |  |  |  |
| 26 |  |  |  |  |  |  |  |  |  |  | Personnel exhibit appropriate safety behavior. |  |  |  |  |  |  |  |  |  |  |
| 27 |  |  |  |  |  |  |  |  |  |  | Personnel avoid walking where not authorized (e.g., Under fuselage, stepping over tow bar, or between carts) |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |  |  |  | Personnel use correct manual handling techniques. |  |  |  |  |  |  |  |  |  |  |
| 29 |  |  |  |  |  |  |  |  |  |  | All traffic rules were followed |  |  |  |  |  |  |  |  |  |
| TUG OPERATIONS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 30 |  |  |  |  |  |  |  |  |  | Apron speed limits were followed |  |  |  |  |  |  |  |  |  |  |  |
| 31 |  |  |  |  |  |  |  |  |  | Correct approach directions used for offload |  |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  |  | Dolly chains within maximum allowed length. |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  |  |  |  |  |  | Clearance maintained between GSE and aircraft. |  |  |  |  |  |  |  |  |  |  |  |
| 34 |  |  |  |  |  |  |  |  |  | Safety stops done with care. |  |  |  |  |  |  |  |  |  |  |  |
| 35 |  |  |  |  |  |  |  |  |  | Seat belt worn |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  |  | Unattended vehicles turned off |  |  |  |  |  |  |  |  |  |  |  |
| CARGO AND BAGGAGE OFFLOADING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 37 |  |  |  |  |  |  |  |  | Conveyor belt loader approach procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |
| 38 |  |  |  |  |  |  |  |  | Belt loader positioning procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |
| 39 |  |  |  |  |  |  |  |  | Cargo door/sills/locks checked prior to ground support equipment (GSE) approaching aircraft |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |  |  | (GSE) operated properly (e.g., Speed, Safety Stops) |  |  |  |  |  |  |  |  |  |  |  |  |
| 41 |  |  |  |  |  |  |  |  | Baggage and cargo properly handled |  |  |  |  |  |  |  |  |  |  |  |  |
| 42 |  |  |  |  |  |  |  |  | All cargo properly transported. |  |  |  |  |  |  |  |  |  |  |  |  |
| 43 |  |  |  |  |  |  |  |  | Cargo hold/handling systems visually checked for damage after downloading. |  |  |  |  |  |  |  |  |  |  |  |  |
| 44 |  |  |  |  |  |  |  |  | The Joint Container Pallet Loader (JCPL) approach procedures followed. |  |  |  |  |  |  |  |  |  |  |  |  |
| 45 |  |  |  |  |  |  |  |  | Dolly locks properly set |  |  |  |  |  |  |  |  |  |  |  |  |
| SERVICING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WASTER SERVICING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 1 |  |  |  |  |  |  |  | Toilet van operating procedures followed. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  | Waste servicing procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WATER SERVICING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 3 |  |  |  |  |  |  |  | Water servicing vehicle operating procedures followed. |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  | Water servicing vehicle operating procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  | Portable water units kept away from lavatory units or other sources of contamination |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FUEL DISPENSER/BOWSER TRUCKS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 6 |  |  |  |  |  |  |  | Fuel truck approach procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  | Fuel truck positioning procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  | Fuel truck operating procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  | Fueling clearance zone requirements followed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  | Fuelling while passengers re on board procedure followed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  | Appropriate fire extinguisher deployed |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HIGH LOADERS - DECKER PLATFORM LOADER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 11 |  |  |  |  |  |  | Pre-use check completed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  | DPL positioned correctly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  |  |  |  |  | Appropriate staff members positioned prior to use. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 |  |  |  |  |  |  | DPL operating procedures followed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CATERING HIGH LOADER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REAMRKS |
| 15 |  |  |  |  |  | Catering truck approach procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  | Catering truck positioning procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 |  |  |  |  |  | Catering truck operating procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 |  |  |  |  |  | Catering truck removal procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 |  |  |  |  |  | Catering truck is free of FOD; any FOD/garbage dropped on ramp removed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  | Vehicles operated safely (e.g., Speed, safety stops). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 |  |  |  |  |  | Catering truck properly configured prior to removing from A/C |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CLEANING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 22 |  |  |  |  | Cleaning vehicle approach procedures |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 |  |  |  |  | Cleaning vehicle positioning procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  | Any FOD/garbage dropped on ramp removed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 |  |  |  |  | Vehicles Operated Safely (e.g., Speed, Safety Stops). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CARGO AND BAGGAGE UPLOADING |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONVEYOR LOADER, BAGGAGE AND CARGO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 1 |  |  |  | Ramp free of any items that could interfere with the safe operation of GSE or cause FOD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  | Conveyor belt loader operating procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  | Conveyor belt loader positioning procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  | Conveyor belt loader removal procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  | Baggage and cargo properly handled (e.g., Dangerous goods (DG) orientation, care taken). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  | All cargo properly transported and manifested Loading Instruction Report (LIR) is available and followed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  | LIR is available and followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  | Cargo hold/handling systems visually checked for damage before loading. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  | All required locks, cargo nets secured (including gaps at top of hold). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  | Door closure procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CONTAINERS AND JUMBO CONTAINERAND PALLET LOADER |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 11 |  |  | JCPL approach procedures followed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  | JCPL positioning procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  |  | JCPL removal procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPARTURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PRE-DEPARTURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REAMRKS |
| 1 |  | Ramp free of any items that could interfere with the safe operation of GSE or cause FOD |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  | Pre-departure walk around performed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  | Steering bypass pin installed, and tow bar connected |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  | Correct size/type of tractor and tow bar used for pushback |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  | Ground power removal procedures. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  | Pre-pushback procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PBB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 7 |  | Receive information from crew prior to closing the door |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  | Passenger door closed and secured properly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  | Loading bridge retraction procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PASSENGER STEPS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 10 |  | Receive information from crew prior to closing the door |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 |  | Passenger door closed and secured properly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  | Mobile Pax steps retraction procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DEPARTURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO | REMARKS |
| 13 | Agent remains at a safe position on the PBB until the A/C clears the stand. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | Correct marshalling signals/communication used during dispatch. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | Wing walking procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | Pushback procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Steering by-pass pin and tow bar disconnect procedure followed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Final dispatch /release procedures followed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspector’s name: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Title: |  |
| Sign: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Date: |  |  |  | Time: |  |  |
| OO/AS Remarks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NAME: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SIGN: |

### Form 14: Apron Equipment Survey During Peak Traffic

| ENTEBBE INTERNATIONAL AIRPORT |  |  |  |  | FORM NO. |  |  | UCAA/EIA/AM/APP1/02/F14 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TITLE |  |  | APRON EQUIPMENT AUDIT DURING PEAK HOUR |  |  |  |  |  |  |  |  |
|  |  |  |  |  | ISSUE DATE |  |  | 30 Aug-2022 |  |  |  |  |  |  |  |  |
|  |  |  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | REVISION NO. |  |  | 01 |  |  |  |  |  |  |  |  |
| DATE: | TIME: |  |  |  |  |  | FLIGHT NO.: |  |  |  |  |  |  | GHA: |  |  |
| AIRCRAFT TYPE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DESCRIPTION |  |  | EQUIPMENT NO. |  |  |  |  |  | SERVICEABLE |  | UNSERVICEABLE |  |  |  |  |  |
| Ground Power Unit (GPU) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Air Starter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| High loader |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Low Loader |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pallet Loader |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Forklift |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tug Master |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Towing Tractor |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water Truck/Pump |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Toilet Van |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Passenger Steps |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Conveyor Belt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ramp Vehicle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Tow Bar |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cargo Dolly/Container Dolly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Baggage Trolly |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Mail Cart |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wheelchair |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Others: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UISL EQUIPMENT USED |  |  |  | No. Deployed: |  |  |  |  | EJAF EQUIPMENT TYPE USED |  |  |  |  |  | No. Deployed |  |
|  |  | Serviceable |  | Unserviceable |  |  |  |  |  |  |  |  | Serviceable |  | Unserviceable |  |
| Catering Truck |  |  |  |  |  |  |  |  | Jet A-1 Dispenser Truck |  |  |  |  |  |  |  |
| Catering Van |  |  |  |  |  |  |  |  | Fuel Bowser |  |  |  |  |  |  |  |
| Catering Cart |  |  |  |  |  |  |  |  | Service Pick-up |  |  |  |  |  |  |  |
| Others: |  |  |  |  |  |  |  |  | Others: |  |  |  |  |  |  |  |
| FRESH HANDLING SERVICES EQUIPMENT USED |  |  |  |  |  |  |  |  | AIRLINE SUPPORT SERVICES |  |  |  |  |  |  | No. Deployed |
|  |  | Serviceable |  | Unserviceable |  |  |  |  |  |  |  | Serviceable |  |  | Unserviceable |  |
| ID/No. |  |  |  |  |  |  |  |  | Apron Van |  |  |  |  |  |  |  |
| Towing Tractors |  |  |  |  |  |  |  |  | Service Pick-up |  |  |  |  |  |  |  |
| Cargo Dollies |  |  |  |  |  |  |  |  | Engineering Motor Steps |  |  |  |  |  |  |  |
| Service Van |  |  |  |  |  |  |  |  | Engineering Trolley |  |  |  |  |  |  |  |
| Others: |  |  |  |  |  |  |  |  | Others: |  |  |  |  |  |  |  |
| GENERAL AVIATION AIRCRAFT OPERATORS |  |  |  | No. Deployed |  |  |  |  | POSTA UGANDA |  |  |  |  |  | No. Deployed |  |
|  |  | Serviceable |  | Unserviceable |  |  |  |  |  |  |  | Serviceable |  |  | Unserviceable |  |
| Apron Van |  |  |  |  |  |  |  |  | Mailing Van/Truck |  |  |  |  |  |  |  |
| Pax Steps |  |  |  |  |  |  |  |  | Mailing Trolley/Cart |  |  |  |  |  |  |  |
| Engineering Steps |  |  |  |  |  |  |  |  | Others: |  |  |  |  |  |  |  |
| Others: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GROUND MAINTENANCE CONTRACTOR |  |  |  | No. Deployed |  |  |  |  | CIVIL AVIATION AUTHORITY |  |  |  |  |  | No. Deployed |  |
|  |  | Serviceable |  | Unserviceable |  |  |  |  |  |  |  | Serviceable |  |  | Unserviceable |  |
| Tipper Truck |  |  |  |  |  |  |  |  | Airside Operations Vehicle |  |  |  |  |  |  |  |
| Service Pick-up |  |  |  |  |  |  |  |  | Wildlife Vehicle |  |  |  |  |  |  |  |
| Bulldozer/Caterpillar |  |  |  |  |  |  |  |  | ARFFS Vehicles |  |  |  |  |  |  |  |
| Dumper |  |  |  |  |  |  |  |  | Engineering Services |  |  |  |  |  |  |  |
| Mixer Machine/Truck |  |  |  |  |  |  |  |  | Others: |  |  |  |  |  |  |  |
| Others: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Inspector's Name: |  |  |  |  |  |  |  |  | Sign: |  |  |  |  |  | Date: |  |
| OO/AS Remark: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  | Sign: |  |  |  | Date: |  |  |  |  |  |  |

### Form 15: Airside Violation Form

| ENTEBBE INTERNATIONAL AIRPORT |  | FORM NO. |  |  | UCAA/EIA/AM/APP1/02/F15 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TITLE |  |  | AIRSIDE VIOLATION FORM |  |  |  |  |
|  |  | ISSUE DATE |  |  | 30 Aug-2022 |  |  |  |  |
|  |  | REVISION DATE |  |  |  |  |  |  |  |
|  |  | REVISION NO. |  |  | 01 |  |  |  |  |
| DATE: | TIME: |  | WEATHER: |  |  |  |  |  |  |
| OFFENDER’S NAME: |  |  |  |  | OTHER NAMES: |  |  |  |  |
| SECURITY PASS NO.: |  |  |  |  | AVOP NO: |  |  |  |  |
| ORGANISATION: |  |  |  |  | DESIGNATION: |  |  |  |  |
| NATURE OF VIOLATION: |  |  |  |  |  |  |  |  |  |
| CAUSE OF VIOLATION: |  |  |  |  |  |  |  |  |  |
| TO BE COMPLETED BY THE OFFENDER |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | YES | NO |
| 1. WERE YOU AWARE THAT THIS VIOLATES AIRSIDE RULES AND REGULATIONS? |  |  |  |  |  |  |  |  |  |
| 2. HAVE YOU UNDERGONE AIRSIDE SAFETY AWARENESS/TRAINING? |  |  |  |  |  |  |  |  |  |
| 3. WHEN WAS THE LAST AIRSIDE SAFETY TRAINING THAT YOU ATTENDED? |  |  |  |  |  |  |  |  |  |
| 4. WHO TRAINED YOU? |  |  |  |  |  |  |  |  |  |
|  |  |  |  | OFFENDER’S SIGNATURE: …............................................... |  |  |  |  |  |
| UCAA ENFORCER’S REMARK: |  |  |  |  |  |  |  |  |  |
| SURNAME: |  | OTHER NAMES: |  |  |  |  | SIGN: |  |  |
| TO BE COMPLETED BY EMPLOYER/REPRESENTATIVE |  |  |  |  |  |  |  |  |  |
| DATE EMPLOYEE WAS FIRST DEPLOYED AT AIRSIDE: |  |  |  |  |  |  |  |  |  |
| WAS HE/SHE TRAINED? |  |  |  |  |  |  |  |  |  |
| COMMITMENT TO ENSURE THAT THE EMPLOYEE ADHERES TO AIRSIDE RULES/REGULATIONS: |  |  |  |  |  |  |  |  |  |
| SURNAME: |  | OTHER NAMES |  |  |  |  | SIGN: |  |  |
| UCAA SOO/AS/SC REMARKS: |  |  |  |  |  |  |  |  |  |
| SURNAME: |  | OTHERNAMES: |  |  |  |  | SIGN: |  |  |

### Form 16: On Sport Equipment and Personnel Report Form

| ENTEBBE INTERNATIONAL AIRPORT |  |  |  | FORM NO. |  | UCAA/EIA/AM/APP1/02/F16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TITLE |  | ON SPOT EQUIPMENT AND PERSONNEL REPORT FORM |  |  |  |  |  |
|  |  |  |  | ISSUE DATE |  | 30 Decemeber-2022 |  |  |  |  |  |
|  |  |  |  | REVISION DATE |  |  |  |  |  |  |  |
|  |  |  |  | REVISION NO. |  | 00 |  |  |  |  |  |
| Date | Time | Equipment No | Operators Name |  | Location |  | Organization |  | defect/ Violation |  | Action Taken |
| INSPECTOR’S NAME: |  |  |  |  |  |  |  | SIGN: |  | DATE: |  |
| OO/AS’ NAME: |  |  |  |  |  |  |  | SIGN: |  | DATE: |  |
| REMARKS: |  |  |  |  |  |  |  |  |  |  |  |

### Form 17: Airside Driving permit Application Form

| ENTEBBE INTERNATIONAL AIRPORT |  |  |  | FORM NO. |  |  | UCAA/EIA/AM/APP1/02/F17 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TITLE |  |  | AIRSIDE DRIVING PERMIT APPLICATION FORM |  |  |  |  |  |  |  |  |
|  |  |  |  | ISSUE DATE |  |  | 30 Aug-2022 |  |  |  |  |  |  |  |  |
|  |  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | REVISION NO. |  |  | 0 |  |  |  |  |  |  |  |  |
| PARTICULARS OF EMPLOYER: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name of Employer/Organization |  |  |  |  |  |  |  |  |  |  |  | Tel: |  |  |  |
| Address: |  |  |  |  |  |  |  |  |  |  |  | Ext: |  |  |  |
| Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PARTICULARS OF APPLICANT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name of Applicant: |  |  |  |  | Sex: |  |  |  | A/Pass No.: |  |  |  |  |  |  |
|  |  |  |  |  | Male |  |  |  | Expiry Date: |  |  |  |  |  |  |
| Job Title: |  |  |  |  | Female |  |  |  | D/Permit No.: |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | Expiry Date: |  |  |  |  |  |  |
| Previous ADP No. |  |  |  |  |  |  |  |  | ID/Passport No. |  |  |  |  |  |  |
| APPROPRIATE MOV'T AREAS OF OPERATION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Apron/Ramp | C/Bond |  | Taxiways |  |  |  |  | Runways |  |  |  |  |  | Roadways |  |
| VEHICLE/EQUIPMENT THE APPLICANT IS COMPETENT AND CERTIFIED TO OPERATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AVSOP CODE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BROWN |  | GREEN |  |  |  | BLUE |  |  |  |  |  | RED |  |  |  |
| NATIONAL DRIVING PERMIT CODE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| B, B1, BE, D1, G |  | C, C1, C1E |  |  |  | D, D1E, DE |  |  |  |  |  | CE, G (Heavy) |  |  |  |
| Car |  | Fuel Dispenser |  |  |  | Rump Bus |  |  |  |  |  | All Apron Equipment |  |  |  |
| Stations Wagon |  | Pushback |  |  |  | Coaster Bus |  |  |  |  |  | Trailer |  |  |  |
| Electric Tugs |  | High loader |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Pick-up |  | Low Loader |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Minibus (Van) |  | Ambulift |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Forklift |  | Pallet Loader |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Towing Tractor |  | Mobile Pax Steps |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Catering Truck |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Fire Truck |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Lorry |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Conveyor Truck |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SPONSOR'S DECLARATION: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (To be completed by an authorised official of an airline, GHA, EJAF, GA, UN, or UCAA, on behalf of their contractor or Employee) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| I, THE UNDERSIGNED, CERTIFY THAT: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| a) THE APPLICANT HOLDS AA VALID UGANDA DRIVING LICENCE AND THAT THE DRIVING LICENCE SERIAL NO. IS …............................ AS STATED IN THE PARAGRAPH 2 ABOVE, IS CORRECT; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| b) THE APPLICANT IS QUALIFIED TO OPERATE THE VEHICLE/EQUIPMENT STATED IN PARAGRAPH 4; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| c) IT IS NECESSARY FOR THE APPLICANT TO HOLD AN ADP; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| d) THE APPLICANT HAS BEEN INSTRUCTED IN THE UCAA AIRSIDE DRIVING CODE SENSITIZATION FOR SAFE OPERATIONS OF VEHICLE/EQUIPMENT IN THE AREAS STATED IN PARAGRAPH 3; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| e) I WILL BRING THE TRAFFIC AND VEHICLE'S REQUIREMENTS AS SET OUT IN THE AIRPORT BYLAWS AND THE DIRECTOR OF AIRPORTS INSTRUCTIONS AS MODIFIED FROM THE TIME TO THE ATTENSION OF THE APPLICANT WHO WILL BE ISSUED WITH AN ADP; |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIGN ON BEHALF OF THE EMPLOYER/SPONSOR: |  |  |  |  |  |  |  |  |  |  | POSITION: |  |  |  |  |

| FOR OFFICIAL USE ONLY |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ADP Serial Number |  | Date of Issue |  | Previous ADP No. |  | Date Collected | Sign: |
| Approved/Not Approved |  |  |  |  |  |  |  |
| Remarks: |  |  |  |  |  |  |  |
| Signing Authority: |  |  |  |  | Designation: |  |  |
|  |  |  |  |  | Cash/Credit/No Charge (Tick) |  |  |

### Form 18: ESSAT Checklist Form

| ENTEBBE INTERNATIONAL AIRPORT |  |  |  | FORM NO. |  |  |  |  | UCAA/EIA/AM/APP1/02/F18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TITLE |  |  |  |  | VEHICLE/EQUIPMENT SAFETY AUDIT CHECKLIST BY ESSAT |  |  |  |  |  |  |  |  |  |
|  |  |  |  | ISSUE DATE |  |  |  |  | 30 Aug-2022 |  |  |  |  |  |  |  |  |  |
|  |  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | REVISION NO. |  |  |  |  | 01 |  |  |  |  |  |  |  |  |  |
| INSPECTION DATE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ORGANIZATOION/COMPANY: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | COMPANY LOGO |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | YES |  | NO |  |
| AIRSIDE VEHICLE NO.: |  |  | COLOUR: |  |  |  |  |  |  | TYPE: |  |  |  |  |  |  |  |  |
| MANUFACTURE DATE: |  | INSPECTION DATE: |  |  |  |  |  |  |  |  |  |  | STICKER NO.: |  |  |  |  |  |
| EQUIPMENT SAFETY STATUS AUDIT TEAM (ESSAT): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID NO. | NAME |  |  |  |  |  |  |  |  |  |  | ORGANIZATION |  |  |  | SIGN |  |  |
| VEHICLE/EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ID NO. | DESCRIPTION |  |  |  | GOOD | BAD |  |  |  | DEFECTS OBSERVED |  |  |  | DATE OF CORRECTION |  |  | YES | NO |
|  | Starting Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Battery Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Steering Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Exhaust Pipe Emission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Side/Tail Reflective Materials |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Windscreen with no Cracks/ Watering Sprinklers/Wiper blades |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Adequate Rear-View Mirrors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Condition of Rear Lenses |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Beacon Color/Yellow |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lighting intensity & Flashing rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ELECTRIC MOTOR VEHICLE (EMV) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | Installed |  | Not Installed |  |  |  |  |  |  |  |  |  |  |  |
|  | Approaching vehicle sound for pedestrians (VSP) System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Headlamp |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hand brake |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Indicators |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ELECTRICAL SYSTEM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Correctly Adjusted Headlights |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hazard Lights / Indicators |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Brake / Parking / Reverse Lights |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Colour of The Flashing Beacon / Serviceability Status |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Horn/Applicable Siren |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Functionality Of Dashboard Instruments |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work Lights/License Plate Lights |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fog Lights |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Rear and Side Illuminative Reflectors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Emergency Stop Switches |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LEAKAGES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fuel |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | PTMO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hydraulic Systems |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Water / Foam |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Engine Oil / CC Oil |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Coolant Levels |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SAFETY EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Serviced Fire Extinguisher |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Valid First Aid Kit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Stretcher |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Oxygen Bottle |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Gloves / Fixed Wash Basin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Patient Resuscitating Equipment |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Body Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bird Scaring Cassettes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Beacon Colour/ Blue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lighting Intensity & Flashing Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BRAKING SYSTEMS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Brakes with adequate stopping Power |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Condition of the Hand Brake |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Tyres with adequate tread and correct pressure |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GROUND HANDLING EQUIPMENT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Tow Bar Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Work Warning Sirens |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Platform Scissors |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Platform Belts / Platform Rollers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hydraulic Lifts / Rams |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Conveyor Belts |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Rubber Guards / Side Guards |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bed Rollers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AERODROME RESCUE AND FIRE FIGHTING SERVICES (ARFFS) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Ground Sprinklers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Turret Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Rescue Appliances |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Water Hose Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Flood Lights |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Beacon Colour / Blue |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Lighting Instensity & Flashing Rate |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ENTEBBE JOINT AVIATION FACILITY (EJAF) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fuel Pump Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Fuel Hose Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Dead Man Condition |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Brake Interlock System |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No Ash Trays in The Cabin |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Warning Safety Signages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| GENERAL REMARKS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RECOMMENDATIONS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SECRETARY: |  |  |  |  |  |  |  | CHAIRPERSON: |  |  |  |  |  |  |  |  |  |  |
| SIGN: |  |  |  |  |  |  |  | SIGN: |  |  |  |  |  |  |  |  |  |  |

### Form 19: Dolly/Cart Audit Form

| ENTEBBE INTERNATIONAL AIRPORT |  | FORM NO. |  | UCAA/EIA/AM/APP1/02/F19 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TITLE |  | DOLLY/CART AUDIT FORM |  |  |  |  |  |
|  |  | ISSUE DATE |  | 30 Aug-2022 |  |  |  |  |  |
|  |  | REVISION DATE |  |  |  |  |  |  |  |
|  |  | REVISION NO. |  | 00 |  |  |  |  |  |
| DOLLY/CART AUDIT FORM |  |  |  |  |  |  |  |  |  |
| DATE | COMPANY |  | DOLLY/CART NO. |  |  | COLOUR | DESCRIPTION |  |  |
| DOLLY/CART PART | DEFECTS IDENTIFIED |  | DEFECTS RECTIFIED |  |  | VALIDATION DATE | REMARKS |  |  |
|  |  |  | YES |  | NO |  |  |  |  |
| 1. Breaks |  |  |  |  |  |  |  |  |  |
| 2. Tyres |  |  |  |  |  |  |  |  |  |
| 3. Side Reflectors |  |  |  |  |  |  |  |  |  |
| 4. Towbar |  |  |  |  |  |  |  |  |  |
| 5. Others |  |  |  |  |  |  |  |  |  |
| ESSAT INSPECTORS |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  | Sign: |  |  |  | Date: |
| Name: |  |  |  |  | Sign: |  |  |  | Date: |
| Name: |  |  |  |  | Sign: |  |  |  | Date: |
| Name: |  |  |  |  | Sign: |  |  |  | Date: |
| ESSAT SECRETARY NAME: |  |  |  |  | SIGN: |  |  | DATE: |  |

### Form 20: FOD Form

| ENTEBBE INTERNATRIONAL AIRPORT |  |  |  | FORM NO |  |  | UCAA/EIA/AM/APP1/02/F20 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TITLE |  |  | FOD CLEANING FORM |  |  |  |  |  |  |  |  |
|  |  |  |  | ISSUE DATE |  |  | 30 Aug-2022 |  |  |  |  |  |  |  |  |
|  |  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | REVIVION NO |  |  | 01 |  |  |  |  |  |  |  |  |
| DATE: |  | START TIME: |  |  |  |  | ENDING TIME: |  |  |  |  |  | WEATHER: |  |  |
| AERAS FOD WAS PICKED/CLEANED: TICK |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| APRON 1 | APRON 2 | APRON 4 |  | APRON 5 |  |  |  |  |  |  |  |  |  |  |  |
| TAXIWAYS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RWY 17/35 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| A1 | A2 | A3 | A4 |  | B |  |  | C1 |  |  | C2 |  | C3 | D |  |
| RWY 12/30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| H1 | J1 | J2 |  | J3 |  | TT |  |  |  |  |  |  |  |  |  |
| OTHER: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CELANING METHOD USED |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| FOD BOSS |  | SWEEPING |  | SCRUBBING |  |  | FOD WALK |  |  |  |  | FOD PIC DURING INSPECTION |  |  |  |
| DESCRIPTION OF FOD PICKED/SWEPT |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DESCRIPTION |  | QTY |  | COLOUR |  | FOD LOCATION |  |  |  | ACTION TAKEN |  |  |  |  |  |
| Aircraft part |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other metal |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| plastic |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Polyethylene |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Concrete |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Asphalt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stone |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Wood |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Paper |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Grass |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Carcass |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Cloth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total Weight |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Kilometre reading of FOD BOSS: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Start |  |  |  | Finish: |  |  |  |  |  | Kms Covered |  |  |  |  |  |
| FOD Reported by: |  |  |  |  |  |  |  |  |  |  |  | Company/Designation: |  |  |  |
| Cleaning/FOD Picked By: |  |  |  |  |  |  |  |  |  | Company/Designation: |  |  |  |  |  |
| Cleaning Supervised By: |  |  |  |  |  |  |  |  |  |  |  | Sign: |  |  |  |
| Remarks: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| AOO/AS Remark: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Name: |  |  |  |  |  |  | Sign: |  |  |  |  |  |  |  |  |

### Form 21: Quarterly FOD Walk Report Form

| ENTEBBE INTERNATIONAL AIRPORT |  |  |  | FORM NO. | UCAA/EIA/AM/APP1/02/F21 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | TITLE | QUARTERLY FOD WALK REPORT FORM |  |  |
|  |  |  |  | ISSUE DATE | 30 Aug-2022 |  |  |
|  |  |  |  | REVISION DATE |  |  |  |
|  |  |  |  | REVISION NO. | 01 |  |  |
| DATE: |  |  | START TIME: |  |  | END TIME: |  |
| AREAS OF FOD WALK: |  |  |  |  |  |  |  |
| WEIGHT OF FOD: |  |  |  |  |  |  |  |
| FOD TYPE PICKED: |  |  |  |  |  |  |  |
| 1 |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |
| 11 |  |  |  |  |  |  |  |
| REMARKS BY AOO/AS IN-CHARGE FOD WALK: |  |  |  |  |  |  |  |
| NAME: |  |  |  |  |  |  | SIGN: |
| REMARKS BY SOO/AS: |  |  |  |  |  |  |  |
| NAME: |  |  |  |  |  |  | SIGN: |
| Attach attendance and pictures |  |  |  |  |  |  |  |
| Cc: MO, POO. |  |  |  |  |  |  |  |

### Form 22: Aircraft Fueling Safety Inspection Checklist

| ENTEBBE INTERNATIONAL AIRPORT |  | FORM NO. | UCAA/EIA/AM/APP1/02/F22 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TITLE | AIRCRAFT FUELLING SAFTEY INSPECTION CHECKLIST |  |  |  |  |
|  |  | ISSUE DATE | 11-Apr-25 |  |  |  |  |
|  |  | REVISION DATE |  |  |  |  |  |
|  |  | REVISION NO. | 0 |  |  |  |  |
| A. PERSONNEL & EQUIPMENT |  |  |  |  |  |  |  |
| No |  |  |  | yes | No | NA | Remarks |
|  | High visibility / reflective clothing worn by fueling staff. |  |  |  |  |  |  |
|  | Use of Personal Protective Equipment (PPE) |  |  |  |  |  |  |
|  | Proper hearing protection, proper safety footwear etc. |  |  |  |  |  |  |
|  | Operator has a valid class 4 ADP. |  |  |  |  |  |  |
|  | Vehicle has a valid AVP |  |  |  |  |  |  |
|  | Fire extinguishers are installed on vehicles and inspection tags are current |  |  |  |  |  |  |
|  | No smoking" signs on both sides of the vehicle |  |  |  |  |  |  |
|  | Externally mounted emergency engine stop controls are clearly identified. |  |  |  |  |  |  |
|  | Elevating platform fitted with aircraft wing proximity sensors. |  |  |  |  |  |  |
|  | Spill containment kit available |  |  |  |  |  |  |
|  | All tyres of the vehicle are in good condition. |  |  |  |  |  |  |
|  | Fuel Operator is trained to operate fuelling vehicle |  |  |  |  |  |  |
| A. PRE-FUELLING OPERATION |  |  |  |  |  |  |  |
| No |  |  |  | yes | No | NA | Remarks |
| 1 | Fuelling operator ensures vehicle’s stabilizers are fully in contact with tarmac before mounting on elevating platform to connect fuelling hose to aircraft coupling |  |  |  |  |  |  |
| 2 | Fuelling vehicle waits until the loaders have docked before moving into take up position (where applicable) |  |  |  |  |  |  |
| 3 | Fuelling vehicle moves forward into fuelling position |  |  |  |  |  |  |
| 4 | If fuel dispenser reverses towards the aircraft marshaller is used |  |  |  |  |  |  |
| 5 | Fuelling vehicle approaches aircraft at driving sped of 5km/hr (walking speed) |  |  |  |  |  |  |
| 6 | Access to hydrant emergency stop switch is visible and clear of obstruction. |  |  |  |  |  |  |
| 7 | Apron staff is aware of the location and use of emergency stop button. |  |  |  |  |  |  |
| 8 | Fuelling vehicle is positioned correctly to ensure strain is not applied to the fuelling hoses, when connected to the aircraft. |  |  |  |  |  |  |
| 9 | Bonding/Grounding connections established between fuelling vehicle and aircraft before fuelling operations |  |  |  |  |  |  |
| 10 | Lanyard connected to the hydrant pit valve is free of obstruction and readily accessible to fuelling operator and ground staff. |  |  |  |  |  |  |
| 11 | Hydrant pit valve identified by a 4winged flag cone or equivalent |  |  |  |  |  |  |
| 12 | Sufficient evacuation areas on ground beneath aircraft exit doors and are clear of obstructions. |  |  |  |  |  |  |
| A. FUELLING SAFETY ZONE (within minimum 3meters from Hydrant Pit) |  |  |  |  |  |  |  |
| No |  |  |  | yes | No | NA | Remarks |
| 1 | All staff don’t use portable electronic devices (phones, camera, etc) within fuel safety zone |  |  |  |  |  |  |
| 2 | Only authorized persons and vehicles are permitted in the fuelling zone |  |  |  |  |  |  |
| 3 | In the event of fuel spillage, GPU and air-conditioning unit (ACU) shall be stopped immediately. |  |  |  |  |  |  |
| A. FUELLING OPERATIONS |  |  |  |  |  |  |  |
| No |  |  |  | yes | No | NA | Remarks |
| 1 | Fuel Operator informs flight crew or qualified personnel onboard the aircraft when fuelling is about to begin and has been completed. |  |  |  |  |  |  |
| 2 | Fuel operator uses handheld deadman device throughout the operations and always remain outside the vehicle cab |  |  |  |  |  |  |
| 3 | Fuelling hoses have sufficient length to allow fuelling platform to be fully lowered whilst hoses/couplings are connected to the aircraft |  |  |  |  |  |  |
| 4 | Fuelling hoses are not entangled on equipment during movement of fuelling platform |  |  |  |  |  |  |
| 5 | Fuelling platform are not raised or lowered when refuelling operations are taking place |  |  |  |  |  |  |
| 6 | Fuelling vehicle has interlock mechanism to prevent it from moving while hoses are connected to the aircraft |  |  |  |  |  |  |
| 7 | No ramp staff go over the fuelling hose |  |  |  |  |  |  |
| 8 | Fuel operator is aware of procedures in event of fire, major fuel spill or fuel overflow |  |  |  |  |  |  |
| 9 | Fuel spillage, if any, is cleaned up properly and reported to airside Operations Centre (AOC) The fuelling operator informs the aircraft ground engineer to suspend all ground handling |  |  |  |  |  |  |
| 10 | Exercise extreme caution when fuelling an aircraft during lighting and thunderstorms. Suspend fuelling operations during severe thunderstorms. |  |  |  |  |  |  |
| 11 | One PLB or mobile passenger step or integral stairs are positioned with the aircraft door fully opened for evacuation of passengers when fuelling with passengers on board. |  |  |  |  |  |  |
| A. POST FUELLING OPERATIONS |  |  |  |  |  |  |  |
| No |  |  |  | yes | No | NA | Remarks |
| 1 | Fuel operator conducts a 360 degree walk around and inspections to ensure all hoses have been discounted and stowed correctly. |  |  |  |  |  |  |
| 4 | Fuel Operator clears foreign Objects Debris (FOD), if any |  |  |  |  |  |  |
| Inspector’s Remarks: |  |  |  |  |  |  |  |
| Recommendations (s): |  |  |  |  |  |  |  |
| Name of Inspector: |  | Date: |  |  | Sign: |  |  |

### Form 23: Staff Deployment/Parking Plan Form

| ENTEBBE INTERNATIONAL AIRPORT | FORM NO. | UCAA/EIA/AM/APP1/02/F22 |
| --- | --- | --- |
|  | TITLE | STAFF DEPLOYMENT/PARKING PLAN FORM |
|  | ISSUE DATE | 30 August, 2022 |
|  | REVISION DATE |  |
|  | REVISION NO. | 01 |

| SCHEDULED MOVEMENT DAY/NIGHT SHIFT: DATE: |  |  |  |  |  |  |  |  |  |  |  |  | NON – SCHEDULED AND OTHER MOVEMENT DATE: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FLIGHT NUMBER | A/C REG. | A/C TYPE | ORIGIN | STA | STD | ON CHCKS | DEST. | FLIGHT NUMBER | OFF CHCKS | A/C STAND | DEPLOYED OFFICER |  | FLIGHT NUMBER |  | A/C REGIST. | A/C TYPE | ORIGIN | STA | STD | ATA CHCKS | DEST. |  | FLIGHT NUMBER | ATD CHCKS | A/C STAND |  | DEPLOYED OFFICER |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other deployments |  |  |  |  |  |  |  | OFFICER |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 1. | APRON / RUNWAY INSPECTION |  |  |  |  |  |  |  | NAME: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SIGN: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 2. | ENFORCEMENT / GENERAL AVIATION |  |  |  |  |  |  |  | NAME: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SIGN: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 3. | PASSENGER BOARDING BRIDGE (PBB) OPS |  |  |  |  |  |  |  | NAME: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SIGN: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 4. | SPECIAL OPS / AIRCRAFT TURN AROUND |  |  |  |  |  |  |  | NAME: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SIGN: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | 5. | WORKS FACILITATION |  |  |  |  |  |  |  | NAME: |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | SIGN: |  |  |  |  |  |  |  |  |

SHIFT LEADER: Name:    ________________________________  	Sign:    __________________________

Comments:   ________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________________

### Form 24: Tank Farm Inspection Checklist

| ENTEBBE INTERNATIONAL AIRPORT | FORM NO. | UCAA/EIA/AM/APP1/02/F24 |
| --- | --- | --- |
|  | TITLE | TANK FARM INSPECTION CHECKLIST |
|  | ISSUE DATE | 7th May, 2025 |
|  | REVISION DATE |  |
|  | REVISION NO. | 00 |

|  | Inspection Check list |  | Location____________ |
| --- | --- | --- | --- |
| S/N | Inspection Requirements | Findings (Yes/No/NA) | Recommendations |
| 1 | Airport Depot Operations and Quality Control |  |  |
|  | Inspector shall check that records of routine checks are readily available & up to date. |  |  |
| 1.1 | QC and Operating Procedures |  |  |
|  | Inspector shall check that Standards and Procedures are available and up to date |  |  |
| 1.1.1 | Is Operations Manuals, Standards available for the organization/aviation fuel supplying company responsible for the quality control and operating manual in use? |  |  |
| 1.1.2 | Is there a designated person at the airport for the day to day operation of the fuel facilities? |  |  |
| 1.1.3 | Are inspection report recommendations and their status reviewed by the organization/fuel supplying company responsible for the operation? |  |  |
| 1.1.4 | Where applicable national or regional legislation requires compliance with a standard that differs from industry Standards, is this clearly documented. |  |  |
| 1.1.5 | Is there a document retention policy consistent with standard requirements? |  |  |
| 1.2 | Product Documents from Supplying Locations |  |  |
|  | Inspector shall check documentation for compliance with Standard requirements. |  |  |
| 1.2.1 | Are Release Certificates confirming compliance with latest issue of industry standards (AFQRJOS, ASTM D1655 or Def. Stan. 91-91) and they contain grade, qty, time of loading, batch density at 20 degrees-C, batch no, water free confirmation, signatories |  |  |
| 1.2.2 | Are Refinery Certificates of Quality (RCQ), Certificates of Analysis (CoA) or Recertification and Periodic Test Certificates, if relevant, available, signed/ dated and do they confirm compliance with the latest issue of AFQRJOS? |  |  |
| 1.2.3 | Do receipt documents have details of receipt date, time and received volume? |  |  |
| 1.2.4 | Do receipt documentations include details of depot storage tanks status (ullage) information? |  |  |
| 1.2.5 | Do records show that product settled for at least the minimum required time? |  |  |
| 1.2.6 | Before release, are bottom samples taken for a Control Check and a Visual Check? |  |  |
| 1.2.7 | Do tank records include receipt details, batching information, and evaluation of batch weighted average, new-batch assigning, signatory for release of product, date and time of product release? |  |  |
| 1.3 | Refueller Loading Facilities |  |  |
|  | Inspector shall witness at least one aircraft refueling equipment loading. |  |  |
| 1.3.1 | Are loading points grade-marked and color coded to EI 1542 and marked with flow direction arrows? |  |  |
| 1.3.2 | Is jet fuel loaded via a filter water separator qualified to EI 1581 5th edition or other approved filter? (e.g. EI 1583 filter monitor? ) |  |  |
| 1.3.3 | Are pump start/stop switches safely accessible, near to the loading point, fully effective and clearly identified? |  |  |
| 1.3.4 | Are hoses and nozzles in good condition and are they fitted with caps to prevent entry of dirt, etc.? Do the hoses comply with the required standards? |  |  |
| 1.3.5 | Is fueller bonded to loading facility before hoses are connected and is bonding maintained until hoses are disconnected? |  |  |
| 1.3.6 | Are bonding wires in good condition? |  |  |
| 1.3.7 | Does the operator remain in attendance throughout the loading and is there direct access to a means of stopping the flow quickly? |  |  |
| 1.3.8 | Are procedures and equipment available to prevent over-filling/ spillage? |  |  |
| 1.3.9 | Is high level pre-check device tested shortly after the commencement of loading? |  |  |
| 1.3.10 | Is loading stopped before the high level device is activated? |  |  |
| 1.3.11 | After loading, and following at least 10 minutes settling time, is a sample drawn from the fueller tank sump for Visual Check? |  |  |
| 1.3.12 | Is Visual check conducted correctly using approved equipment |  |  |
| 1.4 | Storage Tanks and Loading/Offloading Facilities |  |  |
|  | Inspector shall witness at least one tank and loading/offloading facilities QC checks. |  |  |
| 1.4.1 | Daily Low point draining and Appearance Checks for tanks under receipt and settling done? |  |  |
| 1.4.2 | Is draining and sampling carried out at full flow? |  |  |
| 1.4.3 | Is sampling conducted to required standards using suitable equipment? |  |  |
| 1.4.4 | Daily Low point draining and Visual Checks for Jet fuel tanks in operation done? |  |  |
| 1.4.5 | Periodic Test Certificates (static stock – including drums) - 6 months schedule for static product in tanks. |  |  |
| 1.4.6 | Floating suctions buoyancy checks (Minimum monthly schedule) |  |  |
| 1.4.7 | Condition of free vents and coarse mesh screens (or PV valves and flame arrestors where fitted) - Minimum quarterly schedule. |  |  |
| 1.4.8 | High level alarm systems checks-yearly schedule |  |  |
| 1.4.9 | Internal storage tank inspection and cleaning done as per required (5 years if fully lined)? |  |  |
| 1.4.10 | If chemicals had to be used for the cleaning, records of additional steps taken to ensure no contamination of the fuel is happening. |  |  |
| 1.4.11 | Quarterly product recovery tank internal inspections done? |  |  |
| 1.4.12 | Monthly draining of piping low points being done? |  |  |
| 1.4.13 | Is daily visual check of bonding wires and clips done? |  |  |
| 1.4.14 | Is weekly bonding/grounding electrical continuity check done (for systems not permissive bonding design). |  |  |
| 1.4.15 | Is the monthly visual inspection under operating conditions and operating pressure done? |  |  |
| 1.4.16 | Is the six monthly high pressure test of fueller loading hoses done? |  |  |
| 1.4.17 | Are the loading/offloading meter calibration test done as per schedule and results accurate to required standard? (six monthly for meters in service. For new or repaired meters, calibrate before putting in service). |  |  |
| 1.4.18 | Six monthly thermometer and Hydrometers Accuracy check done? |  |  |
| 1.4.19 | Is the monthly fire extinguisher condition check done? |  |  |
| 1.4.20 | Is the yearly fire extinguisher maintenance done? |  |  |
| 1.4.21 | Is the yearly earthing straps/rods electrical resistance checks done? |  |  |
| 1.6 | Fixed Facility Filtration Facility |  |  |
| 1.6.1 | Daily draining of filter vessel sumps under pressure for visual check done? (Inlet side drain for monitors without sumps) |  |  |
| 1.6.2 | Daily (and each fuel transfer) differential pressure monitoring and recording at maximum achievable flow |  |  |
| 1.6.3 | Weekly graphs of differential pressure (dP) readings at or corrected to maximum operating flow rate done? |  |  |
| 1.6.4 | Six monthly DP gauge checks for zeroing & free movement |  |  |
| 1.6.5 | Weekly gauge strainers check and draining |  |  |
| 1.6.6 | Max 3 years coalesce element replacement or with cause? |  |  |
| 1.6.7 | Yearly internal inspections of filter vessel done? Details to verify vessel internal cleanliness, element condition, torque setting during opening, internal lining condition, cover seal condition, inspection port condition, sump/low points free draining, spider plate condition, and spider plate continuity with vessel (max 10 ohms). |  |  |
| 1.6.8 | Yearly Separator elements (Teflon/synthetic) testing with water and maintenance as required done? |  |  |
| 1.7 | Supply Equipment-Receiving |  |  |
| 1.7.1 | Are trucks tankers/ rail storage tanks dedicated? |  |  |
| 1.7.2 | If road tankers tank cars are not dedicated: |  |  |
| 1.7.2.1 | Does delivery documentation state previous grade carried? |  |  |
| 1.7.2.2 | Does documentation show that satisfactory change of grade procedures have been observed? |  |  |
| 1.7.3 | Are loading documents checked, and do they correctly identify the transportation equipment and grade and quantity loaded? |  |  |
| 1.7.4 | On arrival, are seals on all filling orifices, man lids and outlets checked? |  |  |
| 1.7.5 | Is equipment bonded to offloading facility before hoses are connected and is bonding maintained until hoses are disconnected? |  |  |
| 1.7.6 | Is sampling conducted to required standards using approved equipment? |  |  |
| 1.7.7 | Are drain samples drawn for Control Check and results compared with the batch density shown on the Release Certificate? |  |  |
| 1.7.8 | After discharge, is a check made to ensure that all compartments are empty? |  |  |
| 2 | Into-Plane Facility |  |  |
|  | Inspector should inspect at least one aircraft fuelling equipment during the visit, related QC and inspection logs. |  |  |
| 2.1 | Quality Control and Maintenance |  |  |
| 2.1.1 | Service manuals for vehicles & aviation equipment and a preventative maintenance program |  |  |
| 2.1.2 | Defect reporting system |  |  |
| 2.1.3 | Vehicle/fuelling equipment serviceability and maintenance checks |  |  |
| 2.1.4 | Daily draining and sampling of fuelling vehicles done: |  |  |
|  | * At start of morning shift? |  |  |
|  | * After loading fuellers (fueller tanks only)? |  |  |
|  | * After defueling? |  |  |
|  | * After heavy rain/snow (fueller tanks only)? |  |  |
|  | *After washing or maintenance of tank, filter or fuelling system? |  |  |
| 2.1.5 | Yearly Fueller tank visual inspection from top hatch done? |  |  |
| 2.1.6 | Fueller tank visual inspection and internal cleaning done? (Max 5 years schedule). |  |  |
| 2.1.7 | Product recovery tanks visual inspection done (quarterly)? |  |  |
| 2.1.8 | Quarterly fueller overfill protection devices done? |  |  |
| 2.1.9 | Is over-wing fuelling procedure for grade confirmation in place? |  |  |
| 2.1.10 | Is weekly functional check of interlock system, warning lights and check of override system being done? |  |  |
| 2.1.11 | Is daily Check of seals on interlock override switch in place? |  |  |
| 2.1.12 | Is monthly platform emergency lowering system and wand sensor checks being conducted? |  |  |
| 2.1.13 | Monthly Functional check of emergency engine stops |  |  |
| 2.1.14 | Is the daily bonding cables and reels visual check of bonding wires and clips being done? |  |  |
| 2.1.15 | Is the weekly bonding/earthing system electrical continuity check being done? |  |  |
| 2.1.16 | Is the monthly hose pressure test being done? Test shall be conducted at working pressure with verification of hose details like mfg dates, condition of hose) |  |  |
| 2.1.17 | Is the six monthly (or after maintenance) high pressure test done for all hoses in operation? Procedure for pressure requirement if coupling re-fitted to be considered) |  |  |
| 2.1.18 | Monthly hose end strainers inspection and cleaning being done? |  |  |
| 2.1.19 | Is the yearly hydrant pit coupler wear checks being done? |  |  |
| 2.1.20 | Is dead man system checked for opening time and closing time requirements as per standard requirements-monthly? |  |  |
| 2.1.21 | Documents for into plane Job observation and on the spot actions. |  |  |
| 2.2 | Meters and Gauges |  |  |
| 2.2.1 | Six monthly meter calibration test results accurate to required standard for meters in-service and prior to use new or repaired meters in place? |  |  |
| 2.2.2 | Is the monthly dead man performance check on PCV done? |  |  |
| 2.2.3 | Quarterly pressure control system check for HEPCV and ILPCV being done? |  |  |
| 2.2.4 | Six monthly accuracy check for Thermometers and Hydrometers |  |  |
| 2.2.5 | Is the monthly fire extinguisher visual check on re-fuellers done? |  |  |
| 2.2.6 | Is the six monthly fire extinguisher maintenance done? |  |  |
| 2.3 | Supervision of Aircraft fuelling |  |  |
|  | Inspector should witness one complete aircraft fuelling operation, during the visit. |  |  |
|  | Aircraft type |  |  |
|  | Aircraft Operated By |  |  |
|  | Fuelling equipment used |  |  |
|  | Number of fuelling personnel |  |  |
|  | Grade of fuel delivered |  |  |
| 2.3.1 | Are the fuelling personnel equipped with proper PPE? |  |  |
| 2.3.2 | Are the fuelling personnel implementing the safety belt requirement? |  |  |
| 2.3.3 | Are the Airport regulations followed by refueling crew? |  |  |
| 2.3.4 | Does fuelling operator observe vehicle speed limit on apron? |  |  |
| 2.3.5 | Does fuelling vehicle approach aircraft in a safe manner? |  |  |
| 2.3.6 | If fueller has to be reversed into position for fuelling, is the maneuver performed with the assistance of a competent guide person? |  |  |
| 2.3.7 | Is vehicle positioned safely for fuelling and, if parked underwing, is there sufficient clearance between the vehicle and aircraft? |  |  |
| 2.4 | Fuelling Procedures |  |  |
| 2.4.1 | Is vehicle positioned safely? |  |  |
| 2.4.2 | For fuellers a clear exit route from aircraft is required throughout the fuelling. |  |  |
| 2.4.3 | Is bonding cable connected to the aircraft before fuelling commences and is bonding maintained throughout the fuelling operation? |  |  |
| 2.4.4 | For overwing fuelling are the additional bonding requirements followed? |  |  |
| 2.4.5 | Is a single sequence of connection/ disconnection defined and followed? |  |  |
| 2.4.6 | Are hoses positioned to avoid being run over by other aircraft servicing vehicles? |  |  |
| 2.4.7 | Does fuelling operator check (visually) the condition of the aircraft adaptor before connecting and after disconnecting the delivery hose? |  |  |
| 2.4.8 | For overwing fuelling, are procedures followed to prevent miss-fuelling? |  |  |
| 2.4.9 | Is the fuelling equipment free from product leaks? |  |  |
| 2.4.10 | Is sampling procedure and disposal in accordance with procedures? |  |  |
| 2.4.11 | Where required, is chemical water detector within date and correctly used? |  |  |
| 2.4.12 | Does operator record the differential pressure shortly after start of fuelling at maximum delivery flow rate? |  |  |
| 2.4.13 | Is deadman control (if fitted and not push-button type) override switch sealed? |  |  |
| 2.4.14 | For interlock system, is there a warning light system that identifies the status of the interlock system and is it functioning? Is emergency override sealed? |  |  |
| 2.4.15 | Is Quality Control check done before refueling (inspector to verify appropriate procedures are applied as per type of fuelling; refueller, hydrant or drum refuelling). |  |  |
| 2.4.16 | Does the fuelling operator perform a “360 degree” walk around the vehicle and look up at the fuelling connection at the completion of the fuelling operation? |  |  |
| 2.4.17 | Is a high visibility marker/flag used to identify the hydrant pit? |  |  |
| 2.4.18 | Are hydrant Emergency Stop Buttons (ESBs) accessible and clearly identified with high visibility signs? |  |  |
| 2.4.19 | Is the hydrant pit & inlet hose area satisfactorily illuminated at night? |  |  |
| 3 | Hydrant Operations |  |  |
|  | Inspector should inspect at least one hydrant pit during the visit. |  |  |
| 3.1 | Quality Control and Maintenance of Hydrant Pit |  |  |
| 3.1.1 | Low points Flushing Weekly |  |  |
| 3.1.2 | Hydrant Pit Low & High Point Weekly Inspection Report |  |  |
| 3.1.3 | Hydrant Pit Valve integrity check periodically as per required standards. |  |  |
| 3.1.4 | Unused Hydrant Pits Flushing periodically as per required standards. |  |  |
| 3.1.5 | Fuel Hydrant Valve Chamber Inspections Quarterly |  |  |
| 3.1.6 | Hydrant Pit Valve Dynamic test annually. |  |  |
| Inspected by: |  | Site representative: |  |
| Signature: |  | Signature: |  |
| Date: |  | Date: |  |

### Form 25: Low Visibility Procedure Implementation Form

| ENTEBBE INTERNATIONAL AIRPORT |  |  |  |  | FORM NO. |  |  |  | UCAA/EIA/AM/APP1/02/F25 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TITLE |  |  |  | LOW VISIBILITY PROCEDURE IMPLEMENTATION FORM |  |  |  |  |  |  |  |
|  |  |  |  |  | ISSUE DATE |  |  |  | 28 April 2025 |  |  |  |  |  |  |  |
|  |  |  |  |  | REVISION DATE |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | REVISION NO. |  |  |  | 00 |  |  |  |  |  |  |  |
| LOW VISIBILITY PROCEDURE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PREPARATORY PHASE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Time of 2000m RVR declaration: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stakeholders informed |  |  |  | RFFS |  | ELECTRICAL |  |  |  | BHWCU |  |  |  | GHA |  | AVSEC |
| Runways inspected |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17/35 | Time of inspection: |  |  |  |  |  |  | 12/30 |  |  | Time of inspection: |  |  |  |  |  |
| Equipment. Vehicles withdrawn from manoeuvring areas. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOW VISIBILITY PHASE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Critic equipment allowed into manoeuvring areas. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Aircraft lead to the parking |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Taxiways closed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| TERMINATION PHASE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Termination time |  | Declared by: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Stakeholders informed of the termination: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| REMARKS BY OO/AS (Flights Affected by low RVR) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NAME: |  |  |  |  |  |  |  |  |  |  |  |  | SIGN: |  |  |  |
| REMARKS BY SOO/AS-Ops |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NAME: |  |  |  |  |  |  |  |  |  |  |  |  | SIGN: |  |  |  |

References

### References:

- Civil Aviation (Aerodromes) Regulations 2022.

- Daily Flight Schedule

- Apron Parking Configuration Chart

- aircraft clearance information

- Mayfly

- Seasonal Aircraft Movement schedule

- Aircraft Clearance Circulars

- Annex 3: Meteorological Service for International Air Navigation

- Annex 6: Operation of Aircraft (Parts 1 and 2)

- Annex 8: Airworthiness of Aircraft

- Annex 15: Aeronautical Information Services

- PANS Aerodromes (Doc 9981)

- PANS-AIM (Doc 10166)

- PANS-ATM (Doc 4444).